3GPP TSG-SA WG1 Meeting #75	S1-162310

San Francisco, CA, 22-26 August 2016	(revision of S1-162095)

Title:	SMARTER - Migration

Agenda Item:	8.1

Source:				Nokia, Alcatel-Lucent Shanghai Bell

Contact:			Betsy Covell betsy.covell@nokia.com

Abstract: This document proposes text for the SMARTER TS Migration aspects. The requirements are drawn from the NEO TR.

First Proposed Text Change:

# 5	High-level requirements

## 5.x	Migration to 5G

### 5.x.1 Description

The 5G system will support most of the existing EPS capabilities, in addition to many new services. Only new or changed service requirements for new or changed services are specified in this TS. The few EPS capabilities that will not be supported by the 5G system are identified in clause 5.x.2.2 below.

### 5.x.2	Requirements

#### 5.x.2.1	Interworking between 5G systems

The 5G 3GPP system shall provide support for a device with a 5G subscription roaming into a 5G Visited Mobile Network which has a roaming agreement with the device's Home Mobile Network.

A 5G Visited Mobile Network shall provide support for establishing home network provided data connectivity as well as visited network provided data connectivity.

The 5G 3GPP system shall provide a mechanism for a network operator to limit access to its services for a roaming device, (e.g., based on roaming agreement).

The 5G system shall provide a mechanism for a network operator to direct a device onto a partner's Mobile Network for the purposes of data offload, based on operator agreement and policy.

#### 5.x.2.2	Legacy Service Support

The 5G system shall support all service requirements from EPS (e.g., from TSs 22.011, 22.101, 22.278, 22.185, 22.071, 22.115, 22.153, 22.173) with the following exceptions:

CS voice service continuity and/or fallback to GERAN or UTRAN,

Seamless handover between the 5G RAT(s) and GERAN or UTRAN, and

Access to a 5G core network via GERAN or UTRAN.

Editor's Note: 5G will be replaced with the appropriate brand name when it becomes available.