3GPP TSG-SA WG1 Meeting #76	S1-163380

Tenerife, Spain, 7-11 Nov 2016	Revision of S1-163281

Title:	Add activity factor, KPI values per area, and definitions for 7.1 (eMBB)

Agenda Item:	7.1

Source:	Huawei Technologies

Contact:	Andrey Krendzel (andrey.krendzel@huawei.com)

Laurence Meriau (laurence.meriau@huawei.com)

Abstract: This document proposes some changes for the eMBB table of the section ”Performance requirements”. In particular, it is introduced the activity factor, KPI values for are and some definitions.

Introduction

The following changes are proposed:

- The current Table 7.1-1 Performance requirement provides KPI values for certain scenarios assuming 3 operators and for some other scenarios assuming one operator as explained in NOTE2. As a result there is a risk of misinterpretation of the KPI values.

It is proposed to normalise these KPIs values per area of user experience independently of any potential number of operator, and thus, avoiding any misinterpretation of the KPI values. The NOTE2 is corrected correspondingly.

- An activity factor column is added in the table to align with NGMN considerations [2] that allows calculating the connection density of the total density for active users and as a result, to get values for the area traffic capacity that is the product of the user experienced data rate and connection density. With other activity factor consideration, KPIs will be different. Correspondingly, the NOTE3 is extended to specify activity factor values for the indoor hotspot scenario that assumes a mix of services in accordance with NGMN considerations [2].

- Besides, some definitions for the metrics used in the table such as “user experienced data rate”, “connection density”, and “area traffic capacity” are added based on NGMN definitions [2] for the definition section. As a result, the NOTE5 defining the user experienced data rate in Table 7.1-1can be removed.

Proposal

The following changes are proposed to SMARTER TS in line with above 3 aspects:

* * * Start of 1st change * * *

# 7	Performance requirements

## 7.1	High data rates and traffic densities

Several scenarios require the support of very high data rates or traffic densities of the 3GPP system. The scenarios address different service areas: urban and rural areas, office and home, and special deployments (e.g., massive gatherings, broadcast, residential, and high-speed vehicles). The scenarios and their performance requirements can be found in table 7.1-1.

-	Urban macro - The general wide-area scenario in urban area

-	Rural macro - The general wide-area scenario in rural area

-	Indoor hotspot - The scenario for offices and homes, and residential deployments.

-	Broadband access in a crowd - The scenario for very dense crowds, for example, at stadiums or concerts. In addition to a very high connection density the users want to share what they see and hear, putting a higher requirement on the uplink than the downlink.

-	Dense urban - The scenario for pedestrian users, and users in urban vehicles, for example, in offices, city centres, shopping centres, and residential areas. The users in vehicles can be connected either directly or via an onboard base station to the network.

-	Broadcast-like services - The scenario for stationary users, pedestrian users, and users in urban vehicles, for example, in offices, city centres, shopping centres, and residential areas. The passengers in vehicles can be connected either directly or via an onboard base station to the network.

-	High-speed train - The scenario for users in trains. The users can be connected either directly or via an onboard base station to the network.

-	High-speed vehicle - The scenario for users in road vehicles. The users can be connected either directly or via an onboard base station to the network.

-	Airplanes connectivity - The scenario for users in airplanes. The users can be connected either directly or via an onboard base station to the network.

Table 7.1-1 Performance requirements for high data rate and traffic density scenarios.

|  | Scenario | Experienced data rate (DL) | Experienced data rate (UL) (NOTE 5) | Area traffic capacity (DL) (NOTE 8) | Area traffic capacity (UL) (NOTE 8) | User density | Activity factor (NOTE 7) | UE speed | Coverage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a | Urban macro | 50 Mbps | 25 Mbps | 100 Gbps/km2 | 50 Gbps/km2 | 10000/km2 | 20% | Pedestrians and users in vehicles (up to 120 km/h | Full network (NOTE 1) |
| 1b | Rural macro | 50 Mbps | 25 Mbps | 1 Gbps/km2 | 0.5 Gbps/km2 | 100/km2 | 20% | Pedestrians and users in vehicles (up to 120 km/h | Full network (NOTE 1) |
| 2 | Indoor hotspot | 1 Gbps | 500 Mbps | 15 Tbps/km2 | 2 Tbps/km2 | 250000/km2 | NOTE 3 | Pedestrians | Office and residential (NOTE 3) (NOTE 4) |
| 3 | Broadband access in a crowd | 25 Mbps | 50 Mbps | [3.75] Tbps/km2 | [7.5] Tbps/km2 | [150000]/km2 | 30% | Pedestrians | Confined area |
| 4 | Dense urban | 300 Mbps | 50 Mbps | 750 Gbps/km2 | 125 Gbps/km2 | 25000/km2 | 10% | Pedestrians and users in vehicles (up to 60 km/h) | Downtown (NOTE 1) |
| 5 | Broadcast-like services | Maximum 200 Mbps (per TV channel) | N/A or modest (e.g., 500 kbps per user) | N/A | N/A | [15] TV channels of [20 Mbps] on one carrier | N/A | Stationary users, pedestrians and users in vehicles (up to 500 km/h) | Full network (NOTE 1) |
| 6a | High-speed train | 50 Mbps | 25 Mbps | 15 Gbps/train | 7.5 Gbps/train | 1000/train | 30% | Users in trains (up to 500 km/h) | Along railways (NOTE 1) |
| 6b | High-speed vehicle | 50 Mbps | 25 Mbps | [100] Gbps/km2 | [50] Gbps/km2 | [4000]/km2 | 50% | Users in vehicles (up to 250 km/h) | Along roads (NOTE 1) |
| 7 | Airplanes connectivity | 15 Mbps | 7.5 Mbps | 1.2 Gbps/plane | 600 Mbps/plane | 400/plane | 20% | Users in airplanes (up to 1000 km/h) | (NOTE 1) |

NOTE 1: For users in vehicles, the device can be connected to the network directly, or via an on-board moving base station. NOTE 3: A mix of services is assumed; only some users use services that require the highest data rates [2]. NOTE 4: For interactive audio and video services, for example, virtual meetings, the required two-way end-to-end latency (UL and DL) is 2-4ms while the corresponding experienced data rate needs to be up to 8k 3D video [300Mbps] in uplink and downlink. NOTE 5: User experienced data rate is the minimum data rate required to achieve a sufficient quality experience, with the exception of scenario for broadcast like services where the given value is the maximum that is needed. NOTE 6: All the values in this table are targeted values and not strict requirements. NOTE 7: Activity factor is a percentage value of the amount of simultaneous active UEs to the total number of UEs where active means the UEs are exchanging data with the network. NOTE 8: Area traffic capacity is the product of the user experienced data rate, the user density, and the activity factor.

* * * End of 1st change* * *

References:

[2] NGMN 5G white paper, February 2015.