3GPP TSG-SA WG1 Meeting #76bis	S1-170067

Spokane, USA, 16-20 JAN 2017	Revision of S1-17XXXX

Title:	FS_eV2X Additional Consolidation of Requirements

Agenda Item:	6.1

Source:	LG Electronics (Rapporteur), Huawei

Contact:	SungDuck Chun (sungduck.chun@lge.com)

- Abstract

This document proposes additional consolidated potential requirements out of use cases added in SA1#76.

- Discussion

At SA1#76, three use cases were added:

- 5.23	Cooperative lane change (CLC) of automated vehicles

- 5.24	Proposal for secure software update for electronic control unit

- 5.25	3D video composition for V2X scenario

| Section # | PR # | PR | Discussion | CPR # | CPR |
| --- | --- | --- | --- | --- | --- |
| 5.23 | [PR.5.23-001] | The 3GPP network shall support message exchange between UEs of message with less than [25] ms latency, [90] % reliability and message size [300-400] Bytes with semi-automated driving. | [LG][HW] Keep. Category is ‘advanced driving’ | CPR.A-XX1 | The 3GPP network shall support message exchange between UEs of message with less than [25] ms latency, [90] % reliability and message size [300-400] Bytes with semi-automated driving. |
|  | [PR.5.23-003] | The 3GPP network shall support message exchange between UEs with less than [10] ms latency, with [99.99] % reliability and maximum message size of [12] KBytes with full automated driving. | [LG][HW] Keep. Category is ‘advanced driving’ | CPR.A-XX2 | The 3GPP network shall support message exchange between UEs with less than [10] ms latency, with [99.99] % reliability and maximum message size of [12] KBytes with full automated driving. |
| 5.24 | [PR.5.24 -001] | The 3GPP System shall provide a secure mechanism to update V2X applications (e.g an Electronic Control Unit (ECU)). | [LG] Rephrasing is needed. 3GPP system itself does not update V2X application which is out of 3GPP scope. What is missing from CPR.G-022 & CPR.G-006 is the confidentiality and integrity between V2X applications. | CPR.G-XX1 | The 3GPP system shall be able to support confidentiality and integrity of message transfer between a UE supporting V2X application and a V2X application server. |
| 5.25 | [PR.5.25-001] | The 3GPP system shall provide a mechanism so that a server outside the 3GPP domain is able to time-synchronize different videos received from different UEs, each UE having a maximum absolute speed of 250 Km/h. | [LG] This sentence looks like a solution, not a service requirement. [HW] To be discussed during the meeting |  |  |
|  | [PR.5.25-002] | The 3GPP system shall support a data rate of [10] Mbps in the uplink per UE (to support 4K/UHD video). | [LG] remove (). Category is advanced driving | CPR.A-XX3 | The 3GPP system shall support a data rate of [10] Mbps in the uplink per UE. |
|  | [PR.5.25-003] | The 3GPP system shall support a mechanism for UEs to be able to calculate [50] absolute location fixes per second, with [TBD] meters of accuracy for each location fix. NOTE: This requirement is to support 250 Km/h (69 meters per second), so 50 location fixes per second implies one location fix every 1.4 meters. | [LG] What is the 3GPP system requirement is not clear in this sentence. Considering the use case description, it is not clear who calculate the location fix. Furthermore, depending on the capability (e.g. processing power), 50 location fix is either possible or impossible. Also, there is [TBD]. If further clarification is not provided, don’t keep. |  |  |

- Proposal

Below is the result of the consolidation, proposed to be added in Section 7 of FS_eV2X TR.

Accompanying CR is in S1-16XXXX.

Also, it is proposed to further discuss whether [PR.5.25-001] is to be included in the consolidated requirements.

--------------------------------------------------------------------------------------------------

------------------------------- START of change ------------------------------------

--------------------------------------------------------------------------------------------------

# 7	Potential requirements

## 7.1	General

Different V2X scenarios require the transport of V2X messages with different performance requirements for the 3GPP system.

Vehicles Platoonning enables the vehicles to dynamically form a platoon travelling together. All the UEs in the platoon obtain information from the leading vehicle to manage this platoon. These information allow the vehicles to drive closer (short time or distance inter-vehicle gap) than normal in a coordinated manner, going to the same direction and travelling together. These are expected to be a set of sophisticated application.

Extended Sensors enables the exchange of raw or processed data gathered through local sensors or live video images among  vehicles, road site units, devices’ of pedestrian and V2X application servers. The vehicles can increase the perception of their environemnt beyond of what their own sensors can detect and have have a more broad and holistic view of the local situation. High data rate is one of the key characteristics.

Advanced driving enables semi-automated or full-automated driving. Longer inter-vehicle distance is assumed. Each vehicle and/or RSU shares its own perception data obtained from its local sensors with vehicles in proximity and that allows vehicles to synchronize and coordinate their trajectories or manoeuvres. Each vehicle shares its driving intention with vehicles in proximity, too. The benefits of this use case group are safer traveling, collision avoidance, and improved traffic efficiency.

Remote Driving enables a remote driver or a V2X application to operate a remote vehicle for those passengers who cannot drive by themselves or remote vehicles located in dangerous environments. For a case where variation is limited and routes are predictable, such as public transportation, driving based on cloud computing can be used. Also, access to cloud-based back-end service platform can be considered for this use case group. High reliability and short low latency are the main requirements.

## 7.2	Consolidated requirements

Editor's Note: Potential requirements out of new use cases agreed in SA1#76 will be consolidated at SA1#76bis.

### 7.2.1	General requirements

Some requirements in this section are applicable to all V2X scenarios.

[CPR.G-001]	The 3GPP system shall be able to control the communication range for a message based on the characteristic of the messages transmitted by a UE supporting V2X application.

[CPR.G-002]	The 3GPP system shall be able to optimize the communication between UEs supporting V2X application belonging to the same group and in proximity.

[CPR.G-003]	The 3GPP system shall be able to support the message transfer for group management operations as requested by the application layer.

[CPR.G-004]	The 3GPP system shall be able to support message transfer among a group of UEs supporting V2X application.

[CPR.G-005]	The 3GPP system shall be able to support message transfer between two UEs belonging to the same group of UEs supporting V2X application.

[CPR.G-006]	The 3GPP system shall be able to support confidentiality and integrity of message transfer among a group of UEs supporting V2X application.

[CPR.G-007]	The 3GPP system shall support relative lateral position accuracy of 0.1 m between UEs supporting V2X application.

[CPR.G-008]	The 3GPP system shall support mechanisms to ensure sufficient reliability metrics are reached.

[CPR.G-009]	The 3GPP system shall support high connection density for congested traffic.

NOTE:	Example estimate is for worst case US Freeway scenario that does not include arterial roads (i.e. onramps): 5 lanes in each direction or 10 lanes total per highway, for up to 3 highways intersecting = 15,840 cars per mile.

[CPR.G-010]	The 3GPP system shall support efficient coordination of radio resources used for transport of V2X communications to maximize the utilisation of the available spectrum and to ensure the required reliability.

[CPR.G-011]	The 3GPP system shall be able to control the UL and DL reliability of transport of V2X communications, depending on the requirement of V2X application

[CPR.G-012]	Impact to E-UTRA(N) by UE supporting only 5G RAT(s) based V2X communication shall be minimized.

[CPR.G-013]	Impact to 5G RA(N) by UE supporting only E-UTRA based V2X communication shall be minimized.

[CPR.G-014]	The 3GPP system shall be able to support message transfer between UEs or between a UE and a UE-type RSU, regardless of whether or not they are subscribers of the same PLMN supporting V2X communications. In case they are subscribers to different PLMNs, there shall be no service degradation of the message transfer.

[CPR.G-015]	The 3GPP system shall be capable of supporting high reliability without requiring application-layer message retransmissions.

[CPR.G-016]	The 3GPP system shall enable discovery and communication between UEs supporting the same V2X application.

[CPR.G-017]	The 3GPP system shall be able to support for the operator to select which 3GPP RAT to use for a V2X application.

[CPR.G-018]	The 3GPP system shall enable a UE supporting a V2X application to obtain network access via another UE supporting V2X application.

[CPR.G-019]	The 3GPP system shall enable a UE supporting a V2X application to discover another UE supporting V2X application that can offer access to the network.

[CPR.G-020]	The 3GPP system shall enable a UE supporting a V2X application to switch the network access from direct 3GPP connection to an indirect 3GPP connection via another UE supporting V2X application that is connected via 3GPP access to the 3GPP core network, and vice versa.

[CPR.G-021]	The 3GPP system should provide integrity and confidentiality protection (end to end) for the network access traffic of a V2X UE via another such UE.

[CPR.G-022]	The 3GPP system shall allow UEs supporting V2X application to use 5G RAT for direct communication when the UEs are not being serviced by a 5G cell.

[CPR.G-023]	An RSU shall be able to communicate with up to 200 UEs supporting a V2X application.

[CPR.G-024]	The 3GPP system shall support less than 5 ms communication latency for transport of V2V messages between two UEs supporting V2V applications, that are part of a group of UEs supporting V2Vapplications.

NOTE:	The determination of group membership may be done at the upper layers and/or lower layers (application and/or Layer 2).

[CPR.G-026]	The 3GPP system shall be able to support confidentiality and integrity of message transfer between a UE supporting V2X application and a V2X application server.

### 7.2.2	Requirements for platooning

[CPR.P-001]	The 3GPP system shall be able to support up to 5 UEs for a group of UEs supporting V2X application.

[CPR.P-002]	For Vehicle Platooning, the 3GPP system shall be able to support reliable V2V communications between a specific UE supporting V2X applications and up to 19 other UEs supporting V2X applications.

[CPR.P-003]	The 3GPP system shall support relative longitudinal position accuracy of less than 0.5 m for UEs supporting V2X application for platooning in proximity.

Table 7.2.2-1 Performance requirements for platooning

| Communication scenario |  |  |  | Payload (Bytes) | Tx rate (Message/ Sec) | Max latency (ms) | Reliabi-lity (%) | Data rate (Mbps) | Commu-nication range (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Section # | Description |  | CPR # |  |  |  |  |  |  |
| 5.1 | Among a group of UEs (or two UEs) supporting V2X application |  | [CPR.P-004] | 50-1200 (NOTE 1) | 30 | 10 (NOTE 2) |  |  |  |
|  |  |  | [CPR.P-005] | 300-400 | 30 | 25 | 90 |  |  |
| 5.2 | Between UE supporting V2X application and RSU via another UE supporting V2X application |  | [CPR.P-006] | [50-1200] | 2 | 500 |  |  |  |
| 5.5 | Between UEs supporting V2X application | Driver control | [CPR.P-007] | 300-400 (NOTE 3) |  | 25 | 90 |  |  |
|  |  | Fully automated driving | [CPR.P-008] | 1200 |  | 10 | 99.99 |  | 80 |
| 5.12, 5.13 | Between UEs supporting V2X application | Driver control/ Limited automated driving | [CPR.P-009] | [6500] | 50 | [20] |  |  | [10] sec * (max. relative speed) [m/s] |
|  |  | Fully automated driving | [CPR.7.P-010] |  |  | [20] |  | [65] | [5] sec * (max. relative speed) [m/s] |
| 5.12, 5.13 | Between UE supporting V2X application and RSU | Driver control/ Limited automated driving | [CPR.7.P-011] | [6000] | 50 | [20] |  |  | [10] sec * (max. relative speed) [m/s] |
|  |  | Fully automated driving | [CPR.7.P-012] |  |  | [20] |  | [50] | [5] sec * (max. relative speed) [m/s] |
| NOTE 1: This value does not including security related messages component. NOTE 2: This value is end-to-end latency. NOTE 3: This value is applicable for both triggered and periodic transmission of data packets. |  |  |  |  |  |  |  |  |  |

### 7.2.3	Requirements for advanced driving

Table 7.2.3-1 Performance requirements for advanced driving

| Communication scenario |  |  |  | Payload (Bytes) | Tx rate (Message/Sec) | Max latency (ms) | Reliabi-lity (%) | Data rate (Mbps) | Commu-nication range (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Section # | Description |  | CPR # |  |  |  |  |  |  |
| 5.9 | Between UEs supporting V2X applications Fully automated driving |  | [CPR.A-001] | [2000] |  | [10] | [99.99] | [10] |  |
| 5.10, 5.11 | Between UEs supporting V2X application | Driver control/ Limited automated driving | [CPR.A-002] | [6500] | 10 | [100] |  |  | [10] sec * (maximum relative speed) [m/s] |
|  |  | Fully automated driving | [CPR.A-003] |  |  | [100] |  | [53] | [5] sec * (max. relative speed) [m/s] |
| 5.10, 5.11 | Between the UE supporting V2X application and the RSU | Driver control/ Limited automated driving | [CPR.A-004] | [6000] | 10 | [100] |  |  | [10] sec * (max. relative speed) [m/s] |
|  |  | Fully automated driving | [CPR.A-005] |  |  | [100] |  | [50] | [5] sec * (max. relative speed) [m/s] |
| 5.20 | Between UEs supporting V2X application Fully automated driving |  | [CPR.A-006 |  |  | [3] | [99.999] | [30] | [500] |
| 5.22 | Between RSU and UE supporting V2X application |  | [CPR.A-007] | 450 | 50 |  |  | DL: [0.5] UL: [50] |  |
| 5.23 | Between UEs supporting V2X application | Limited automated driving | [CPR.A-008] | [300-400] |  | [25] | [90] |  |  |
|  |  | Full automated driving | [CPR.A-009] | [12000] |  | [10] | [99.99] |  |  |
| 5.25 | Between a UE supporting V2X application and a V2X application server |  | [CPR.A-010] |  |  |  |  | UL: [10] |  |

### 7.2.4	Requirements for extended sensors

Table 7.2.4-1 Performance requirements for extended sensors

| Communication scenario |  |  |  | Payload (Bytes) | Max latency (ms) | Reliabi-lity (%) | Data rate (Mbps) | Communication range (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Section # | Description |  | CPR # |  |  |  |  |  |
| 5.3 | Between UEs supporting V2X application Fully automated driving |  | [CPR.E-001] |  | 10 | 95 | Peak data rate [25] |  |
| 5.6 | Between UEs supporting V2X application | Driver control | [CPR.E-002] | [1600] | 100 | 99 |  | 1000 |
|  |  | Fully automated driving | [CPR.E-003] |  | 3 | 99.999 |  | 200 |
|  |  |  | [CPR.E-004] |  | 10 | 99.99 |  | 500 |
|  |  |  | [CPR.E-005] |  | 50 | 99 |  | 1000 |
|  |  |  | [CPR.E-006] |  |  |  | 1000 | 50 |
| 5.16 | Between UEs supporting V2X application | Driver control/ Automated driving | [CPR.E-007] |  | [50] | 90 | [10] | [100] |
|  |  | Fully automated driving | [CPR.E-008] |  | [10] | 99.99 | [700] | [500] |

### 7.2.5	Requirements for remote driving

[CPR.R-001]	The 3GPP system shall support user experienced data rate up to 1 Mbps at DL and 20 Mbps at UL for UE supporting V2X application between V2X application server and UE for an absolute speed of up to 250 km/h.

[CPR.R-002]	The 3GPP system shall support ultra-high UL and DL reliability [99.999 or higher] % for UE supporting safety-related V2X application.

[CPR.R-003]	The 3GPP system shall support end-to-end latency 5 ms between V2X application server and UE supporting safety-related V2X application for an absolute speed of up to 250 km/h.

Table 7.2.5-1 Performance requirements for remote driving

| Communication scenario |  |  | Payload (Bytes) | Max latency (ms) | Reliabi-lity (%) | Data rate (Mbps) | Communication range (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Section # | Description | CPR # |  |  |  |  |  |
| 5.21 | Between a UE supporting V2X application & V2X Application Server. Driver Control | [CPR.R-004] |  | [20] | [99.999] | UL: 25 DL: 1 |  |

--------------------------------------------------------------------------------------------------

------------------------------- END of change ------------------------------------

--------------------------------------------------------------------------------------------------