3GPP TSG-SA WG1 Meeting #76bis	S1-170074

Spokane, USA, 16-20 JAN 2017	Revision of S1-17XXXX

Title:	Discussion on Access Control

Agenda Item:	5.1	Smarter: New Services and Markets Technology Enablers

Source:	LG Electronics Inc.

Contact:	SungDuck Chun <sungduck.chun (at) lge.com>

Abstract: This contribution discusses requirements on access control in next generation system

- Discussion

In previous two meetings, SA1 discussed requirements related to access control for 5G system. There was a general consensus in SA1 that:

- Current access control is too complex due to existence of different access mechanisms.

- A simple and extensible access control mechanism is needed for 5G system.

During the SA1#76, S1-163267 was the outcome of offline discussion but not agreed because there were still missing details, such as requirements regarding network sharing, etc.

- Proposal

It is proposed to use single access control mechanism which is flexible and extensible. It is proposed to agree on the text proposal below for the new smarter TS.

* * * * Start of Text Proposal * * * *

# 6	Basic capabilities

## 6.X	Access control for 5G system

### 6.x.1	Description

The 5G system will support diverse types of UEs and different types of network slices while ensuring QoS for various applications. Thought the capacity of 5G system will be superior to previous systems, the 5G system may occasionally not be able to handle entire traffic demand. To guarantee QoS of services and users in this situation, 5G system performs access control to allow access attempts from certain UEs while prevent access attempts from other UEs.

Depending on operator policies, deployment scenarios, subscriber profiles, provided services and so on, different criterion will be used in determining which access attempt should be allowed or prevented when congestion actually occurs in the system. While it is important to support different criterions for access control, this should not result in multiple layers of different access checks or inter-dependency among different access checks. Thus, the 5G system will provide a single access control framework where operators can flexibility categorize various access attempts and control the access attempts per each category.

### 6.x.2	Requirements

#### 6.x.2.1	General

The 3GPP system shall be able to provide means for operators to classify access attempts into access categories, based on various criteria (e.g. device type, subscription, applications and so on).

The 3GPP system shall be able to provide means for operators to control (e.g. allow, prevent, throttle) access attempts per each access category.

The 3GPP system shall be able to provide means for operators to configure a UE with mapping information between access attempts and access categories.

The 3GPP system shall be able to support for independent control of access when multiple core networks or operators share the same access network.

* * * * End of Text Proposal * * * *