3GPP TSG-SA WG1 Meeting #76-bis	S1-170091

Spokane, US, 16-20 January 2017

Title:	Proposed requirement on network slicing elasticity exposure

Agenda Item:	5.1 SMARTER

Source:	China Mobile

Contact:	chenweiyj@chinamobile.com

Abstract: : This document proposes to add requirements to section 6.10.2 Network Capability Exposure.

### Discussion

In the TS22.261, the 5G system has the following capabilities:

Based on operator policy, a 5G network shall provide suitable APIs to allow a 3rd party to create, modify and delete network slices used for the 3rd party.

The 5G system shall support the adaptation of capacity, i.e. elasticity of capacity of a network slice.

Based on above requirements, 5G system allows the 3rd party to create a network slice and have the capability to support the adaptation of capacity. The auto scaling mechanism can be used by 5G system to support the elasticity. However, it is not enough for a 3rd party. For example, after a 3rd party created a customized network slice, it can plan its business development (e.g., users’ traffic variation), and it also can predict the trend of traffic load of network slice according to its operation experience. So it is essential to allow the trusted 3rd party to perform elastic scaling to ensure service quality, e.g., scale out in advance to be faced with big increase of traffic.

Furthermore, the operator will charge 3rd party when 3rd party creates a network slice. Meanwhile, operator can have some refund policy that 3rd party can be refunded if it uses less virtual resources by scale in mechanism. The 3rd party can make a scale in/out plan to improve the resource utilization according to itself consideration. So, network slicing elasticity capability exposed to the 3rd party is beneficial to improve network resource utilization efficiently and save operating costs of operators and 3rd party.

Therefore, we propose a requirement on network slicing elasticity exposure, i.e., the 5G system shall provide suitable APIs to allow a trusted 3rd party to adapt capacity, i.e. elasticity of capacity of a network slice.

### Proposal

***** START OF CHANGES *****

### 6.10.2	Requirements

Based on operator policy, a 5G network shall provide suitable APIs to allow a 3rd party to create, modify and delete network slices used for the 3rd party.

Based on operator policy, the 3GPP system shall allow a 3rd party to define and update the set of services supported in a network slice.

Based on operator policy, the 3GPP system shall allow a 3rd party to assign a device to a network slice based on subscription, device type, and services provided by the network slice.

The 5G network shall provide a mechanism to expose broadcasting capabilities to 3rd party broadcasters’ management systems.

Based on operator policy, a 5G network shall provide suitable APIs to allow a trusted 3rd party to manage this trusted 3rd party owned application(s) in the operator's Service Hosting Environment.

Based on operator policy, the 5G system shall provide suitable APIs to allow a trusted 3rd party to adapt capacity, i.e. elasticity of capacity of a network slice.

***** END OF CHANGES *****