3GPP TSG-SA WG1 Meeting #77	    S1-171036

Jeju Island, South Korea, 13-17 February 2017	(revision of S1-17xxxx)

Title:	Cleaning-up – eMBB KPIs

Agenda Item: 	7.1

Source:	Vodafone

Contact:	Alice Li (alice dot li at vodafone dot com)

Abstract: This document proposes the clean-up pCR to TS22.261 in order to align with NGMN recommendations in RP-161106.

Definitions:

NGMN recommendation
User experienced data rate	The 5%-percentile of the user throughput. User throughput (during active time) of an individual burst is defined as the size of a burst divided by the time between the arrival of the first packet of a burst and the reception of the last packet of the burst.

Area traffic capacity	Total traffic throughput served per geographic area (in Mbit/s/m2).

TS 22.261

User experienced data rate: The minimum data rate required to achieve a sufficient quality experience, with the exception of scenario for broadcast like services where the given value is the maximum that is needed.

Area traffic capacity: The product of the user experienced data rate, the user density, and the activity factor.

NGMN recommended KPI values for different deployment scenarios

| Deployment scenario | KPI | KPI value |
| --- | --- | --- |
| Indoor hotspot | User experienced data rate Area traffic capacity (non-full buffer) for a single operator | DL: 1 Gbps UL: 500 Mbps DL: 15 Tbps/km2 UL: 2 Tbps/km2 |
| Dense urban | User experienced data rate Area traffic capacity (non-full buffer) | DL: 300 Mbps UL: 50 Mbps DL: 250 Gbps/sq km UL: 42 Gbps/sq km |
| Urban macro | User experienced data rate Area traffic capacity (non-full buffer) | DL: 50 Mbps UL: 25 Mbps DL: 33 Gbps/sq km UL: 17 Gbps/sq km |
| Rural macro | User experienced data rate Area traffic capacity (non-full buffer) | DL: 50 Mbps UL: 25 Mbps DL: 0.33 Gbps/sq km UL: 0.17 Gbps/sq km |
| High speed Train | User experienced data rate Users/train Traffic/train | DL: 50 Mbps UL: 25 Mbps 1000 /train DL: 15 Gbps/ train UL: 7.5 Gbps/ train |

NGMN recommended assumptions for deriving Area traffic capacity (non-full buffer) for a single operator

|  | Overall User density | User density (1 out of 3 operators) | Active user average data rate | Activity factor | Average user busy hour usage rate | Area traffic capacity (non-full buffer) |
| --- | --- | --- | --- | --- | --- | --- |
|  | per sq km | per sq km | Mbps | % | Mbps | Gbps/sq km |
| Indoor | 250,000 | 250,000 | DL: 200 UL: 26.67 | 30% | 60 | DL: 15000 UL: 2000 |
| Dense Urban | 25,000 | 8333 | DL: 300 UL: 50 | 10% | 30 | DL: 250 UL: 42 |
| Urban | 10,000 | 3333 | DL: 50 UL: 25 | 20% | 10 | DL: 33 UL: 17 |
| Rural | 100 | 33 | DL: 50 UL: 25 | 20% | 10 | DL: 0.33 UL: 0.17 |
| High speed | 1000/train | 1000/train | DL: 50 UL: 25 | 30% | 15 | DL: 15 bps/train UL: 7.5 bps/train |

*************************************************************1st change***********************************************************

# 3		Definitions, symbols and abbreviations

## 3.1		Definitions

… …

Area traffic capacity: Total traffic throughput served per geographic area.

*********************************************************End of 1st  change******************************************************

*************************************************************2nd change***********************************************************

# 7		Performance requirements

## 7.1		High data rates and traffic densities

… …

Table 7.1-1 Performance requirements for high data rate and traffic density scenarios.

|  | Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 4) | Area traffic capacity (UL) (note 4) | User density (note 4) | Activity factor | UE speed | Coverage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Urban macro | 50 Mbps | 25 Mbps | 33 Gbps/km2 | 17 Gbps/km2 | 3 333/km2 | 20% | Pedestrians and users in vehicles (up to 120 km/h | Full network (note 1) |
| 2 | Rural macro | 50 Mbps | 25 Mbps | 0,33 Gbps/km2 | 0,17 Gbps/km2 | 33/km2 | 20% | Pedestrians and users in vehicles (up to 120 km/h | Full network (note 1) |
| 3 | Indoor hotspot | 1 Gbps | 500 Mbps | 15 Tbps/km2 | 2 Tbps/km2 | 250 000/km2 | note 2 | Pedestrians | Office and residential (note 2) (note 3) |
| 4 | Broadband access in a crowd | 25 Mbps | 50 Mbps | [3,75] Tbps/km2 | [7,5] Tbps/km2 | [500 000]/km2 | 30% | Pedestrians | Confined area |
| 5 | Dense urban | 300 Mbps | 50 Mbps | 250 Gbps/km2 | 42 Gbps/km2 | 8 333/km2 | 10% | Pedestrians and users in vehicles (up to 60 km/h) | Downtown (note 1) |
| 6 | Broadcast-like services | Maximum 200 Mbps (per TV channel) | N/A or modest (e.g., 500 kbps per user) | N/A | N/A | [15] TV channels of [20 Mbps] on one carrier | N/A | Stationary users, pedestrians and users in vehicles (up to 500 km/h) | Full network (note 1) |
| 7 | High-speed train | 50 Mbps | 25 Mbps | 15 Gbps/train | 7,5 Gbps/train | 1 000/train | 30% | Users in trains (up to 500 km/h) | Along railways (note 1) |
| 8 | High-speed vehicle | 50 Mbps | 25 Mbps | [100] Gbps/km2 | [50] Gbps/km2 | 4 000/km2 | 50% | Users in vehicles (up to 250 km/h) | Along roads (note 1) |
| 9 | Airplanes connectivity | 15 Mbps | 7,5 Mbps | 1,2 Gbps/plane | 600 Mbps/plane | 400/plane | 20% | Users in airplanes (up to 1 000 km/h) | (note 1) |
| NOTE 1: For users in vehicles, the device can be connected to the network directly, or via an on-board moving base station. NOTE 2: A certain traffic mix is assumed; only some users use services that require the highest data rates [2]. NOTE 3: For interactive audio and video services, for example, virtual meetings, the required two-way end-to-end latency (UL and DL) is 2-4 ms while the corresponding experienced data rate needs to be up to 8K 3D video [300 Mbps] in uplink and downlink. NOTE 4: These values are based on the assumptions for a single operator. NOTE 5: All the values in this table are targeted values and not strict requirements. |  |  |  |  |  |  |  |  |  |

*********************************************************End of 2nd  change*****************************************************