3GPP TSG-SA WG1 Meeting #77	S1-171078

Jeju Island, South Korea, 13-17 February 2017	(revision of S1-17xxxx)

Title:	FS_eLESTR- Operator controlled Caching service delivery

Agenda Item:	8.6

Source:	China Telecom, Huawei, CATT

Contact:	Xia Xu xiaxu.bri@chinatelecom.cn

Abstract: This document proposes a use case for UEs access to operator controlled Caching content-aware service delivery.

First Change

X.1	Operator controlled Caching service delivery

X.1.1	Description

A user purchases an additional streaming service fromoperator. For this service, a user may subscribe to the operator for interested streaming of audio and video. Also, the operator may suggest to the user some streaming content that maybe of interest the user based on intelligent data learning. The user caching service delivery takes place at different times during the day (peak hour) and night (off-peak hour). Further, the user caching service delivery may also differs from indoor to the outdoor.

X.1.2	Pre-conditions

The operator provides information on how to use the caching service to the customer who has subscribed this service.

Authorized content providers list and data volume are configured into UE.

X.1.3	Service Flows

The operator network provides an indication that subscription streaming services are supported.

The subscribed user uses his UE to watch a show from the operator provided list .

His UE is able to caching the showfor the user at manners (at off-peak hour or indoor) under that subscription.

X.1.4	Post-conditions

The UE disconnects from operator’s streaming service.

The next origination attempt from the user’s  UE goes through the normal session set up process, including authentication. The UE may cach the unseen show or some other show for free during certain time. If user requests any content of the cache list in the UE, the user will be charged or it will be overwritten by new content.

5.1.5	Potential Requirements and Gaps analysis

A 3GPP system shall support a mechanism to indicate to UEs that additional operator streaming services are available.

Based on operator policy, the 3GPP system shall support a mechanism to configure a list of authorized content providers and data volume for cache the streaming services.

Note: There are already content provider specific UE caching but they have some gaps:

  - Not flexible for user

  - Operator cannot get any revenue