3GPP TSG-SA WG1 Meeting #77	S1-171133

Jeju, South Korea 13-17 February 2017	(revision of S1-171xxx)

Title:	SMARTER – definition Editor’s note clean up

Agenda Item:	7.1

Source:				Nokia, Alcatel-Lucent Shanghai Bell

Contact:			Betsy Covell betsy.covell@nokia.com

Abstract: This document proposes to remove the Editor’s note on moving some definitions to 21.905.  As a companion contribution, a CR in S1-171134 proposes to make the corresponding modifications in 21.905.

First Change

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G-RAN: A Radio Access Network connecting to the 5G Core Network which uses NR, E-UTRA, or both.

Active communication: A UE is in active communication when it has one or more connections established. A UE may have any combination of PS connections (e.g., PDP contexts, active PDN connections).

Activity factor: Percentage value of the amount of simultaneous active UEs to the total number of UEs where active means the UEs are exchanging data with the network.

Area traffic capacity: The product of the user experienced data rate, the user density, and the activity factor.

Communication service availability (%): Percentage value of the amount of time the end-to-end communication service is delivered according to an agreed QoS, divided by the amount of time the system is expected to deliver the end-to-end service according to the specification in a specific area.

NOTE 1:  The end point in "end-to-end" is assumed to be the communication service interface.

NOTE 2:  The communication service is considered unavailable if it does not meet the pertinent QoS requirements. If availability is one of these requirements, the following rule applies: the system is considered unavailable in case an expected message is not received within a specified time, which, at minimum, is the sum of end-to-end latency, jitter, and survival time.

Direct network connection: One mode of network connection, where there is no relay UE between the device and the 5G network.

Direct device connection: The connection between two devices without any network entity in the middle.

End-to-end latency: The time that takes to transfer a given piece of information from a source to a destination, measured at the communication interface, from the moment it is transmitted by the source to the moment it is received at the destination.

Editor's Note: The definition of this term in the eV2X TS should be harmonised with this definition.

Hosted Service: A service containing the operator's own application(s) and/or trusted 3rd party application(s) in the Service Hosting Environment, which can be accessed by the user.

Indirect network connection: One mode of network connection, where there is a relay UE between the device and the 5G network.

Network slice: A set of network functions and corresponding resources necessary to provide the required telecommunication services and network capabilities.

NR: The new 5G radio access technology.

Priority service: A service that requires priority treatment based on regional/national or operator policies.

Private network: An isolated network deployment that does not interact with a public network.

Reliability (%): The amount of sent network layer packets successfully delivered to a given node within the time constraint required by the targeted service, divided by the total number of sent network layer packets.

Satellite access: Direct connectivity between the user terminal and the satellite.

Service area: Geographic region where a 3GPP communication service is accessible.

NOTE 3:  The service area can be indoors.

NOTE 4:  For some deployments, e.g., in process industry, the vertical dimension of the service area can be considerable.

Service continuity: The uninterrupted user experience of a service that is using an active communication when a UE undergoes an access change without, as far as possible, the user noticing the change.

NOTE 5: 	In particular service continuity encompasses the possibility that after a change the user experience is maintained by a different telecommunication service (e.g., tele- or bearer service) than before the change.

NOTE 6: 	Examples of access changes include the following. From pre-5G: CS/PS domain change. From 5G: switching between a direct network connection and an indirect network connection.

Service Hosting Environment: The environment, located inside of 5G network and fully controlled by the operator, where Hosted Services are offered from.

Survival time: The time that an application consuming a communication service may continue without an anticipated message.

Tactile internet: System where a human controls real or virtual objects by tactile input, and receives tactile, audio, or visual feedback of his actions. The feedback has to arrive fast enough to give the impression of zero-latency control.

User experienced data rate: The minimum data rate required to achieve a sufficient quality experience, with the exception of scenario for broadcast like services where the given value is the maximum that is needed.

Wireless backhaul: Provides interconnection link between 5G network nodes and/or transport network using 5G radio access technology.