3GPP TSG-SA WG1 Meeting #77	S1-171339

Jeju Island, South Korea, 13-17 February 2017	(revision of S1-171086)

Title:	Virtual Coupling

Agenda Item:	8.2 FS_FRMCS

Source:	Korea Railroad Research Institute

Contact:	Sang Won Choi <swchoi@krri.re.kr>

Abstract: This contribution proposes a new service concept of train operation, in which vehicles in close distance moves together without physical coupling. Similar to the platooning scenarios in road, successive vehicles exchange instantaneous control information for autonomous train control within a long braking distance.

--------------------------------------- Start of Change ----------------------------------------

9.x Virtual Coupling

9.x.1 Description

One of the important missions that the future railway service should achieve is to increase its transport capacity. A straight-forward solution is to minimize the distance between successive trains so that train interval is reduced. It is difficult to do so in a legacy train control system, because the successive trains need to have distance as much as a safety margin, which should be larger than the full braking distance.

Figure  9.x.1 Sharing acceleration and braking control information to shorten safety margin

Figure  9.x.1 Sharing acceleration and braking control information to shorten safety margin

This safety margin can be further shortened if the successive trains share control information (acceleration and braking) and apply it to its own train control. As shown in Figure 9.x.1, the safety margin can be shortened if the following train immediately knows that the leading train starts braking and also triggers braking. This is the fundamental principle and the main purpose of virtual coupling. Figure 9.x.2 shows the basic concept of the virtual coupling scenario. Multiple trains which are in close distance move together as they are physically coupled. As the distance between two trains gets smaller, the control information of a train should be delivered to the other in shorter time.

Figure  9.x.2 The concept of virtual coupling scenario

9.x.2.	Pre-conditions

1. The leading and following trains in operation recognize each other and have just got configured to be coupled virtually.

2. The user equipments for train control support device to device communications in the perspective of transport layer.

Figure  9.x.3 Overall procedure of virtual coupling

9.x.3	Service Flows

1. The following train begins to approach to the leading train by exchanging the information about their position. The two trains, which are far enough to allow a certain amount of end-to-end latency, are initially connected through the 3GPP network.

2. As the following train approaches to the leading train, they start exchanging the information about movement control and each train then controls itself while considering the control of the other train. The two trains, which are still far enough to allow a certain amount of end-to-end latency, keep the connection through the 3GPP network.

3. As the following train further approaches to the leading train, the safe braking distance gets shorter and they are required to have a connection of which end-to-end latency is very short for delicate train control. At this moment, they seamlessly switch the connection from the 3GPP network to PC5 interface

9.x.4	Post-conditions

For safety train operation, integrity needs to be checked for train control information delivery in the perspective of application or transport layer. Any error on integrity check shall be immediately reported to the train control application.

9.x.5	Potential requirements and gap analysis

| Requirement Reference Number | Requirement text | Application/Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [9.x-001] | An On-board FRMCS UE for train control shall communicate through off-network up to 3km (TBD) in the line of sight channel environment. | T | Partly supported |  |
| [9.x-002] | Integrity protect shall be checked for each data transfer between FRMCS UEs for train control | A/T | Partly supported |  |
| [9.x-003] | The end to end delay between FRMCS UEs for train control communicating through PC5 shall be less than 10 ms (TBD). | T | Not supported |  |
| [9.x-004] | The 3GPP system shall support service continuity between on-network based connection and off-newtork based connection. | T | Not supported |  |
| [9.x-005] | FRMCS UEs for train control shall communicate through off-network, where the FRMCS UEs’ relative speed is less than 50km/h (TBD). | T | Not supported |  |