3GPP TSG-SA WG1 Meeting #78	S1-172044

# Porto, Portugal, 8-12 May 2017												(revision of S1-17xxxx)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.889 | CR | 0001 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the gap analysis for real time data transmission in section 7.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_FRMCS |  |  |  |  |  |  | Date: |  | 2017-04-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | As currenlty majority of the potential requirements in 7.4 use case are already covered by TS 22.282/22.280, for example, it already supports for real-time data communication in TS 22.282, as [R-6.1.3.2-001] The MCData Service shall provide a mean to share in real-time operational status information between members of a selected group. .Other potential requirements also covered as explaining in the “comment” column. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To change “not covered” to “covered by 22.282” or “covered by 22.280” in “SA1 spec covering” column, and update the comment column for each requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The currently gap analysis results are wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***************************** Start of Change  ***************************************

## 7.4	Use case: Data transmission in real time

### 7.4.1	Description

This use case allows the authorized FRMCS Users to exchange data in real time from each other, for example, health and status of train are transferred between intelligent on-train systems and train maintenance.

### 7.4.2	Pre-conditions

The user A is authorized FRMCS User (e.g., administrator who has authorization to the data).

User B is an equipment for monitoring the train.

User C is a normal FRMCS User (e.g., user without authorization to the data)

### 7.4.3	Service flows

User A requests User B to transmit the data (e.g., health, status of train, etc.). in real time.

User B receives the request from User A, and accepts the request.

User B starts to transmit the requested data in real time to User A.

User C requests User B to transmit the data (e.g., health, status of train, etc.) in real time.

User B receives the request from C, and rejects the request (e.g., without authorization to the data).

User C receives a notification that the request was rejected by User B.

### 7.4.4	Post-conditions

The data of User B is sent to User A in real time.

The data of User B is not sent to User C in real time.

### 7.4.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-7.4.5-001] | The FRMCS System shall enable an authorized FRMCS-user to request another FRMCS User to establish real time data transmission. | A | covered TS 22.282, in [R-6.1.3.2-001] | This requirement is covered [R-6.1.3.2-001] The MCData Service shall provide a mean to share in real-time operational status information between members of a selected group |
| [R-7.4.5-002] | The FRMCS System shall provide a list of identities for the requesting authorized FRMCS User to select from. The list may contain functional identities, FRMCS User identities, or FRMCS Equipment Identities | A | covered TS 22.282, In [R-5.2.2-009] | This requirement is covered [R-5.2.2-009] The MCData Service SDS shall provide a selectable read receipt indication. When requested, the receiving entity shall provide receipt indication for delivered and read messages as appropriate. The read indication will implicitly indicate delivered as well. |
| [R-7.4.5-003] | The real time data request shall either be based on the functional identity, FRMCS User Identity, or FRMCS Equipment Identity. | A | Not covered |  |
| [R-7.4.5-004] | The FRMCS User shall be able to accept a real time data transmission request. | A | covered TS 22.282, in[R-5.3.2-003] | This requirement is covered [R-5.3.2-003] The MCData file distribution capability shall provide an option for each recipient to choose to receive the file or not (e.g. by storing the file and sending a link (URL) to all relevant members). By using this capability a user may choose when to receive the file. |
| [R-7.4.5-005] | The FRMCS User shall be able to reject a data in real time transmission request. | A | covered TS 22.282, in [R-5.3.2-004] | This requirement is covered [R-5.3.2-004] The MCData file distribution capability shall allow a user to reject to receive the file where appropriate. |
| [R-7.4.5-007] | The requesting FRMCS User shall be notified if the request is rejected or ignored. | A | covered TS 22.280, in [R-6.19.1.1-006] | This requirement is covered [R-6.19.1.1-006] An MCX Service User shall be notified when his request to transmit is removed from the request queue. |

***************************** End of  Change  ***************************************