3GPP TSG-SA WG1 Meeting #79	S1-173507

# Guilin, CHINA，21-25 Aug，2017		                                (revision of S1-173364 rev of 3087)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.179 | CR | 0054 | rev | 2 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | clarify MCPTT Group in clause 6.2.3.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MONASTERY |  |  |  |  |  |  | Date: |  | 2017-08-24 |
| Category: | D |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In section 6.2.3.7, - there are some different descriptions for the “group”, e.g., a MCPPT Group, the group, an MCPTT Group. To align the description with the other places in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To change “a MCPTT Group” and “a group” into “an MCPTT Group” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Same idea with different wording may lead to different understanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.7.1, 6.2.3.7.2, 6.2.3.7.3, 6.2.3.7.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the change  ***************

6.2.3.7	MCPTT Groups configured for multi-talker control

6.2.3.7.1	Overview

The multi-talker control applies to designated MCPTT Groups and results in allowing several Participants talking simultaneously within the MCPTT Group. For example, Multi-talker control is used by railway communication e.g. during shunting operation.

When an MCPTT Group is configured for multi-talker control, the requirements listed below apply.

6.2.3.7.2	General aspects

[R-6.2.3.7.2-001]  An MCPTT Group shall be configurable to allow multi-talker control.

[R-6.2.3.7.2-002] The MCPTT Service shall provide a mechanism for multiple MCPTT Users to talk simultaneously in an MCPTT Group configured for multi-talker control.

[R-6.2.3.7.2-003] The MCPTT Service shall determine which Participant(s) are allowed to transmit to all other Participant(s) in an MCPTT Group configured for multi-talker control.

[R-6.2.3.7.2-004] The MCPTT Service shall support all Participant(s) to receive audio from all other Participant(s) that are transmitting in an  MCPTT Group configured for multi-talker control.

[R-6.2.3.7.2-005] The MCPTT Service shall provide a mechanism for the MCPTT Administrator to configure the maximum number of simultaneous talkers in an MCPTT Group configured for multi-talker control.

[R-6.2.3.7.2-006] The MCPTT Service shall allow an authorized MCPTT User to change the maximum number of simultaneous talkers at any time during a group call in an MCPTT Group configured for multi-talker control.

6.2.3.7.3	Requesting permission to transmit

[R-6.2.3.7.3-001] The MCPTT Service shall enable authorized Participants to request to transmit to an MCPTT Group configured for multi-talker control.

[R-6.2.3.7.3-002] At call setup the MCPTT Service shall provide a notification, for example audio and/or visual, to the MCPTT Group Member attempting to transmit that there are no other Group Members who have affiliated to the MCPTT Group configured for multi-talker control.

[R-6.2.3.7.3-003] The MCPTT Service shall determine the transmitting Participant(s) when there are simultaneous requests for permission to transmit within the same call for an MCPTT Group configured for multi-talker control.

[R-6.2.3.7.3-004] Following an MCPTT Request for permission to transmit on the Selected MCPTT Group configured for multi-talker control the MCPTT Service shall provide an Affiliated MCPTT Group Member that made and was granted the request an indication of being allowed to transmit.

6.2.3.7.4	Override

6.2.3.7.4.1	General aspects

[R-6.2.3.7.4.1-001] If the number of MCPTT Users requesting the permission to talk exceeds the maximum number of simultaneous talkers in an MCPTT Group configured for multi-talker control, the MCPTT Service shall apply the override mechanism.

[R-6.2.3.7.4.1-002] The MCPTT Service shall enable MCPTT Administrators to create a priority hierarchy for determining what Participants, Participant types, and urgent transmission types shall be granted a request to override an active MCPTT transmission on an MCPTT Group configured for multi-talker control.

[R-6.2.3.7.4.1-003] The priority hierarchy used for granting a request to override an active MCPTT transmission on a group configured for multi-talker control shall contain at least four (4) levels.

[R-6.2.3.7.4.1-004] The transmitting Participant on an MCPTT Group configured for multi-talker control shall be determined by the relative priorities of the Participants and Call type based on priority (e.g MCPTT Emergency).

[R-6.2.3.7.4.1-005] Transmission requests of Participants with insufficient relative priority shall be rejected.

[R-6.2.3.7.4.1-006] The MCPTT Service shall provide a mechanism for Participants, to override an active MCPTT transmission of a transmitting Participant when the priority level of the overriding Participant or Call type based on priority (e.g MCPTT Emergency) are ranked higher than the priority level of the transmitting Participant or Call type based on priority for an MCPTT Group configured for multi-talker control.

[R-6.2.3.7.4.1-007] If an authorized Participant overrides an MCPTT transmission, the MCPTT Service shall provide a means of notifying the overridden Participant(s) that the transmission has been overridden for an MCPTT Group configured for multi-talker control.

[R-6.2.3.7.4.1-008] The MCPTT Service shall revoke the transmit permission of the overridden transmitting Participant on an MCPTT Group configured for multi-talker control.

*************** End of the change  ***************