3GPP TSG-SA WG1 Meeting #79	S1-173103

# Guilin, China, 21-25 August 2017													(revision of S1-17xxxx)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0041 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enable CS fallback from 5G to 3G |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | XXX |  |  |  |  |  |  | Date: |  | 2017-08-10 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 4G network development has expanded to a global scale, but the limited deployment of VoLTE based emergency call, wide range of interoperability deployment, international roaming application, remind us to reconsider the services of 5G which has not include 3G capabilities. So far, some LTE networks do not deploy IMS and rely on CSFB to provide voice service. In Rel15 SMARTER, the interworking for voice service between 5G and 3G is excluded. Thus the MNO cannot provide quality voice if a UE is leaving 5G coverage to 3G coverage. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove current exclusion in order to support interworking for voice service between 5G and 3G |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | the MNO cannot provide quality voice if a UE is leaving 5G coverage to 3G coverage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***************************** Start of 1st Change  ***************************************

5	High-level requirements

5.1	Migration to 5G

5.1.1 	Description

The 5G system supports most of the existing EPS services, in addition to many new services. The existing EPS services may be accessed using the new 5G access technologies even where the EPS specifications might indicate E-UTRA(N) only. Only new or changed service requirements for new or changed services are specified in this TS. The few EPS capabilities that are not supported by the 5G system are identified in clause 5.1.2.2 below.

5.1.2	Requirements

5.1.2.1	Interworking between 5G systems

The 5G system shall support a UE with a 5G subscription roaming into a 5G Visited Mobile Network which has a roaming agreement with the UE's 5G Home Mobile Network.

The 5G system shall enable a Visited Mobile Network to provide support for establishing home network provided data connectivity as well as visited network provided data connectivity.

The 5G system shall enable a Visited Mobile Network to provide support for services provided in the home network as well as provide services in the visited network. Whether a service is provided in the visited network or in the home network is determined on a service by service basis.

The 5G system shall provide a mechanism for a network operator to limit access to its services for a roaming UE, (e.g., based on roaming agreement).

The 5G system shall provide a mechanism for a network operator to direct a UE onto a partnership network for routing all or some of the UE user plane and associated control plane traffic over the partnership network, subject to an agreement between the operators.

5.1.2.2	Legacy service support

The 5G system shall support all EPS capabilities (e.g., from TSs 22.011, 22.101, 22.278, 22.185, 22.071, 22.115, 22.153, 22.173) with the following exceptions:

-	CS voice service continuity and/or fallback to GERAN ,

-	seamless handover between NG-RAN and GERAN,

-	seamless handover between NG-RAN and UTRAN, and

-	access to a 5G core network via GERAN or UTRAN.

***************************** End of  Change  ***************************************