3GPP TSG-SA WG1 Meeting #80	S1-174033

Reno, USA, Nov 27 – Dec 1, 2017

Title:	Mandatoriness of access category in Unified Access Control

Agenda Item:	4

Source:	LG Electronics Inc.

Contact:	SungDuck Chun <sungduck.chun (at) lge.com>

Abstract: This paper discuss whether multiple matching occurs and whether all access categories should be supported.

- Introduction

SA1 received LSes from SA2/CT1/RAN2 regarding access control. One of issue is whether an access attempt, regardless of what definition is, can trigger mapping to multiple access categories.

- Support of various operating options.

Before discussing issue of matching multiple access categories, following needs to be considered.

ACDC

ACDC supports different operators to differently categorize applications into ACDC categories. Depending on operator’s choice, number of active ACDC categories also can be different among operators.

- The home network shall be able to configure a UE with at least four and a maximum of sixteen ACDC categories to each of which particular, operator-identified applications are associated. The categories shall be ordered as specified in sub-clause 4.3.5.2.2. Note: Provisioning of the ACDC categories in the UE is the responsibility of the home network, and the categorization is outside the scope of 3GPP.

To support the case of different number of category, requirements related to default category and different number of categories were specified in the TS 22.011

Observation 1:

The use of number of categories and how to categorize can be different per operators.

Support of IoT or not

In current TS22.261 following is captured:

2 (NOTE 3); UE is configured for delay tolerant service and subject to access control for access category 2, which is judged based on relation of UE’s HPLMN and the registred PLMN.; All.

Depending on operator’s policy or strategy, the operator may decide not to support any IoT UEs or services in its network. In that case, there is no need for the operator to support/configure IoT UE.

Observation 2:

Operator may decide not to support/implement any IoT related service.

If a PLMN does not support IoT, there is no need to signal anything related to Access Category 2.

Observation 3:

If IoT is not supported in a network, there is no need to signal anything related to Access Category 2.

Support of emergency/special AC

Except regulatory requirements, a network may not support emergency calls or special ACs. If so, there is no need to signal information related access category 1 or 3.

Observation 4:

If network does not support emergency call or special Access class, there is no need to signal information related to access category 1 and 3.

Operator specific support

In case of IoT-focused operator which only supports IoT traffic, the operator may want to categorize IoT services into different access categories.

For example, one access category can be allocated to IoT devices for logistics, another access category can be allocated to IoT devices for remote weather sensors. Then, operator A may decide not use standardized access category 2 for delay tolerant services, but it may use operator-defined access category 40 for logistics IoT and access category 41 for weather sensors IoT. But, other operator B may not bother to further categorize delay tolerant IoT services and decides to use access category 2 for all delay-tolerant IoT service.

On the other hand, if a network does not support IoT services, there is no need to deliver any information related to access category 2. Actually, for EPS, following can be found:

If the network is not broadcasting the EAB information, the UE shall be subject to access barring as described in clause 4.3.1

Thus, it is not mandatory for operator to support IoT or access-control based on delay tolerant service.

Other situation is also possible. For example, a UE is delivered with access category mapping information only for access category 50 and 51 in the range of operator-defined access category. However, the network may broadcast access category barring information also for access category 55 and 56. This may occurs when the network does not want to allow certain UEs to use that access category. For example, that access category 55 can be used for critical communication, but other normal UEs do not need to know that.

Observation 5:

Depending on operator policy and need, some standardized access categories are not used/broadcast but operator-defined access categories can be used instead. Depending on scenario, access barring information for certain access category can be broadcast while some UEs does not know mapping information for that access category.

- Support of roaming scenario

In current TS, following is stated:

8; All except for the cases of access categories 1-2.; MO data that do not belong to any other access categories. 9-31; Reserved standardized access categories. 32-63; All except for the cases of access categories 1-2 and except for roaming-UEs; Based on operator classification.

Unfortunately, SA1 had not much discussion on how access category 32-63 should be handled, considering roaming scenario. In previous meeting, there was no discussion on:

- Who configures operator-defined access categories? HPLMN only? Or serving PLMN?

- Whether information on operator-defined access category is static? Or dynamically updated?

- Whether requirement are different based on Home-routed scenario and Local-breakout scenario?

- How VPLMN can specifically control access of specific application also for roaming UE as it applies the same principles to its own subscribers?

Observation 6:

Further discussion is needed for applicability of operator-defined access category in roaming scenario.

In principle, for a roaming case, there is an opportunity for a network to re-configure a UE with access category information that is customized to the serving PLMN.

To get basic services, first thing that a UE should do is to register itself to a visited network. Before this registration, the access attempt of the UE falls into one of following:

- Mobile originated signalling for registration. I.e., ATTACH/REGISTRATION message is the first message from the UE. Before this registration, there is no user plane data and no paging message. I.e., other access categories are not used.

- Emergency call. Even before registration or after registration failure, emergency call can be triggered if network supports.

The other access attempt (e.g., MO data, Paging) does not occur until successful registration. Thus, during this registration, serving PLMN can configure UE with relevant information, for example:

- Whether the UE is configured as ‘delay-tolerant’ in serving PLMN

- Whether the UE is configured as ‘prioritized user’ in serving PLMN

Actually, for EAB in EPS, following is found:

- EAB information shall define whether EAB applies to UEs within one of the following categories: a) UEs that are configured for EAB; b) UEs that are configured for EAB and are neither in their HPLMN nor in a PLMN that is equivalent to it; c) UEs that are configured for EAB and are neither in the PLMN listed as most preferred PLMN of the country where the UE is roaming in the operator-defined PLMN selector list on the SIM/USIM, nor in their HPLMN nor in a PLMN that is equivalent to their HPLMN

I.e, a PLMN can provide different barring information based on how the registered PLMN is related to the UE. If similar/higher level of granularity of UEs are required to control access from IoT UEs, operator-defined access category can be used.

It is possible for network to indicate whether the UE can be provided with specific services for e.g. IoT, or with prioritized access based on special access class. If VPLMN does not support, it will not configure roaming UE with access category information including ‘delay-tolerant’ or prioritized access. This automatically mean that access category 2 is not relevant for the UE. And, if ‘delay-tolerant’ or prioritized access needs to be supported, VPLMN can indicate relevant operator-defined access category information by configuration message during registration.

Observation 7:

Information on access category mapping can be delivered to UE during registration or with pre-configuration per PLMN.

Observation 8:

UE characteristic (e.g. delay tolerant, prioritized access) can be considered/configured when network delivers access category mapping information to UE.

BTW, there are two elements regarding information for access category.

- Mapping information

  - This information indicates the mapping between access attempt and access category

- Barring information

  - This information indicates barring factors for a certain access category

From SA1’s perspective, both information should be delivered to UE to control congestion/overload. When or how this information is delivered to UE is stage-2/3 issue. However, if both information is broadcast over SIB, all UEs within a same network will behave in a same way. I.e, the same operator-defined access category information can be delivered to any UE in that network, regardless of whether it is roaming or not.

Observation 9:

It is possible to make all UEs within the same PLMN have the same information regarding operator-defined access categories.

- Issues among standardized access categories

Emergency call

In the current TS 22.261, following is captured.

2 (NOTE 3); UE is configured for delay tolerant service and subject to access control for access category 2, which is judged based on relation of UE’s HPLMN and the registred PLMN.; All. 3; All except for the cases of access categories 1-2.; Emergency.

This table seems to suggest that ‘emergency’ call is not supported for UE configured for ‘delay tolerant’. But, in EPS, even ‘delay-tolerant’ UE can use ‘emergency call’.

Observation 10:

It is not clear why ‘delay-tolerant UE’ cannot use access category for ‘emergency’

MO signalling

Access category number; Conditions related to UE; Type of access attempt. 0 (NOTE 1); All; MO signalling resulting from paging. 4; All except for the cases of access categories 1-2.; MO signalling.

In the description of access category 4, there is no limitation regarding ‘MO signalling’. Thus, “MO signalling” also covers “MO signalling resulting from paging”

Observation 11:

“MO signalling resulting from paging’ can be mapped to both access category 0 and access category 4

Order

Regarding observation 11, one can argue that order of access category is same as the order of priority. If this is the case, UE should check whether an access attempt is mapped, for example, to access category 0 before checking access category 4.

However, this seems not always true.

8; All except for the cases of access categories 1-2.; MO data that do not belong to any other access categories. 9-31; Reserved standardized access categories. 32-63; All except for the cases of access categories 1-2 and except for roaming-UEs; Based on operator classification.

For access category 8, the description says that there is no match for other access categories. Thus, before checking access category 8, the UE should first check whether access category 9-63 matches an access attempt or not.

Observation 12:

There is no relationship between the order of access category in the current TS and the order with which the UE should check whether an access attempt is mapped to certain access category or not. Current specification does not imply that lower numbered access category has higher priority than higher numbered access category.

BTW, if priority order is provided to each access category, then multiple matching to access category can be avoided. For example, similar mechanism as defined for TFT in section 4.7.2.1 of TS 23.401 can be considered.

Observation 13:

If there is a rule on which order mapping to an access category is checked, it may be easy to select only one of access categories. Thus, issue resulting from matching to multiple access categories can be ignored.

- Issues among standardized access category and operator-defined access category

Whether one access attempt can be mapped simultaneously to one of standardized access category and one of operator-defined access category depends on how an operator wants to categorize access attempts.

For example, as discussed in previous section, an operator has several choices:

- Choice 1: To use access category within standardized access category, and not to define any operator-defined access categories for similar purpose.

  - E.g., use access category 3 and do not define any delay-tolerant services in operator-defined access category.

- Choice 2: Not to use an access category within standardized access category and to define/use operator-defined access categories for similar purpose. All services of similar purpose should be mapped to one of the operator-defined access categories

  - For example, if operator wants to handle differently MO signalling based on core network nodes (AMF signalling, SMF signalling), the operator does not use/broadcast access category 4. Instead, the operator can assign access category 40 to ‘MO signalling for MM procedure’ and access category 41 to ‘MO signalling for SM procedure’

With observation 9, UEs can use operator-defined access categories even before registration to a network. Thus, as long as network delivers properly-composed access category mapping information, simultaneous matching to multiple access category can be avoided.

Observation 14:

With proper configuration, access attempt from a UE can mapped to only one access category.

- Proposal

Below is text proposal based on above discussion.

Based on discussion paper [1], [2] and [3], CR consolidating proposals in [1], [2] and [3] is in [4] and related outgoing reply LS is in [5].

- Reference

[1] S1-174032, Access Class in Unified Access Control

[2] S1-174033, Mandatoriness of Access Category in Unified Access Control

[3] S1-174044, other topics of Access Category in Unified Access Control

[4] S1-174045, CR to 22.261 on Unified Access Control

[5] S1-174046, [draft] reply LS on Unified Access Control

[6] C1-174626, LS on requirements on unified access control for 5GS

[7] LS from RAN2

[8] S2-178191, 	Reply LS on unified Access Control for 5G NR

* * * * Start of First Change * * * *

### 6.22.1	Description

Depending on operator policies, deployment scenarios, subscriber profiles, and available services, different criterion will be used in determining which access attempt should be allowed or blocked when congestion occurs in the 5G System. These different criteria for access control are associated with access categories with minimized inter-dependency among the different access control categories. The 5G system will provide a single unified access control where operators control accesses of each category.

In unified access control, each access attempt is categorized into one of the access categories. There are two types of access categories. One type is standardized access categories and the other is operator-defined access categories. Standardized access categories is used for well-known access attempts and common to all operators. Based on the access control information applicable for the corresponding access category of the access attempt, the UE performs a test whether the actual access attempt can be made or not. An access attempt is not mapped to an access category for which the access control information is not broadcast by the network.

The unified access control supports extensibility to allow inclusion of additional standardized access categories and supports flexibility to allow operators to define operator-defined access categories using their own criterion (e.g. applications, network slicing aspects). This flexibility also support operator to use operator-defined access categories instead of standardized access categories.

Additionally, the use of legacy access classes 11-15 is supported to potentially allow an access attempt to succeed that otherwise might have been barred.

### 6.22.2	Requirements

Based on operator’s policy, the 5G system shall be able to prevent UEs from accessing the network using relevant barring parameters that vary depending on access category. Access categories applicable in a cell is mutually exclusive.

Access category number and criterion for an access category is listed in Table 6.22.2-1.

Table 6.22.2-1: Access Categories

| Access category number | (MEMO: NOT USED COLUMN ) | Criterion |
| --- | --- | --- |
| 0 (NOTE 1) | All | MO signalling resulting from paging |
| 1 (NOTE 2) | One or some of Access Classes 11-15 are set. At least one of them is valid in the registered PLMN and justified its priority handling by the registered PLMN with regards to access control. | All |
| 3 | All except for the cases of access categories 1-2. | Emergency |
| 4 | All except for the cases of access categories 1-2. | MO signalling resulting from other than paging |
| 5 | All except for the cases of access categories 1-2. | MMTEL voice |
| 6 | All except for the cases of access categories 1-2. | MMTEL video |
| 7 | All except for the cases of access categories 1-2. | SMS |
| 8 | All except for the cases of access categories 1-2. | MO data that do not belong to any operator-defined access categories |
| 9-31 |  | Reserved standardized access categories |
| 32-63 | All except for the cases of access categories 1-2 | Based on operator classification |
| NOTE 1: Access category 0 is not barred. NOTE 2: Access Classes 11 and 15 are valid in Home PLMN only if the EHPLMN list is not present or in any EHPLMN. Access Classes 12, 13 and 14 are valid in Home PLMN and visited PLMNs of home country only. For this purpose the home country is defined as the country of the MCC part of the IMSI. If the barring control information contains flag for “unbarred” for at least one of these valid Access Classes, all access attempts from the UE require priority handling and fall into access category 1. Otherwise the UE does not require priority handling with regards to access control and other access categories apply. Access category 1 is not barred. NOTE 3: The barring parameter for access category 2 is accompanied with information on whether the access control applies to UEs registered in UE’s HPLMN/EHPLMN, the most preferred VPLMN, or other PLMNs. |  |  |

The 5G network shall be able to provide UE with access category mapping information which indicate what kind of access attempt can be mapped to a specific operator-defined access category.

The 5G network shall be able to broadcast barring control information (i.e. a list of barring parameters associated with an access category) in one or more areas of the RAN.

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and access category of the access attempt.

In the case of multiple core networks sharing the same RAN, the RAN shall be able to apply access control for the different core networks individually.

The unified access control framework shall be applicable both to UEs accessing the 5G CN using E-UTRA and to UEs accessing the 5G CN using NR.

The unified access control framework shall be applicable to UEs in RRC Idle, RRC Inactive, and RRC Connected at the time of initiating a new access attempt (e.g. new session request).