3GPP TSG-SA WG1 Meeting #84 	S1-183064

Spokane, USA, 12 – 16 November

Title:	Definitions for TR22.827

Agenda Item:	8.3

Source:	EBU, BBC

Contact:	Ian.Wagdin@bbc.co.uk

Abstract: This contribution proposes definitions to be added to section 3.1 of TR22.287, i.e. audio-visual content and service production to be supported by 3GPP systems.

---------- begin new text ----------

| Term | Definition | Reference document(s) |
| --- | --- | --- |
| Broadcast Centre | A central building or headquarters of a broadcaster where all transmission originates from. This may include studio facilities and/or technical areas where content is received, routed, created and managed. Typical Broadcast centres act as hubs for live content as well as playout centres for pre-recorded materials |  |
| Remote Production | A production where the content being acquired is remote to the broadcast centre but controlled from the centre. this may include single or multiple sources such as video or audio content but also command and control functions to operate the technical facilities located at the production site. | https://www.thebroadcastbridge.com/content/entry/8031/the-benefits-of-remote-production-for-live-events |
| Outside Broadcast (OB) | A production where content is being acquired remote to the broadcast centre but controlled from the location. usually requires communication with the broadcast centre but facilities are located at the production site usually in a mobile OB truck. Generates output for broadcast which may be sent back to the broadcast centre for inclusion into a programme or distribution. as an outside source****** | https://en.wikipedia.org/wiki/Outside_broadcasting |
| SDI | Serial Digital Interface, a standard by which video and audio signals are transported digitally, usually over an coax cable defined in SMPTE 292M. A number of different protocols have been implemented in order to support higher resolutions and framerates with the current maximum being 12 GB/s to support 4K UHD at 50 fps | https://en.wikipedia.org/wiki/Serial_digital_interface |
| Production (TV) Studio | Area used to create media content usually consists of a studio floor with cameras, presenters and microphones which are controlled from the production gallery. |  |
| Production (TV) Gallery | An area in a TV studio where producers, directors and technical staff work together to produce content. functions include control of cameras, lighting, sound and video feeds. brings together feeds from both local and outside sources |  |
| Control Room | the place in a broadcast centre where outside sources are received, monitored and routed to the Production Gallery |  |
| SMPTE 2110 | SMPTE ST 2110 standards suite specifies the carriage, synchronization, and description of separate elementary essence streams over IP for real-time production, playout, and other professional media applications. | https://www.smpte.org/st-2110 |
| Broadcast over IP | carriage of broadcast signals over an IP network |  |
| audio/ video contribution | audio or Video content sent from the location to broadcast centre to become programme content |  |
| Audio Clean feed (mix minus) | Programme output (minus Audio Contribution) sent from broadcast centre to Reporter so the reporter can hear studio output also known as a mix-minus | https://en.wikipedia.org/wiki/Mix-minus |
| Video Clean Feed | Studio output with no graphics or text overlays |  |
| cue/talkback | Audio messages sent from broadcast centre to location usually to instruct a radio reporter when to speak. not audible in broadcast audio |  |
| PTP | precision time protocol (IEE 1588) used to timestamp audio or video content |  |
| codec | Device (hardware or software deployed) used to Compress (and/or decompress) and encode audio or video |  |
| PA system | Public Address system or large scale amplification used at music events. |  |
| multicam | the use of 2 or more cameras in an OB which can be cut between, important considerations are colour imagery, timing, framing and picture size and frequency |  |
| NPN | Non-public Network- network dedicated to specific users. |  |
| Uncompressed Video | Uncompressed video is digital video that either has never been compressed or was generated by decompressing previously compressed digital video. It is commonly used by video cameras, video monitors, video recording devices (including general purpose computers), and in video processors that perform functions such as image resizing, image rotation, deinterlacing, and text and graphics overlay. It is conveyed over various types of baseband digital video interfaces, such as HDMI, DVI, DisplayPort and SDI. Standards also exist for carriage of uncompressed video over IP (ST2110) | https://en.wikipedia.org/wiki/Uncompressed_video |
| frame rate (fps) | the amount of video frames in a second that need to be captured to assure smooth motion. typically dictate by the base system clock which was historically generated from the mains frequency. In 50 Hz regions a 25 fps (frames per second) was standard with a migration to 50 fps for higher resolutions. in 60 Hz countries this is reflected as 30 fps and 60 fps. |  |
| Resolution | Also known as raster size this is the resolution of the video image being captured. It is usually defined by the number of horizonal lines but in combination with the aspect radio this defines a whole image size eg 1920 x 1080 is an HD image with 1080 x 1920 pixels this commonly combined with a frame rate and also a letter to define if the picture is interlaced or progressive eg 1080p50 is a progressive image at 50 fps | https://en.wikipedia.org/wiki/Display_resolution |

---------- end new text ----------