3GPP TSG-SA WG1 Meeting #85 	S1-190114

Tallinn, Estonia, 18 - 22 February 2019	(revision of S1-xxxxx)

Title:	FS_5G_ATRAC – Corrections on Container and Wagon use cases

Agenda Item:	8.5

Source:	Novamint

Contact:	Thierry Bérisot, tberisot@novamint.com

Abstract: This contribution proposes to make small corrections in TR22.836 mostly editorials and to add one potential requirement in the fourth changes.

------------------------- START OF FIRST CHANGES ----------------------------

### 5.1.6	Potential New Requirements needed to support the use case

Note: 	All new requirements associated to relay to support this use case will be addressed in TR 22.866 (enhanced Relays for Energy Efficiency and Extensive Coverage).

| Reference number | Requirement text | Comment |
| --- | --- | --- |
| 5.1.6-1 | The 5G system shall be able to support a 12 years’ lifetime of the service without changing the UE or the battery of the UE. Note: This assumes a message of 200 bytes every hour (24 messages a day) with a battery of capacity of 21,6 Wh. |  |
| 5.1.6-2 | The UE shall be able to support a payload size up to 2500 bytes per message. | The maximum payload size for each message in NB-IoT is 1600 bytes. |
| 5.1.6-3 | The 5G System shall be able to support a very high density of UE in the same area (up to [30.000] UE in [21.500] m2). | Equivalent to 1,4 UE per m2 or 1,4 Million UE per km2 – to compare with 1 Million UE per km2 (current KPIs). |
| 5.1.6-4 | The 5G system shall be able to support cellular IoT/mMTC type of communications by satellite. |  |

------------------------- END OF FIRST CHANGES ----------------------------

------------------------- START OF SECOND CHANGES ----------------------------

### 5.2.3	Service Flows

A freight train is composed of 19 freight wagons of different varieties and usages. Each wagon has been tracked individually to be placed in a certain order in order to compose the train. Not all wagons may be equipped with an UE.

One of the main challenges is to communicate the information to the conductor as well with the WAN network on such a length and as the UE attached to the wagon can be in different places depending on the type of wagons. This needs to be done while still optimising the battery level of the UE attached to wagon as well as the fact that part of the train and some of the wagons can be in a tunnel where the propagation of the signal is limited. Another challenge is to communicate the information in a relative short time to allow the conductor as well as the train operator to react appropriately: 15 secondes from the time there is an event detected and the event is communicated to the locomotive.

At the constitution of the train, the conductor of the train triggers through the gateway in the locomotive the establishment of a dedicated chain of relays between all wagons of the train that are equipped with an UE to route the data as illustrated by figure 5.

Figure 1: Constitution of a virtual dedicated chain of relays initiated by the locomotive gateway

This chain of relays is always managed by the gateway of the locomotive and will be valid for the whole duration of the travel until the train arrives its destination and the wagons are decoupled from the train. The virtual dedicated chain of relays is restricted only to the wagons composing the train.

Once this virtual dedicated chain of relays is established, the train can start its journey. The chain of relays allows different routing possible between UEs in case of failure of one of the relays.

The wagon UEs are using other UEs as relay in single hop or multi hops to communicate with the locomotive or with the WAN network in order to optimize their battery as illustrated by figure 6.

The wagons are sending their status message to 5G communication module of the locomotive acting as a gateway which is aggregating the messages to be sent to the wagon Tracking Management System. The UE very distant from the train engine are using other UEs as relay in multi hops to communicate with the 5G communication module of the train engine. To be able to use the locomotive gateway, 90% of the wagons composing the train need to be equipped with the 5G communication module to be able to relay all messages.

Figure 2: Communication between wagons and locomotive

All the communications need to be secured and only wagons that are part of the train composed are allowed to communicate between them and with the gateway of the locomotive.

### 5.2.4	Post-conditions

All wagons have provided their data, with minimal impacts on their battery and they are ready for their next travel(s). Once a travel is finished, the virtual dedicated chain of relays is disabled and the wagons are detached and decoupled from the train and can be tracked separately.

### 5.2.6	Potential New Requirements needed to support the use case

Note: 	All new requirements associated to communicating between wagons and usage of wagons as relay will be addressed in TR 22.866 (enhanced Relays for Energy Efficiency and Extensive Coverage).

| Reference number | Requirement text | Comment |
| --- | --- | --- |
| 5.2.6-1 | The 5G system shall be able to support a [20] years’ lifetime of the service without changing the UE or the battery of the UE. Note: This assumes a message of 200 bytes every hour (24 messages a day) with a battery of capacity of 21,6 Wh. |  |

------------------------- END OF SECOND CHANGES ----------------------------

------------------------- START OF THIRD CHANGES ----------------------------

### 5.2.5	Existing features partly or fully covering the use case functionality

| Reference number | Requirement text | Comment |
| --- | --- | --- |
| 5.2.5-1 | The 5G system shall be able to support an end-to end secured link between the UEs (wagons) and the wagon Tracking Management System. |  |
| 5.2.5-2 | The 5G System shall be able to support the request from the wagon Tracking Management System to the UE (wagon) to provide its status and tracking information periodically with an update rate ranging from one status every [15 minutes] to one status every [24 hours] (i.e. 96 messages to 1 message per day). |  |
| 5.2.5-3 | The 5G system shall be able to support terrestrial and non terrestrial (satellite) access networks. |  |
| 5.2.5-4 |  | The 5G system shall be able to support a continuity of the service in roaming case. |  |  |  |

------------------------- END OF THIRD CHANGES ----------------------------

------------------------- START OF FOURTH CHANGES ----------------------------

### 5.2.6	Potential New Requirements needed to support the use case

Note: All new requirements associated to communicating between wagons and usage of wagons as relay will be addressed in TR 22.866 (enhanced Relays for Energy Efficiency and Extensive Coverage).

| Reference number | Requirement text | Comment |
| --- | --- | --- |
| 5.2.6-1 | The 5G system shall be able to support a [20] years’ lifetime of the service without changing the UE or the battery of the UE. Note: This assumes a message of 200 bytes every hour (24 messages a day) with a battery of capacity of 21,6 Wh. |  |
| 5.2.6-2 | The UE shall be able to support a payload size up to 2500 bytes per message. | The maximum payload size for each message in NB-IoT is 1600 bytes. |

------------------------- END OF FOURTH CHANGES ----------------------------