3GPP TSG-SA WG1 Meeting #86 	S1-191107

Suzhou, P. R. China, 6-10 May 2019	(revision of S1-19xxxx)

Title:	Use Case on AR/VR with lightweight VR HMD

Agenda Item:	8.1

Source:	Huawei, China Mobile, Huawei Device, OPPO

Contact:	Yuan Wang

wangyuan5@huawei.com

Abstract: This document proposes to support high data rate for AR/VR with lightweight VR devices.

x.y	Use case for AR/VR with lightweight VR HMD

x.y.1	Description

Current VR device (e.g., VR HMD) is heavy and expensive, and will become over heated easily. The cost and also the bad user experience will restrain the VR device and service popular, which further may restrain the usage of 5G system for VR market.

Lightweight VR HMD will facilitate their introduction in homes. Decouple complexity functions from the HMD may be a way to achieve lightweight HMD, as shown in the figure below.

In this scenario, the video decoder functionalities and the GPU rendering functionalities can be decoupled from the HMD and will be implemented in another home device (e.g. smartphone or home gateway). In this case, avoiding decompression and rendering mechanism should facilitate the HMD to be light as this would reduce processing needs in the HMD.

When no compression is supported in the lightweight HMD, two scenarios are possible:

1) Either the network support transport of uncompressed data and can be sent over Uu to the HMD

2) Or there shall be a possibility to ensure another home device close to the HMD is able to receive compressed data from the Uu interface and then send uncompressed data to the HMD by using D2D communication.

x.y.2	Pre-conditions

Eric buys a new light HMD to play VR games. Eric registers this light HMD to the PLMN as a UE.

Eric also have a smartphone which is installed with video decoder and GPU rendering functionalities.

The PLMN provide the radio coverage which can support Ultra-high data rate and low latency for Eric’s Home.

x.y.3	Service Flows

1. Eric turns on the light HMD and start to play VR game in the home.

2. The VR application server sends VR video stream to the light HMD via the network. Since in the area, the PLMN can provide Ultra-high data rate and low latency, the VR application can sends uncompressed video stream.

3. After a few hours, Eric decides to visit his friends David. He takes both his smartphone and the light HMD together in order to share the game with David.

4. In David’s home, the Ultra-high data rate and low latency link is not available so that cannot send the uncompressed video stream directly to the light HMD. Then the network decides to use a combined way to serve the light HMD.

5. The network inform both Eric’s smartphone and light HMD to establish a D2D communication path between the smartphone and the light HMD. The establishment of the D2D communication path is fully controlled by the PLMN. Also, a data connection between the network and the smartphone is established by using Uu interface.

6. The network indicate the VR application to send compressed VR video stream.

7. The smartphone received the compressed VR video stream from the VR application via the Uu interface.

8. The smartphone uncompressed the VR video stream and then send the uncompressed video stream to the light HMD via the established D2D communication path.

x.y.4	Post-conditions

Eric and David can play VR game by using the light HMD.

x.y.5	Existing features partly or fully covering the use case functionality

TS 22.261 specified an Indoor hotspot scenario, which can support 1Gbps for DL.

|  | Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) | Area traffic capacity (UL) | Overall user density | Activity factor | UE speed | Coverage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | Indoor hotspot | 1 Gbps | 500 Mbps | 15 Tbps/km2 | 2 Tbps/km2 | 250 000/km2 | note 2 | Pedestrians | Office and residential (note 2) (note 3) |

x.1.6	Potential New Requirements needed to support the use case

The 5G system shall be able to exchange interactive service data via the ProSe Communication path (direct) or via the 5GC path in order to guarantee the user experience with impact minimized on receiving UE’s limited resources (e.g., battery consumption, processor capabilities). The KPIs for 5GC path and ProSe Communication path in the below two tables shall be met:

Table 1. KPIs for 5GC path

| Use Case | Packet type | Required Data Rate | link latency | Packet size | Reliability | active of UE | Service area | mobility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightweight HMD (Uu) | 2K resolution VR/AR | 5Gbit/s | 10ms | 5MB | 99.99% | <5 | 10-100m2 | Stationary or limited mobility (<1m/s) |
|  | Motion tracking | 192Kbit/s | 8ms | 24 Bytes | 99.999 |  |  |  |

Table 2. KPIs for ProSe Communication path

| Use Case | Packet type | Required Data Rate | link latency | Packet size | Reliability | Comm range | Service area | mobility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightweight HMD (Prose) | 2K resolution VR/AR | 5Gbit/s | 4ms | 5MB | 99.99% | > 10 meters | 10-100m2 | Stationary or limited mobility (<1m/s) |
|  | Motion tracking | 192Kbit/s | 2ms | 24 Bytes | 99.999 |  |  |  |