3GPP TSG-SA WG1 Meeting #86 	S1-191180

Suzhou, China, 6 - 10 May 2019	(revision of S1-19xxxx)

Title:	FS_REFEC – Metering use case

Agenda Item:	8.6

Source:	NOVAMINT

Contact:	Thierry Bérisot, tberisot@novamint.com

Abstract: This document contains the metering use case for FS_REFEC.

## 6.2 Traffic Scenario: Metering use case

### 6.3.1 Description

When deploying IoT metering solutions, utilities (gas, water, electricity…) are faced with the critical issue of Network ”black spots”. Some meters are very deep indoor and their coverage could not be assured by the network provider. These meters are likely not to be able to reach the eNB in a single hop. In some city, it could represent more than 5 % of the UEs deployed.

Furthermore, many meters that have coverage are at the edge of a cell and a subject to many repetitions. To be able to communicate, these meters need to increase drastically their battery consumption by emitting at full power. And therefore their battery life is strongly impacted and reduced – 2 or 3 years’ battery life instead of 15 years required for the service.

Enhanced relays will play a key role in achieving extensive coverage as well as better energy efficiency in smart metering scenarios.

### 6.3.2 Assumptions

It is assumed that up to 5 hops will be needed to provide indirect network connection to out of coverage meters and up to 3 hops for meters at the edge of the cell in EC level 2.

The figure 6.3.2-1 is illustrating the different

Figure 6.3.2-1: Smart Metering

In the figure, meters A & D are well positioned in EC level 0 while Meter B is very deep indoor and cannot communicate directly. Meter B could use Meter A as Relay. Meter C can communicate but battery is largely affected by more retransmission (EC Level 2). Meter C could use Meter A as well as Relay.

### 6.3.3 Potential New Functional Requirements

The 5G system shall support that a UE can establish an indirect network connection to the eNB even when a direct network connection is available (Note: this could be restricted to some UEs).

### 6.3.4 Potential Key Performance Requirements

The KPIs attached to this use case are listed in Annex C.