3GPP TSG-SA WG1 Meeting #91e	S1-203349

Electronic Meeting, 24 - 28 Aug 2020	(revision of S1-203121)

Title:	Smart Vehicle Key use case for FS_Ranging TR22.855

Agenda Item:	7.10.1

Source:	Spreadtrum, Xiaomi

Contact:	Chunhui Zhu (tom.zhu@unisoc.com), Xiaowei Jiang (jiangxiaowei@xiaomi.com)

Abstract: This contribution proposes a use case about Smart Vehicle Key for the FS_Ranging TR 22.855.

===================Proposed Changes==========================

X	Smart Vehicle Key

X.1	Description

The smart key allows the driver to keep the key pocketed when unlocking, locking and starting the vehicle. Vehicles with a smart-key system can distinguish whether the user carrying the key is approaching or not. When the key is approaching to the vehicle, it will be automatically unlocked; when the key is in the vehicle, the vehicle can be automatically started; when the key moving away from the vehicle, the vehicle will be automatically locked.

In this case it is assumed that one big parking place (100m*100m) can provide the place for hundreds of cars to park.  And it also assumes that there are max 40 users/drivers operating their cars at the same time, including locking, unlocking, starting engine. When there are many cars close to each other, it is not easy to identify one from the others if the distance accuracy is too relax, e.g., accuracy is around 2meters, and it results 4meters of error value, so there can be parked two cars when the width of car is less than 2meters. So it is reasonable to assume that accuracy is less than 1meter, then the total error value is 2meters.

X.2	Pre-conditions

Alice’s car, Bill’s car and Jack’s car are supporting for the smart key system, and are Ranging enabled.

Alice’ and Jack’s smart phone are set  as the smart key for her/his vehicle.

Bill’s smart watch is set as the smart key for his vehicle.

The smart key is paired to the specific car (for Alice, Bill and Jack)

There are hundreds of cars parked in the parking area in front of the shopping mall.

X.3	Service flow

A typical service flow is as following:

Figure x.1-1 use case for the smart vehicle key

- For unlock case.

Alice is walking out of the Shopping mall with her smart phone.

Alice’s car discovers the Alice’s smart phone

Alice’s car connect to Alice’s smart phone after successful authentication and authorization between them.

Alice is walking towards to her car based on the location information provided on her smart phone.

- For lock case.

Bill stops his car in the parking spot.

Bill is walking out of his car with his smart watch and moving away from his car.

- For engine start case.

(Assuming that Jack’s car is already connecting to Jack’s smart phone)

Jack is approaching to his car within certain distance (e.g., 2meter)

Jack’s car is unlocked

Jack is getting more close to his car within e.g., 0.5meter or Jack is getting into his car.

X.4	Post-conditions

Alice’s car is unlocked automatically when she is approaching to her car within certain distance, e.g., 2meters.

Bill’s car is locked automatically when he is moving away from his car more that certain distance, e.g., 2meters

The engine of Jack’s car is started automatically when he is getting into the car or he is 0.5meter away from the car.

X.5	potential new requirements

.

| Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Effective Ranging distance, Coverage, NLOS/LOS, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE , Number of concurrent ranging operation in an area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Distance Accuracy | Direction Accuracy |  |  | Effective Ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation in an area |
| Smart Vehicle Key | [1]m | - | [95 %] | [50ms] | [100m] | IC/PC/OOC | - | Static/ Moving (<1m/s) | - | - | [40]UEs/ (104m2) |