3GPP TSG-SA WG1 Meeting #93e	S1-210070

Online, Feb. 22  –  Mar. 4 2021

Title:	Clarify the KPI table in section 5.2

Agenda Item:	7.9.1

Source:	CEPRI, ZTE, China Telecom

Contact:	xu.ling@zte.com.cn

duanxiaomeng@epri.sgcc.com.cn

yangdelong@epri.sgcc.com.cn;

Abstract: This contribution proposes to clarify related values in KPI table and to remove editor note.

Proposed Changes:

5.2.6	Potential New Requirements needed to support the use case

[PR 5.2.6 - 1]The 5G system shall be able to support at least logical isolation communication service for advanced meter applications.

[PR 5.2.6 - 2]The 5G system shall be able to support the 5G UE working as a home data gateway to collect energy data from energy equipment and deliver them to back-end energy applications.

[PR 5.2.6 - 3]The 5G system shall be able to provide required communication service to advanced metering according to KPI given in table 5.2.6-1

Table 5.2.6-1 Communication KPI for advanced metering (note1)

| User experienced Data rate (bps) | Latency (ms) | Reliability % | Connection density | coverage |
| --- | --- | --- | --- | --- |
| UL:<2M DL:<1M | Accuracy fee control: < 100 (note2); General information data collection: <3000 | >99.99 | <10000/km2 (note3) |  |
| NOTE1: refer [3]. NOTE2: the accuracy fee control latency here is for communication one way latency from smart energy meter to backend system while the distance between them is no more than 40km i.e. city range. The command implementation need 100ms. NOTE3: it is the typical connection density in today city environment. With the evolution from meter centralization collection to sockets in home directly collection, the connection density is expected to increase 5-10 times. |  |  |  |  |