3GPP TSG-SA WG1 Meeting #93e	  S1-210177r5

Online, 22 February – 04 March 2021

Title:		Use Case of ensuring uninterrupted MTC service availability during emergencies

Agenda Item:	7.9.1

Source:	Ericsson, Orange

Contact:	Robert Farac (robert.farac@ericsson.com)

Abstract: This contribution proposes a new use case of power distribution grid power control service.

Acknowledgment

This contribution proposes a new use case of ensuring uninterrupted MTC service availability during emergencies using public mobile land networks. This contribution has been prepared in collaboration with RWTH University of Aachen Prof. Monti, Antonello. The use case has been implemented and tested in a lab trial in the eSafeNet project [X7] and Insulae project [X8].

Discussion

-

Proposal

It is proposed to agree to add the following change to TR 22.867.

Begin First Change

2		References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	                       3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	SunWeiqing etc.  Generalized Energy Storage Control Strategies on User Side in Power Ancillary Service Market.  Automation of Electric Power System Vol.44 No.2 Jan.25, 2020

[3]	China Southern Power Grid, China Mobile, HuaWei: "White Paper: 5G enables smart grid applications".http://www.sgcio.com/technology/anli/2018/0716/86656.html

[4]  	GB/T 1936-2018 Guidelines of cyber security protection for electric power system supervision and control http://www.howtoabc.com/book8/14078-0.html, https://www.secrss.com/articles/24761

[5]	IEEE C37.2-2008 IEEE Standard Electrical Power System Device Function Numbers, Acronyms, and Contact Designations.

[6]	IEC TR 61850-90-1:2010, Communication Networks and Systems for Power Utility automation – Part 90-1: Use of IEC61850 for the communication between substations.

[7]	IEEE 1588-2019 – IEEE Standard for a precision clock synchronization protocol for networked measurement and control systems.

[8]	IEEE Guide for Application of Digital Line Current Differential Relays Using Digital Communication.

[9]	61850-9-3-2016 – IEC/IEEE International Standard - Communication Networks and Systems for Power Utility automation – Part 90-3: Precision time protocol profile for power utility

[10]                     3GPP TR 22.804: "Study on Communication for Automation in Vertical domains (CAV)".

[11]	Sendin, A., et. al., “Telecommunication Networks for the Smart Grid,” Artech House, 2016.

[12]	Goel, S., S. F. Bush, and D. Bakken, (eds.), IEEE Vision for Smart Grid Communications: 2030 and Beyond, New York: IEEE, 2013.

[13]	US Department of Energy, “Communications Requirements of Smart Grid Technologies, 2010”, accessed 14.08.20, http://energy.gov/sites/prod/files/gcprod/documents/Smart_Grid_Communications_Requirements_Report_10-05-2010.pdf

[14]	IT Process Wiki – The ITIL Wiki:. Content is available according to Creative Commons Attribution-NonCommercial-ShareAlike 3.0 Germany License. Access 14.10.20.

[15]	Incident Management: https://wiki.en.it-processmaps.com/index.php/Incident_Management Content is available according to Creative Commons Attribution-NonCommercial-ShareAlike 3.0 Germany License. Access 14.10.20.

[16]	Change Management: https://wiki.en.it-processmaps.com/index.php/Change_Management Content is available according to Creative Commons Attribution-NonCommercial-ShareAlike 3.0 Germany License. Access 14.10.20.

[17]	Service Asset and Configuration Management: https://wiki.en.it-processmaps.com/index.php/Service_Asset_and_Configuration_Management Content is available according to Creative Commons Attribution-NonCommercial-ShareAlike 3.0 Germany License. Access 14.10.20.

[18]	China Innovation Alliance for Power Distribution Internet of Things, “Technical Specifications of Smart Distribution Transformer Terminal”

[19]	Open PLC European Research Alliance (OPERA). https://cordis.europa.eu/project/id/026920

[20]	“Applications of ITU-T G.9960, ITU-T G.9961 transceivers for Smart Grid applications: Advanced metering infrastructure, energy management in the home and electric vehicles”, 06/2020. https://www.itu.int/dms_pub/itu-t/opb/tut/T-TUT-HOME-2010-PDF-E.pdf

[21]	M. Amin and A. Giacomoni, "Toward more secure, stronger and smarter electric power grids," IEEE PES’11, 2011.

[22]	Eric D. Knapp, Raj Samani, "Smart Grid Network Architecture", "Applied Cyber Security and the Smart Grid", 2013

[23]	5G DNA White Paper: "5GDN@Smart Grid White Paper: Requirements, Technologies, and Practices" https://www.5gdna.org/

[24]	IEEE Std 1344™-1995 (R2001), IEEE Standard for Synchrophasors for Power Systems

[25]                  	IEEE Std C37.118™-2011, IEEE Standard for Synchrophasors for Power Systems

[26]	IEEE Std 1588-2008, Precision Clock Synchronization Protocol for Networked Measurement and Control Systems

[27]	IEEE Std C37.238-2011, IEEE Standard Profile for Use of IEEE Std 1588™ Precision Time Protocol in Power System Applications.

[28]	IEEE Std C37.224-2013

  - [29]	IEC 61850-90-5:2012, use of IEC 61850 to transmit Synchrophasors information according to IEEE C37.118

[30]	IEEE Std C37.118.1a-2014: amendment 1 with modification of selected performance requirements.

[31]	IEEE Std C37.118.2-2011

[32] 	61850-90-3-2016 – IEC/IEEE International Standard - Communication Networks and Systems for Power Utility automation – Part 90-3: Precision time protocol profile for power utility

[33]	Budka, K, et. al., “Communication Networks for Smart Grids”, Springer Verlag, 2014.

[X1]	McKeever, P.; De Din, E.; Sadu, A.; Monti, A: "MAS for automated black start of multi-microgrids". In Proceedings of the 2017 IEEE International Conference on Smart Grid Communications (SmartGridComm), Dresden, Germany, 23–27 October 2017; pp. 32–37.

[X2]	Gazis, V.ASurvey of Standards for Machine-to-Machine and the Internet of Things. IEEE Commun. Surv. Tutor. 2017, 19, 482–511. [CrossRef]

[X3]	Ghorbanian, M.; Dolatabadi, S.; Masjedi, M.; Siano, P. Communication in Smart Grids: A Comprehensive Review on the Existing and Future Communication and Information Infrastructures. IEEE Syst. J. 2019, 13, 4001–4014. [CrossRef]

[X4]	Oueis, J.; Conan, V.; Lavaux, D.; Stanica, R.; Valois, F. Overview of LTE Isolated E-UTRAN Operation for Public Safety. IEEE Commun. Stand. Mag. 2017, 1, 98–105. [CrossRef]

[X5]	Ali, S. Next Generation and Advanced Network Reliability Analysis Using Markov Models and Software Reliability Engineering; Springer Nature Switzerland AG: Basel, Switzerland, 2019.

[X6]	McKeever, Padraic & Sadu, Abhinav & Rohilla, Shubham & Mehdi, Zain & Monti, Antonello. (2020). Ensuring Uninterrupted MTC Service Availability during Emergencies Using LTE/5G Public Mobile Land Networks. Telecom. 1. 181-195. 10.3390/telecom1030013.

[X7]	eSafeNet Project, [Online], Available: https://e-safe-net.de/

[X8]	Insulae Project, [Online], Available: http://insulae-h2020.eu/

End First Change

Begin Second Change

### 5.x	Use Case of ensuring uninterrupted MTC service availability during emergencies

#### 5.x.1	Description

During emergencies, public mobile land networks (PLMNs) may restrict network access, which may lead to a lack of service reliability for machine-type communication (MTC) in critical applications, such as power systems and in particular in microgrids. Microgrids are separate parts of a power grid, which can be controlled and operated individually in a so-called island mode or together as a whole. Existing features of a mobile network can be used to differentiate MTC of devices in a microgrid from other MTC or human-to-human (H2H) communication and ensure that these microgrid devices have service during emergencies, which enables use of mobile communication to co-ordinate the use of Distributed Energy Resources (DER) in microgrids, so that they can autarkically perform blackout recovery of an islanded microgrid. The idea is to ensure reliable communications for selected MTC devices during emergency conditions, without giving these devices additional priority over other network users during normal conditions or adversely affecting the service to prioritised users during emergencies. It was shown that this method allows the blackout recovery 100 times faster than with a conventional black start [X6].

5.x.1.1		Introduction

Figure X1 shows an example of power grid restoration after a large-scale blackout using conventional and autarkic microgrid blackout recovery methods.

Figure X1: (a) Conventional grid restoration (hierarchical top-down blackout recovery) and (b) autarkic microgrid restoration (each islanded microgrid autonomously recovers from blackout). Figure taken from [X6].

Grid restoration after a large-scale blackout is conventionally done by first starting power stations with black start capability, then expanding the re-energised grid to include more large synchronous power stations and expanding the re-energised grid. In this way, medium voltage and low voltage levels are re-energised hierarchically and top-down, i.e., from the transmission level.

The autarkic microgrid blackout recovery method presented in [X1] assumes that the MV/LV grid is divided into a set of microgrids (Figure X1 (b)), which can be operated in island mode and are assumed to be autarkic, meaning that they have sufficient local energy generation and storage to be self-sufficient, and autonomous, meaning that they can manage themselves, although in normal operation they may be managed by distribution grid (DG) management systems external to the microgrid.

In the context of a microgrid, having reliable communications during a blackout means that the microgrid can continue to manage itself during blackout, i.e., even when the distributed energy resources (DER) in the microgrid are connected to a de-energised grid, the distributed controllers at the DER sites can continue to communicate with each other.

If communications from the microgrid to external management systems are working, the microgrid can perform autarkic blackout recovery under their direction. If, however, there is a loss of communication from the microgrid to external management systems, the microgrid can autonomously perform the autarkic blackout recovery. In any case, the autarkic blackout recovery relies on having communications which continue to function locally in the microgrid during blackout to allow the agents to act together to perform a black start using local DERs.

In the autarkic microgrid blackout recovery method, the medium/low voltage grid is assumed to be divided into a set of autarkic microgrids, as shown in Figure X1(b), which are islanded in case of blackout. The DERs remain connected to the grid and continue to use their Multi-Agent System (MAS) MTC devices to communicate with each other and with the distribution grid control centre during the blackout [X1]. In this scenario, each microgrid has its own gNB equipped with an emergency power supply. In case the emergency power supply for an gNB is not available, only this specific microgrid covered by the affected gNB cannot perform autarkic blackout recovery. However, other microgrids which are not affected by the misfunctioning gNB can still perform blackout recovery.

#### 5.x.2	Pre-conditions

The MAS’s within one microgrid must be covered by a large-scale, wide-area communications network, and the communications network must continue to operate during blackouts.

The network must support MTC and the MAS’s can communicate within the microgrid using MTC.

#### 5.x.3	Service Flows

Blackout recovery using autarkic microgrids:

- Identify which nodes in the MAS are working

- Identify the current microgrid topology and DER capacity

- Re-energise the microgrid and the loads.

#### 5.x.4	Post-conditions

The microgrids have recovered from the blackout and operate normally again.

#### 5.x.5	Existing features partly or fully covering the use case functionality

-

#### 5.x.6	Potential New Requirements needed to support the use case

| Characteristic parameter (KPI) |  |  |  | Influence quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability | Communication service reliability: mean time between failures | Max Allowed End-to-end latency (note 1) (note 2) | Service bit rate: user-experienced data rate (note 2) | Message size [byte] | Survival time | UE speed | # of UEs | Service Area |
| 99.999 9 % |  | 100 ms | < 1 Kbit/s per DER |  |  | Stationary |  |  |
| NOTE 1: Unless otherwise specified, all communication includes 1 wireless link (UE to network node or network node to UE) rather than two wireless links (UE to UE). NOTE 2: It applies to both UL and DL unless stated otherwise. |  |  |  |  |  |  |  |  |

Editor note: this table should be revisited whether they are existing requirements

End Second Change