3GPP TSG-SA WG1#93 e-Meeting	S1-210194r1

22 February – March 4 2021

Title:	Update to FS_PALS use case: Automatic discovery and selection of 3rd party provider services over Hosting network access.

Agenda Item:	7.13.1

Source:	Qualcomm

Contact:	Jack Nasielski (jackn at qti.qualcomm.com)

Abstract: This document describes a use case and (possible) potential requirements, as part of the FS_PALS study.

## Discussion

Use Case 5.6, (E-)Sports events at a venue: Automatic discovery and selection of 3rd party provider services over Hosting network access, provides potential requirements for configuration of the network access for a 3rd party service both on the hosting network and the UE. However, the potential requirements lack an action on the UE on how to use the configuration.

This document proposes to add a potential requirement on the UE to use the configuration for PALS access.

Also, it is important to clarify in the requirements that the UE shall abide by HPLMN network selection rules when accessing a PALS network with HPLMN credentials.

## Use Case 1

## 5.6	(E-)Sports events at a venue: Automatic discovery and selection of 3rd party provider services over Hosting network access.

Network A infrastructure is deployed in multiple sport venues. Network A may be a PLMN or an NPN. A 3rd Party content provider negotiates with Network A to set-up access to the 3rd party provider’s services during the event and at the venue location, and for Network A to manage resources for that service.

The 3rd party provider services are offered to people attending and/or participating in the event, without any previous subscription to Network A.  The users have a previous relationship with the 3rd party provider.

During the event service negotiation between 3rd party provider and Network A, the Network A provides discovery information to 3rd party provider. The 3rd party provider configures the UE with event service discovery, which may contain Network A’s information and the event service information, and time validity.

During the time validity the UE searches for Network A and event service based on configuration from the 3rd party provider. If the UE finds a suitable cell from Network A offering the event service, the UE notifies the user who can manually initiate access to Network A. The UE requests to connect to 3rd party event service(s) access, Network A validates with 3rd party provider that the UE is authorized to access content for event.

If necessary, the UE is provisioned temporary access credentials for Network A.  The UE then accesses Network A for 3rd party event service(s) with the temporary credentials.

During registration Network A may provide UE with configuration based on negotiation with 3rd party, e.g. slicing configuration.

Automatic selection of Network A is possible

if the UE is capable to maintain two separate registrations, one with configured credentials for PALS access and an independent registration using a USIM. In that case, if the UE was already registered to a PLMN when identifying the PALS network, the UE may initiate access and registration to the PALS network with the configured credentials while maintaining the access to the registered PLMN independently.

### 5.6.2	Pre-conditions

Network A and 3rd Party Content Provider B have a service pre-agreement for 3rd Party Content Provider B to offer exclusive service via Network A’s access during the game event at the event location.

The 3rd party service is meant to be offered to event participants that may not have any subscription to Network A and have prior service agreement with 3rd party provider for the event service.

### 5.6.3	Service Flows

- A user is made aware of the 3rd party service being offered via Network A’s access. This may be done either offline (e.g., in ticket, flyer, posters) or online (e.g. Internet advertisement.)

- During the event service negotiation between 3rd party provider and Network A, the Network A provides configuration information to 3rd party provider.

- The 3rd party provider provides the UE with Network A’s information, configuration (e.g. slicing configuration) for the event service and the event service information, and time validity.  This is sent over any access.

- During the time validity the UE searches for Network A.

- If the UE finds a suitable cell from Network A, the UE notifies the user who can manually initiate access to Network A. The UE indicates request to access to 3rd party event service(s) access.

- Network A validates with 3rd party provider that the UE is authorized to access content for event.

- If necessary, the UE is provisioned temporary access credentials for Network A.  The UE then accesses Network A for 3rd party event service(s) with the temporary credentials.

- The UE may if capable maintain two separate registrations, one with configured credentials for PALS access and an independent registration using a USIM. In that case, if the UE was already registered to a PLMN when identifying the PALS network, the UE may initiate access and registration to the PALS network with the configured credentials while maintaining the access to the registered PLMN independently.

### 5.6.4	Post-conditions

The user is able to enjoy the e-game event services provided by 3rd party provider via Network A’s access.

### 5.6.5	Existing features partly or fully covering the use case functionality

### 5.6.6	Potential New Requirements needed to support the use case

[PR.5.6.6-1] The 5G system shall support means for a PALS Network to provide a 3rd party with configuration information for automatic discovery of the Network and available access to specific 3rd party services.

[PR.5.6.6-2] The 5G system shall support means for the 3rd party to provide the UE with information for discovery and access to a PALS Network, and configuration to access specific services with a certain time and location validity. The configuration may include credentials necessary for access to the PALS network.

[PR.5.6.6-3] The 5G system shall enable a UE supporting PALS to be able to use the 3rd party provided configuration for identification and access to a PALS Network, and configuration to access for specific services with a certain time and location validity.

[PR.5.6.6-4] The 5G system shall allow a UE with configured credentials for PALS network access and a USIM to be registered at the same time to the PALS network and the PLMN. In that case, the 3GPP system shall treat each registration independently. If the configured credentials for PALS network access are from a USIM, Multiple USIM requirements (e.g., as specified in TS 22.101[x]) apply.)

[PR.5.6.6-5] The 5G system shall be able to inform UEs of 3rd party services the UE can use at a given point in time and location.