3GPP TSG-SA WG1 Meeting #94e 	S1-211455

Electronic Meeting, 10 – 20 May 2021	(revision of S1-211225r2)

Title:	RESIDENT – Consolidated requirements

Agenda Item:	7.11.1

Source:	KPN, Vivo Mobile Communications Ltd

Contact:	Toon(dot)Norp(at)tno(dot)nl

Abstract: This document proposes a list of consolidated requirements.

Discussion

Document contains consolidated requirements from the usecases.  For ease of referencing a table has been created, the table labels each consolidated requirement and provides a reference to with PR it was.  There is also a column that identifies is an equivalent CPR exists in TR 22.859.

****Changes****

# 7	Consolidated requirements

## 7.1	Introduction

This section provides Consolidated Potential Requirements for consideration to include in the normative specifications.  The CPR’s have been grouped into different functional categories, each category contains a table that lists the original PR.

## 7.2	General

Table 7.2-1 – RESIDENT General Consolidated Requirements

| Potential Requirement |  |  |
| --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text |
| [CPR.7.2-1] |  | Subject to local regulations, the 5G system shall support regulatory requirements for emergency calls, PWS and eCall for UEs connected to a CPN. |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.3	Gateway

Table 7.3-1 – RESIDENT Gateway Consolidated Requirements

| Potential Requirement |  |  |
| --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text |
|  | PR 5.1.6-002 | The 5G system shall enable the evolved Residential Gateway (eRG) to provide backhaul with the required QoS for the services provided via a Premises Radio Access Station (PRAS)connected via the eRG. NOTE3: Backhaul for the PRAS includes the residential network between PRAS and eRG and the (fixed) access network between the eRG and the 5G core network. |
|  | PR 5.3.6-002 | The 5G system shall be able to indicate to the evolved residential gateway that a specific UE connected to it either via 3GPP RAT or non-3GPP (R)AT needs specific QoS and what the required QoS characteristics are. |
|  | PR 5.5.6-003 | The 5G system shall support a mechanism to minimize the security impact on any PLMN or broadband access network when using an Evolved Residential Gateway. |
|  | PR 5.5.6-004 | The 5G system shall support a mechanism to minimize the security impact on the UE when using an Evolved Residential Gateway. |
|  | PR 5.5.6-005 | The 5G system shall enable the network operator associated with an Evolved Residential Gateway to control the security policy of an Evolved Residential Gateway. |
|  | PR 5.5.6-006 | The 5G system shall ensure an Evolved Residential Gateway does not compromise user privacy for UEs that are using the Evolved Residential Gateway, including communication confidentiality, location privacy and identity protection. |
|  | PR 5.13.6-001 | The 5G system shall be able to support IP traffic offload in CPN. |
|  | PR 5.19.6-001 | The 5G system shall support use of an eRG that is connected to the 5G Core Network over wireless access, fixed broadband access or hybrid access. |
|  | PR 5.19.6-002 | The 5G system shall support a mechanism for the network operator to provide policies to the eRG on which transport (e.g. wireless, cable, etc.) is best suited for different services. |
|  | PR 5.17.6-001 | The PRAS and eRG shall be able to detect a loss of connection with the 5GC. |
|  | PR. 5.17.6-002 | When the eRG detects a loss of connection with the 5GC, the eRG shall allow communication using non-3GPP access within the CPN to continue. |
|  | PR. 5.17.6-003 | When the CPN detects a loss of connection with the 5GC and the PRAS is serving UEs under its coverage using: - licensed spectrum, the CPN may allow the PRAS to continue transmission using the licensed spectrum for an operator-defined time period. - During this time period, if the PRAS continues transmission using licensed spectrum, the CPN shall allow communication using licensed spectrum within the CPN to continue. NOTE: The time period could be set to "indefinite". - At the end of this time period, the CPN shall deactivate the PRAS air interface. - If the time period is undefined, the CPN shall deactivate the PRAS air interface. - unlicensed spectrum, the CPN shall allow communication using unlicensed spectrum within the CPN to continue. NOTE: In the requirement above, only data traffic exchange within the CPN is allowed to continue. |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.4	Service Discovery

Table 7.4-1– RESIDENT Service Discovery Consolidated Requirements

| Potential Requirement |  |  |
| --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text |
|  | PR 5.2.6-008 | The 5G system shall provide a mechanism to prevent or allow a (guest) UE to discover and/or use the services provided by the devices on the CPN. |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.5	Service Hosting

Table 7.5-1 – RESIDENT Service Hosting Consolidated Requirements

| Potential Requirement |  |  |
| --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text |
|  | PR 5.11.6-001 | The 5G system shall be able to provide support for an eRG to interconnect with an Application Server. |
|  | PR 5.11.6-002 | The 5G system shall be able to maintain QoS for a UE when it is moving between a Service Hosting Environment and an Application Server in the CPN that is connected to an eRG. |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.6	Identification and Privacy

Table 7.6-1 – RESIDENT Privacy Consolidated Requirements

| Potential Requirement |  |  |
| --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text |
|  | PR 5.2.6-009 | The 5G system shall ensure that communications associated with individual UEs in a CPN be identifiable (e.g., subscriber identifier) in the 5G network. |
|  | PR 5.22.6-001 | The 5G system should support “User Identity” requirements (as defined in 22.101 clause 26.a [22.101]) for a user (human) using a CPN authorized UE to access external non-3GPP applications/services hosted in a CPN (behind a eRG). |
|  | PR 5.22.6-002 | The 5G system shall support to allow a CPN authorized UE or non-3GPP device accessing to a CPN based on successful User Identity authentication. |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.7	Direct Communications

Table 7.7-1 – RESIDENT Direct Communications Consolidated Requirements

| Potential Requirement |  |  |
| --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text |
|  | PR 5.10.6-001 | The 5G system shall be able to minimize disruption to the user when a UE switches from a direct communication path between the UEs to an indirect communication path going through the eRG and there is connectivity to the 5G system. |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.8	Connectivity - QoS - charging

Table 7.8-1 – RESIDENT Connectivity, QoS and Charging Consolidated Requirements

| Potential Requirement |  |  |
| --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text |
|  | PR 5.5.6-002 | The 5G system shall support routing efficiency for data traffic between two UEs through an Evolved Residential Gateway. |
|  | PR 5.4.6-001 | The 5G system shall support routing efficiency for data traffic between a UE and a non-3GPP device through an Evolved Residential Gateway. |
|  | PR 5.6.6-001 | The 5G system shall provide a mechanism for supporting real time E2E QoS monitoring for the data traffic path (i.e., from/to a UE to/from the 5GC via a Premises Radio Access Station and a Evolved Residential Gateway). |
|  | PR 5.10.6-002 | The 5G system shall support real time E2E QoS monitoring and control for any data traffic path (i.e. from/to a UE to/from the 5GC and to/from another UE) via a PRAS and an eRG when there is connectivity to the 5G system. NOTE 1: Related requirements on QoS are also outlined in clause 5.1 and clause 5.7. NOTE 2: The above requirements may result in no new stage 2/3 work unless it is shown there is impact from introducing the PRAS/eRG in the path. |
|  | PR 5.13.6-001 | The 5G system shall be able to support IP traffic offload in eRG. |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.9	Provisioning

Table 7.9-1 – RESIDENT Provisioning Consolidated Requirements

| Potential Requirement |  |  |
| --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text |
|  | PR 5.18.6-001 | The 5G system shall enable configuration and management of an eRG by both the operator of the public (mobile) network the eRG is connected to and, within the boundaries defined by the operator, by an Authorised Administrator. |
|  | PR 5.18.6-003 | The 5G system shall enable configuration and management of a PRAS by a PLMN and, within the boundaries defined by the operator, by an Authorised Administrator. Specifically the PLMN shall be able to configure: radio settings pertaining to licensed spectrum shall be configured by the PLMN that owns the spectrum. Specifically the Authorised Administrator shall be able to configure:Whether visitor access network via the PRAS is allowed (allowing all or no visitors, or allowing specific visitors only) |
|  | PR 5.7.6-001 | The 5G system shall provide mechanisms for the network operator or an Authorised Administrator (e.g. a homeowner) to trigger remote provisioning of evolved residential gateways and for the network operator to perform remote provisioning of eRGs, which includes verification and configuration of evolved residential gateway identity and initial OA&M provisioning. |
|  | PR 5.7.6-002 | The 5G system shall provide mechanisms for the network operator or an Authorised Administrator (e.g. a homeowner) to trigger remote provisioning of Premises Radio Access Stations and for the network operator to perform remote provisioning of PRASs, which includes verification and configuration of Premises Radio Access Station identity and initial OA&M provisioning. |
|  | PR 5.12.6-002 | The 5G system shall provide means for an authorized user to prioritize access for a certain UE in a PRAS and CPN, within the limits given by the operator policy. |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.10	Premises Radio Access Station

Table 7.10-1 – RESIDENT Premises Radio Access Station Consolidated Requirements

| Potential Requirement |  |  |
| --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text |
|  | PR 5.1.6-001 | The 5G system shall enable the network operator to provide any 5G services to any 5G UE via a Premises Radio Access Station (PRAS) connected via an evolved Residential Gateway (eRG). NOTE1: The eRG may be connected via fixed access, via 5G Fixed Wireless Access, or hybrid access. NOTE2: The PRAS may also be co-located with the eRG |
|  | PR 5.2.6-001 | The 5G system shall enable the network operator to provide any 5G services via a Premises Radio Access Station (PRAS) to any 5G UE with a valid subscription to the HPLMN associated with the PRAS. |
|  | PR 5.2.6-002 | The 5G system shall enable the network operator to provide any 5G services via a Premises Radio Access Station (PRAS) to any 5G UE with a valid subscription to any VPLMN that has a roaming agreement with the HPLMN. NOTE: Whether 5G UEs from VPLMNs in the same country as the HPLMN can use the PRAS is subject to regulatory policy on national roaming. |
|  | PR 5.2.6-003 | The 5G system shall ensure the use of a Premises Radio Access Station (PRAS) does not compromise the security of any PLMN or broadband access network. |
|  | PR 5.2.6-004 | The 5G system shall ensure the use of a Premises Radio Access Station (PRAS) does not compromise the security of the UE. The PRAS (and its associated backhaul connectivity) shall provide a level of security equivalent to regular 5G base stations. |
|  | PR 5.2.6-005 | The 5G system shall enable the network operator associated with the Premises Radio Access Station (PRAS) to control the security policy of the PRAS. |
|  | PR 5.2.6-006 | The 5G system shall ensure the Premises Radio Access Station (PRAS) does not compromise user privacy for UEs that are using the PRAS, including communication confidentiality, location privacy and identity protection. |
|  | PR 5.2.6-007 | The 5G system shall be able to generate charging information that can differentiate between UEs connected to the Premises Radio Access Station (PRAS) and between backhaul for the PRAS and other data traffic over the same access. |
|  | PR 5.5.6-001 | The 5G system shall be able to provide QoS control for the communication path between a UE and an Evolved Residential Gateway via a Premises Radio Access Station. |
|  | PR 5.12.6-001 | The 5G system shall provide means to enable/disable a UE to connect to a Customer Premises Network device via a particular Premises Radio Access Station. NOTE: Enabling/disabling a UE to a particular PRAS in the CPN does not prevent the UE to connect to other PRASs in the CPN. |
|  | PR 5.12.6-003 | The 5G system shall minimize service disruption when a CPN communication path changes between two PRASes. |
|  | PR 5.14.6-001 | The 5G system shall be able to support PRAS sharing between multiple PLMNs. |
|  | PR 5.18.6-002 | The 5G System shall support PRAS that use licensed spectrum, use unlicensed spectrum, or can use both unlicensed and licensed spectrum. |
|  | PR 5.21.6-001 | The 5G system shall provide support for a network operator to authenticate a PRAS. |
|  | PR 5.21.6-002 | The 5G system shall provide support for a network operator to authorize a PRAS for its use in a CPN. |
|  | PR 5.21.6-003 | The 5G system shall support a secure mechanism to provide credentials to a PRAS. |
|  | PR 5.21.6-004 | The 5G system shall provide mechanisms for the network operator to provision an authenticated PRAS with operation settings, e.g. carrier frequencies. |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.11	5G-LAN

Table 7.11-1 – RESIDENT 5G-LAN Consolidated Requirements

| Potential Requirement |  |  |
| --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text |
|  | PR 5.8.6-001 | The 5G system shall be able to support large amounts of small 5G LAN-VNs targeting residential deployments. NOTE: Targeting residential requirements translate into millions of 5GLAN-VN per operator per country. These 5G LAN-VNs may contain between 10-50 devices per LAN |
|  | PR 5.8.6-002 | The 5G system shall support authorized 3rd parties to authorize/deauthorize UEs to be able to access a 5G LAN-VN. |
|  | PR 5.9.6-001 | The 5G system shall support the use of an evolved Residential Gateway to connect 5G devices from the 5G LAN VN it belongs to with non-3GPP devices on an in-home LAN. |
|  | PR 5.16.6-001 | The 5G system shall support interconnection of a 5G LAN-Virtual Network (5G LAN-VN) with a fixed IP VPN. |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.

## 7.12	Broadcast Multicast

Table 7.12-1 – RESIDENT Broadcast Multicast Consolidated Requirements

| Potential Requirement |  |  |
| --- | --- | --- |
| CPR No. | Original Potential requirement No. | Potential requirement text |
|  | PR.5.20.6-001 | The 5G system shall provide means to deliver 5G multicast/broadcast services to an eRG. |
|  | PR.5.20.6-002 | Under operator control, an eRG shall be able to receive multicast/broadcast services from its access network. NOTE: The access network can be wireless or wireline. |
|  | PR.5.20.6-003 | Under operator control, an eRG, shall be able to efficiently deliver 5G multicast/broadcast services to authorized UEs and non-3GPP devices in the CPN. |
|  | PR. 5.15.6-001 | The 5G system shall support multicast service access control based on eRG subscription that enables eRG to forward authorized multicast services to multiple non-3GPP devices behind the eRG. NOTE: The multicast services that each of multiple non-3GPP devices is allowed to access may be different. |

Editor’s Note: the consolidated requirements above are only an indication of grouping of potential requirements in the different sections and as a basis for further work on consolidation. Further consolidation of these requirements is needed. This can imply that requirements are changed, merged or not accepted as consolidated requirement.