3GPP TSG-SA WG1 Meeting #94e	S1-211045

Electronic Meeting, 10 May – 20 May 2021	(revision of S1-21xxxx)

Title:	FS_AMMT: Additional requirements for use case 5.5

Agenda Item:	7.4.1

Source:	InterDigital, OPPO

Contact:	Atle Monrad <Atle.Monrad@InterDigital.com>

Discussion

This PCR proposes updates to the requirement for the use case in clause 5.5 (Session-specific model transfer split computation operations).

Proposal

It is proposed to add new requirements in 3GPP TR 22.874 version 1.0.0.

* * * *   First Change   * * * *

### 5.5.6	Potential New Requirements needed to support the use case

[P.R.5.5-001] Based on operator policy, the 5G network shall provide the means to allow an authorized third-party to monitor the resource utilisation of the network service that are associated with the third-party.

[P.R.5.5-002]	Based on operator policy, the 5G system shall be able to expose QoS information and QoS update conditions to an authorized 3rd party. The QoS update condition can be one or more of the bitrate, latency, reliability per location and/or time.

[P.R.5.5-003] The 5GS shall provide predictions of network condition changes (i.e. bitrate, latency, reliability) to the devices operating split AI completed by their confidence level and prediction validity duration.

[P.R.5.5-004] The UE shall be able to access predictions of network condition changes from the 5GS.

* * * *   End of Changes   * * * *