3GPP TSG-SA WG1 Meeting #95e 	S1-213292

Electronic Meeting, 23 August – 2 September 2021	(revision of S1-213025)

Title:	Updates to “Virtual coupling” use case

Agenda Item:	7.6.1

Source:	Korea Railroad Research Institute (KRRI)

Contact:	Woojin Ahn <woojin.ahn@krri.re.kr>, Ilmu Byun <ilmubyun@krri.re.kr>

Abstract: This contribution proposes virtual coupling requirements to enhance the virtual coupling use case in chapter 5 of TR22.990. We propose the requirements for urban railways.

--------------------------------------- Start of Change ----------------------------------------

## 5.8	Virtual Coupling

### 5.8.1	Description

One of the important missions that the future railway service should achieve is to increase its transport capacity. A straight-forward solution is to minimize the distance between successive trains so that train interval is reduced. It is difficult to do so in a legacy train control system, because the successive trains need to have distance as much as a safety margin, which should be larger than the full braking distance.

Figure 5.8.1-1 Sharing acceleration and braking control information to shorten safety margin

This safety margin can be further shortened if the successive trains share control information (acceleration and braking) and apply it to its own train control. As shown in Figure 5.8.1-1, the safety margin can be shortened if the following train (T1) immediately knows that the leading train (T2) starts braking and also triggers braking. This is the fundamental principle and the main purpose of virtual coupling. Figure 5.8.2 shows the basic concept of the virtual coupling scenario. Multiple trains which are in close distance move together as they are physically coupled. As the distance between two trains gets smaller, the control information of a train should be delivered to the other in shorter time.

Figure 5.8.1-2 The concept of virtual coupling scenario

### 5.8.2	Pre-conditions

1. The leading and following trains in operation recognize each other and have just got configured to be coupled virtually. Here, the recognition is achieved by on-network or off-network.

2. The user equipments for train control support device to device communications in the perspective of transport layer.

Figure 5.8.1-3 Overall procedure of virtual coupling

### 5.8.3	Service Flows

1. The following train begins to approach to the leading train by exchanging the information about their position. The two trains, which are far enough to allow a certain amount of end-to-end latency, are initially connected through the on-network.

2. As the following train approaches to the leading train, they start exchanging the information about movement control and each train then controls itself while considering the control of the other train. The two trains, which are still far enough to allow a certain amount of end-to-end latency, keep the connection through the on-network.

3. As the following train further approaches to the leading train, the safe braking distance gets shorter and they are required to have a connection of which end-to-end latency is very short for delicate train control. At this moment, they seamlessly switch the connection from the on-network to off-network. Each train can maintain the on-network connection, as an auxiliary connection, as long as it is within the coverage.

4. As the following train even further approaches the leading train so that the distance between the tail-end of the leading train and the front-end of the following train becomes shorter than the safe braking distance, the off-network connection requires extremely high reliability as well as short end-to-end latency to prevent potential accidents. In this case, two trains can be regarded as if they are physically coupled, communicating via wireless train backbone (WLTB).

### 5.8.4	Post-conditions

For safety train operation, integrity needs to be checked for train control information delivery in the perspective of application or transport layer. Any error on integrity check shall be immediately reported to the train control application.

### 5.8.5	Existing features partly or fully covering the use case functionality

The potential new requirement in the followings have been considered only in TR 22.989 V18.0.0.

### 5.8.6	Potential New Requirements needed to support the use case

### 5.8.6.1	Requirements related to the Service layer

NOTE: Off-network is used even when network coverage is available.

[PR 5.8.6.1-1] The FRMCS System shall be able to provide integrity protection for off-network train control communication

[PR 5.8.6.1-2] The FRMCS System shall support seamless service continuity between on-network based connection and off-network based connection

[PR 5.8.6.1-3] The FRMCS System shall support UEs capable of utilizing off-network and on-network communications at the same time.

[PR 5.8.6.1-4] The FRMCS System shall provide a mechanism to change the QoS requirements of the off-network communication adaptively.

### 5.8.6.2	Requirements related to the Transport layer

[PR 5.8.6.2-1] The FRMCS System shall be able to provide integrity protection for off-network train control communication

[PR 5.8.6.2-2] The FRMCS Service in Off-Network mode shall support the following traffic characteristics of data transfer for direct communication for virtual coupling:

Note: this table is intended to be included in Section 6.2.2.2 of TS 22.289 [2]

| Scenario | End-to-end latency | Reliability (Note 1) | UE speed | UE Relative Speed | User experienced data rate (UL and DL and SL) | Payload size (Note 2) | Area traffic density (UL and DL and SL) | Overall UE density | Communication range | Service area dimension (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Virtual coupling, critical data communication (Korea, urban railway) | ≤100 ms | 99.99% | ≤100 km/h | ≤50km/h | ≤1Mbps | Small to large | ≤5 Mbps/km | ≤15 (3000 m) | ≤3000 m (Note 6) | ≤3000 m along rail tracks including bad weather conditions |
| Virtual coupling, Very critical data communication (Korea, urban railway) (Note 5) | ≤10 ms | 99.9999% | ≤100 km/h | ≤50km/h | ≤1Mbps | Small to large | ≤20 Mbps/km | ≤6 (300 m) | ≤300 m (Note 6) | ≤300 m along rail tracks including bad weather conditions (Note 4) |
|  | NOTE 1: Reliability as defined in TS 22.289 sub-clause 3.1. NOTE 2: Small: payload ≤ 256 octets, Medium: payload ≤512 octets; Large: payload 513 -1500 octets. NOTE 3: Estimates of maximum dimensions. NOTE 4: Non-Line-of-Sight (NLOS) between UEs shall be supported. NOTE 5: Data link between trains can work as like wireless train backbone NOTE 6: UEs are assumed to be located at the tail of the leading train and the front of the following train |  |  |  |  |  |  |  |  |  |  |

Table 5.8.6.2-1: Traffic characteristics for Virtual coupling communication

[PR 5.8.6.2-3] The FRMCS System shall establish an immediate communication session between entities for virtual coupling, as defined in TS 22.289 sub-clause 5.2.3.

[PR 5.8.6.2-4] The FRMCS System shall support seamless service continuity between on-network based connection and off-network based connection  [Editor’s note: This requirement is FFS]

[PR 5.8.6.2-5] The FRMCS System shall support UEs capable of utilizing off-network and on-network communications at the same time. [Editor’s note: This requirement is FFS]

--------------------------------------- End of Change ----------------------------------------