3GPP TSG-SA1 Meeting #95	S1-213304

Electronic Meeting, 23 August – 2 September 2021	(revision of S1-213151, 212129, 212020)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.859 | CR | 0001 | rev | 3 | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements on PIN element discovery restriction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_PIN |  |  |  |  |  | Date: |  |  | 2021-09-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In use case#13, when Hummel’s mobile phone gets added to the hospital’s bed PIN, Hummel may decide to only add personal health devices (not all his PIN elements) to the hospital bed’s PIN due to privacy reason, e.g. location tracker which stored history data will not be added to hospital’s bed PIN. On the other hand, the hospital bed’s PIN may only allow Hummel’s personal health devices can discover/communicate specific hospital bed’s PIN elements due to security reason. For example, the hospital bed’s PIN restricts Hummel’s personal health devices to discover/communicate some controllers in the bed PIN, e.g. Oxygen valve controller, which may cause serious result if it’s manipulated incorrectly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add requirement on PIN element discovery restriction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | All PIN elements will be exposed to each other/to guest PIN element, which may cause privacy/security problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.13.3, 5.13.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****FIRST CHANGE****

### 5.13.3	Service Flows

-	Upon entering the cardiology department inpatient room, Hummel’s mobile phone and the smart hospital bed can discover each other. Also the 5G PIN enabled wearable heart monitor and the smart hospital bed can discover each other.

-	When Hummel looks at his mobile phone, he notices that it has discovered the smart hospital bed. It also shows the list of connected PIN elements connected to the hospital bed.

-	Hummel can connect his mobile phone to the smart hospital bed and upon connecting, Hummel’s mobile phone gets added to the hospital’s bed PIN. Also the personal devices that are connected to Hummel’s mobile phone can be discovered by the hospital bed and added to the hospital bed’s PIN. Hummel can decide which personal devices can be discovered and added to the hospital bed’s PIN.

-	On the screen of the mobile phone (or the screen attached to the hospital bed), it also shows that the 5G PIN enabled wearable heart monitor in the list of discovered PIN elements. Hummel presses a button to also add that device to the hospital bed’s PIN.

-	After exchange of credentials, the PIN elements can now securely communicate between each other. This allows Hummel to use the screen attached to the bed for streaming some videos from his mobile phone, allows the data from his heart monitor to be received by the high-end patient monitor, and show the patient monitor’s data on his mobile phone.

-	As part of the PIN configuration, the PIN elements are also allowed to gain patient access to the hospital network (via the hospital bed’s PIN gateway capability). On the screen of his mobile phone (or the screen attached to the hospital bed), Hummel allows his wearable heart monitor and his other connected health devices to send their data to the hospital network.

-	The data from the connected devices are useful in improving the pre-surgical preparation of Hummel and saves operational overhead for the hospital staffs.

-	Thanks to the PIN feature, Hummel had an easy transfer to the hospital and had a fairly comfortable experience (despite his condition) in his smart hospital bed.

****Next Change****

### 5.13.6	Potential New Requirements needed to support the use case

[PR 5.13.6-1] The 5G system shall enable PIN elements to discover and identify each other.

NOTE 1: This may be done directly (depending on the discovery capabilities of the PIN elements) or via a PIN element with gateway capability or via a PIN with management capability.

[PR 5.13.6-2] The 5G system shall enable PIN elements with management capability to add PIN elements to the PIN.

[PR 5.13.6-3] The 5G system shall allow only authorized PIN elements to connect to the 5G network via a PIN element with gateway capability.

NOTE 2: Next to authorization of the 5G network operator, additional authorization/consent from the PIN user may be needed depending on whether the user wants to allow PIN elements to connect to the 5G network or not.

[PR 5.13.6-4] The 5G system shall support a mechanism for the PIN user to indicate whether a PIN element is discoverable by other PIN elements of the same PIN.

[PR 5.13.6-5] The 5G system shall support a mechanism for the PIN user to indicate whether a PIN element is discoverable by UEs that are not members of the PIN.

****END Change****