3GPP TSG SA WG 1 Meeting 95-e		 S1-213207r1

Electronic Meeting, 23 August –2 September 2021

Title:	Reply LS on Indication of country of UE location and its use in PLMN selection

Response to:	LS C1-214778 on Indication of country of UE location and its use in PLMN selection from CT1

Release:	Release 17

Source:	SA1

To:	CT1

Cc:	SA2, SA3-LI

Contact person:	Toon Norp

Toon .Norp @ tno. nl

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA1 thanks CT1 for their LS on Indication of country of UE location.

On the CT1 questions, SA1 provides the following answers:

1)	Should the indication of UE location represent a "country" in the manner a "country" is defined in TS 23.122, or should the "country" be the TR 22.926 definition of "a country is defined as the area embedded within a set of borders and for which a unique set of regulations applies for the provision of communication services through mobile networks"?

Answer SA1: The indication of UE location should represent a country in the manner a country is defined in TS 23.122.

Further CT1 would like to ask SA1's guidance on the usage/applicability, if at all, of the indicated country of UE location, in the subsequent PLMN selection, i.e. shall it be considered by the UE:

2)	that the UE is not allowed in the current geographical position to select any PLMN which does not correspond to the indicated country of UE location; or

Answer SA1: Yes if there is no available and allowable PLMN with shared MCC. If there is an available and allowable PLMN with a shared MCC, the UE may select the PLMN with a shared MCC.

3)	the UE might select available and allowable PLMNs of that indicated country and other countries.

Answer SA1: No.

# 2	Actions

To CT1

ACTION: 	SA1 invites CT1 take the above information into account

# 3	Dates of next TSG SA WG 1 meetings

SA1#96e	8-18 Nov 2021			Electronic Meeting

SA1#97e	21-25 Feb 2021			Poland