3GPP TSG-SA1 Meeting #96e	S1-214092

# Electronic Meeting, 08 November – 18 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0606 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the 5G LAN requirements for PIRates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, KPN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIRates |  |  |  |  |  | Date: |  |  | 2021-10-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In 22.859, there is requirement “[PR 5.8.6-1a] The 5G system shall enable an authorized PIN user to configure which UEs connected to the public network can perform service discovery of PIN Elements in a PIN. The 5G system shall support configuration per 5GLAN VN, per group of UEs, or per individual UE.” In clause 6.26.2.2, it has defined “The non-3GPP device in a CPN can communicate with a UE in 5G-VN via an eRG of the CPN, which is part of 5G- LAN VN. ”. This requirement can also be applied to the non-3GPP device in a PIN, which can communicate with a UE in 5G- LAN VN via a PIN Element with the Gateway Capability of the PIN, as the PIN Element with the Gateway Capability is part of the 5G LAN-VN. Furthermore, in order to support the communication between non-3GPP device and UE in one 5G LAN-VN, the 5G network shall provide suitable APIs to allow a trusted third-party to add/remove an eRG or a PIN Element with the Gateway Capability to/from a specific 5G LAN-VN managed by the trusted third-party. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the 5G LAN requirements related to PIRates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The existing 5G LAN requirements related to PIRates is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.26.2.2, 6.26.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***START OF CHANGES****

6.26	5G LAN-type service

6.26.2.2	5G LAN-virtual network (5G LAN-VN)

A UE shall be able to select a 5G LAN-VN, that the UE is a member of, for private communication.

A 5G system shall support 5G LAN-VNs with member UEs numbering between a few to tens of thousands.

The 5G system shall be able to support large numbers of small 5G LAN-VNs.

NOTE:	Targeting residential deployments translate into millions of 5GLAN-VN per operator per country. These residential 5G LAN-VNs typically contain between 10-50 devices.

The 5G LAN-VN shall support member UEs that are subscribed to different PLMNs, e.g. a 5G LAN-VN may span multiple countries and have member UEs that have a subscription to a PLMN in their home country.

The 5G system shall support on-demand establishment of UE to UE, multicast, and broadcast private communication between members UEs of the same 5G LAN-VN. Multiple types of data communication shall be supported, at least IP and Ethernet.

The 5G network shall ensure that only member UEs of the same 5G LAN-VN are able to establish or maintain private communications among each other using 5G LAN-type service.

The 5G system shall allow member UEs of a 5G LAN-VN to join an authorized multicast session over that 5G LAN-VN.

The 5G system shall be able to restrict private communications within a 5G LAN-VN based on UE’s location (i.e. when the UE moves out of the area it can no longer communicate on the 5G LAN-VN).

The 5G network shall enable member UEs of a 5G LAN-VN to use multicast/broadcast over a 5G LAN-type service to communicate with required latency (e.g. 180 ms).

The 5G system shall support a mechanism to provide consistent QoE to all the member UEs of the same 5G LAN-VN.

The 5G system shall support routing based on a private addressing scheme within the 5G LAN-VN.

The 5G system shall support a communication path between a non-3GPP device in the CPN and a UE in the 5G-LAN VN via an eRG of the CPN, for the eRG that is part of the 5G LAN-VN.

The 5G system shall support communications between two PIN Elements that are part of the same PIN that use different PIN Elements with Gateway capabilities to communicate with each other via the 5G system, as the PIN Elements with the Gateway Capability are part of the same 5G LAN-VN.

***NEXT CHANGES****

6.26.2.9	Service exposure

Based on MNO policy, the 5G network shall provide suitable APIs to allow a trusted third-party to create/remove a 5G LAN-VN.

Based on MNO policy, the 5G network shall provide suitable APIs to allow a trusted third-party to manage a 5G LAN-VN dedicated for the usage by the trusted third-party, including the address allocation.

Based on MNO policy, the 5G network shall provide suitable APIs to allow a trusted third-party to authorize/deauthorize UEs to access a specific 5G LAN-VN managed by the trusted third-party.

Based on MNO policy, the 5G network shall provide suitable APIs to allow a trusted third-party to add/remove an authorized UE to/from a specific 5G LAN-VN managed by the trusted third-party.

Based on MNO policy, the 5G network shall provide suitable APIs to allow a trusted third-party to add/remove an eRG or a PIN Element with the Gateway Capability to/from a specific 5G LAN-VN managed by the trusted third-party, in order to support the communication between non-3GPP device and UE in one 5G LAN-VN.

****END OF CHANGES****