3GPP TSG-SA1 Meeting #96e	S1-214147

Electronic Meeting, 8 November –18 November 2021	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0617 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Performance requirements for Passive IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone, BMW Brilliance Automotive, CAICT, CATT, China Mobile, China Telecom, China Unicom, EDF, Futurewei, HiSilicon, Huawei, KPN, Novamint, NTT DOCOMO, Quanray, Spreadtrum |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | xxx |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The basic KPIs are captured for Passive IoT. These requirements should be added to Performance Requirements section. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Documenting initial Performance Requirements for Passive IoT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The basic Performance KPIs for Passive IoT would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------- start of the first change -----------------------------

## 7.x	Passive IoT

### 7.x.1	Description

The 5G support for Passive IoT allows the operator to maximise the use of their networks by providing IoT services to market segments that rely on extremely small, thin, lightweight, low complexity IoT devices that do not have a traditional power source. These Passive IoT devices use energy harvested from their environment.

### 7.x.2 		KPIs for Passive IoT

The 5G system shall support the performance requirements listed below with an energy harvesting system that produces [250 micro Watts] of power.

Table 7.x.2-1 Performance requirements for Passive IoT scenario

| Scenario | Max device power consumption | Service bit rate: user-experienced data rate | Message Size | Device density | Communication range | Service Area | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Asset and material identification and tracking | 100 uW | 10 kbit/s uplink (note 1) | 96 bits (note 2) | < 1,5 Million/ km2 (note 3) | 25 m Indoors | 600 000 m2 (note 4) | automobile manufacturing |
| Sensor operation indoors | 100 uW | 10 kbit/s uplink | < 1000 bytes (note 5) | 850 000 / km2 | 25 m indoors | 1200 m2 (note 6) | livestock temporarily indoors |
| Sensor operation outdoors | 500 uW | 320 bit/s uplink | < 40 bytes (note 7) |  | 100 m outdoors |  | livestock outdoor |
| NOTE 1: This value is estimated from the need for the system to support some other services as well as the infrequent transmission of the 100 bit payload from a very large number of units of material during a 24 hour period. (e.g. 5 reports per day, each of 100 bits, for 1 million units, spread over 600 000 m2, served by 20 cells of 100m radius implies an average of 300 bit/s per cell). NOTE 2: Electronic Product Code standard. NOTE 3: Daily around 1 million units of materials are used in the manufacturing area, but they are not used at the same time. NOTE 4: A typical car manufacturing plant takes upto 600 000 m2 in surface. NOTE 5: The type of sensors supported by Passive IoT are very simple. Hence their information packets should be significantly smaller than NB-IoT’s max data packet size (PDCP PDU) of 1600 bytes. E.g. the daily report (during milking) from a cow’s sensor that stores heart rate and temperature measurements that were made every 5 minutes can be less than 600 bytes. NOTE 6: Temporary activity with 1000 animals in surface area of around 1200 m2. NOTE 7: For example, hourly reporting from a sensor on a cow in a field that measures heart rate and temperature every 5 minutes needs less than 30 bytes. |  |  |  |  |  |  |  |

----------------------------- end of changes -----------------------------