3GPP TSG-SA WG1  Meeting #96e	S1-214325

Electronic Meeting, 8 – 18 November 2021	(revision of S1-214302)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.867 | CR | 0020 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Filling all CPRs into a normalized table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GSEI |  |  |  |  |  | Date: |  |  | 2021-11-18 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As chair suggested, all CPRs in Rel-18 SIDs should be filled in a normalized table which is convenient for reading and finding the related PRs and use cases as well as introducing some comments. Also, some bugs in the same section need to be fixed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR does changes as listed below: - Filling all CPRs into a normalized table with original PRs and comments. - Removing the second CPR-007 whose original PR was removed before. - Removing the CPR-008 which has no content. - Renumbering the CPRs after CPR-007. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CPRs are still displayed in the former way which is difficult for readers to find out the related PRs and use cases. Also, there are two CPR-007 statements and the CPR-008 shows void. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start Change

7.1	Consolidated potential requirements

Table 7.1-1 - Consolidated Potential Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR-7.1-1 | The 5G system shall support an end-to-end latency of less than 5ms or 10ms, as requested by the UE initiating the communication. | PR[5.4]-001 |  |
| CPR-7.1-2 | The 5G system shall support communication channel symmetry in terms of latency (latency from UE1 to UE2, and latency from UE2 to UE1) between the two UEs, with the max asymmetry < 2ms. | PR[5.4]-002.option1 |  |
| CPR-7.1-3 | Based on MNO policy, the 5G network shall provide suitable means to allow a trusted third party to monitor LAN-VN performance parameters, to configure and receive information for conditions relevant to a specific UE, network and specific configuration aspects of the UE in the VN. | PR[5.7]-001 |  |
| CPR-7.1-4 | The 5G system shall provide a means by which an MNO informs 3rd parties of network events (failure of network infrastructure affecting UEs in a particular area, etc.). | PR[5.7]-002 |  |
| CPR-7.1-5 | Based on operator policy, the 5G system shall provide means by which an MNO informs 3rd parties of changes in UE subscription information. The 5G system shall also provide a means for 3rd parties to request this information at any time from the MNO. (note 1) | PR[5.7]-005 |  |
| CPR-7.1-6 | Based on operator policy, the 5G system shall provide means for the 3rd parties to request changes to UE subscription parameters for access to data networks, e.g. static IP address and APN. | PR[5.7]-005a |  |
| CPR-7.1-7 | The 5G system shall provide a means by which an MNO can inform 3rd parties of changes in the RAT type serving UE, cell ID, quality of signal information, change in frequency band assigned with a suitable frequency via OAM and/or 5G core network to aid the 3rd party user in taking proactive actions to achieve their own service availability. | PR[5.7]-006 |  |
| CPR-7.1-8 | The 5G system shall enable support of a mechanism to support authentication and secured communication between the 5G system Core Network and a 3rd party's application function, in order to provide secure end to end communication service. | PR[5.10]-001 |  |
| CPR-7.1-9 | The 5G system shall provide a mechanism for a 3rd party to report to an MNO service degradations, communications loss and sustained connection loss. These reports use a standard form. The specific values, thresholds and conditions upon which alarms occur could include e.g. the measured values for latency, data rate, availability, jitter, etc. for a UE, its location, and the time(s) in which the degradation occurred. (notes 2, 3) | PR[5.11]-001 |  |
| CPR-7.1-10 | The 5G System should support the IEC 61850-9-3 [29] profile and IEEE Std C37.238-2017 [24]. | PR[5.13]-001 |  |
| CPR-7.1-11 | 5G system should support at least one of the two profiles for synchrophasor communications: IEC 61850-90-5:2012 [26], or IEEE Std C37.118.2-2011 [28]. | PR[5.13]-002 |  |
| CPR-7.1-12 | The 5G system should support the IEEE 802.1Q QoS profile as defined IEC 61850-90-5 [26]. | PR[5.13]-003 |  |
| CPR-7.1-13 | The 5G system shall support delivery of the same UE originated data to a group of recipient UEs distributed over a large geographical area in a resource-efficient manner in terms of bandwidth. | PR[5.13]-004 |  |
| CPR-7.1-14 | The 5G system shall allow an originating UE to request a communication service to send data to different groups of UEs at the same time. | PR[5.13]-005 |  |
| CPR-7.1-15 | The 5G system shall allow a UE to request different QoS for the communication in each of those groups. | PR[5.13]-006 |  |
| CPR-7.1-16 | The 5G system shall provide a mechanism for an MNO, based on MNO policy, to automatically report to 3rd party' service degradations, communications loss and sustained connection loss in a specific geographic area (e.g. a cell sector, a cell or a group of cells.) These reports use a standard form. The specific values, thresholds and conditions upon which alarms occur could include e.g. the measured values for latency, data rate, availability, jitter, etc. for a UE, its location, and the time(s) in which the degradation occurred. | PR[5.17]-002 |  |
| CPR-7.1-17 | Subject to regulatory requirements and operator policy, the 5G system shall support a mechanism by which an MNO can identify the uninterruptable power supply status of the MNO's infrastructure, specifying which physical regions would be affected in terms of physical topology, as this information will facilitate energy system recovery operations | PR[5.18]-001 |  |
| CPR-7.1-18 | Subject to regulatory requirements, the 5G system shall support a mechanism by which a third party can communicate the energy system recovery status in terms of location and time table to the MNO, as this information will facilitate MNO operations to facilitate energy system recovery. (note 4) | PR[5.18]-002 |  |
| CPR-7.1-19 | The 5G system shall enable recipient UEs to indicate their interest in receiving data from a specific originating UE. | PR[5.16]-003 |  |
| NOTE 1: Examples of UE subscription information include IP, 5G LAN-VN membership address and APN/DNN. These changes can have strong impacts in the stability of the 3rd party service. NOTE 2: What the MNO does with such reports is out of scope of 3GPP specifications. NOTE 3: The above potential requirement expresses the need for reporting by a third party to the 5GS and leaves it to downstream groups (in this case SA5) to work out the implications. NOTE 4: It is assumed that once aware of the proximity and duration of the energy outage, the MNO may manage the 5G nodes affected by the outage to make use of the power autonomy remaining in the 5G nodes, e.g. prioritizing the delivery of resources to support the energy system operations communications. Power consumption for energy system operations must be optimized so that the service recovery can be remotely orchestrated by the energy utility. |  |  |  |

End Change