3GPP TSG-SA WG1 Meeting #98e 	S1-221053r01

Electronic Meeting, 9 – 19 May 2022	(revision of S1-22xxxx)

Title:	New use case: Ambient_IoT in personal belongings finding

Agenda Item:	7.3

Source:	vivo

Contact:	        Huan Li <huan.li@vivo.com>

Abstract: This document provides a Text Proposal for the use case about using Ambient_IoT services for personal belongings finding.

# Proposal

It is proposed to add the following reference and use case description in TR 22.840.

************* Start of Changes ***************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1]	3GPP TS 22.261: "Service requirements for the 5G system".

************* Next Change ***************

## 5.x	Use Ambient_IoT service for personal belongings finding

### 5.x.1	Description

Sometimes in daily life, Mickey cannot remember where he put his wallet, or his favourite pair of shoes. He may become crazy if he is in a hurry to go out.

With Ambient power-enabled IoT (Ambient_IoT) service provided by 5GS. Mickey can attach Ambient_IoT Devices to his wallet and shoes. Then he can easily find them using his mobile phone which supports Ambient_IoT service. The Ambient_IoT Device is solely dependent on harvested ambient energy, being maintenance-free, of extremely-low complexity, weight, and size.

### 5.x.2	Pre-conditions

Mickey bought a mobile phone supporting Ambient_IoT service. He also obtains multiple Ambient_IoT Devices for personal belongings positioning.

### 5.x.3	Service Flows

- Mickey’s and his roommate Minnie’s mobile phones may be authorized by their mobile operators to perform the Ambient_IoT service.

- Mickey attaches one AmbinetIoT Device to his wallet, registers “Mickey’s wallet” to the application server.

- When Mickey wants to find his wallet, he opens the application in his mobile phone to search his wallet.

- Mickey’s mobile phone performs the inventory operation (e.g. read out unique IDs of the Ambient_IoT Device attached to his wallet) and identify that the wallet is nearby. The Ambient_IoT Device attached to his wallet may play a sound for Mickey to easily find it. Furthermore, Mickey’s mobile phone may display the position of the wallet (e.g. relative position of the mobile phone).

- Mickey authorizes Minnie’s mobile phone to perform the inventory operation to his wallet Device.

NOTE:	Mickey’s mobile phone, Minnie’s mobile phone and the AmbinetIoT Device to Mickey’s wallet may belong to one Personal IoT Networks as described in TS 22.261 [x1] clause 6.38.

- The other day, Mickey goes out to the bank. When he arrives at the bank, he cannot find his wallet. He would like to check whether his wallet is left at home. Mickey opens the application in his mobile phone and see the position of his wallet is in his home with the help of Minnie’s mobile phone.

- Mickey may also see the position of his wallet is at a coffee shop with the help of the pico cell near the coffee shop.

### 5.x.4	Post-conditions

Thanks to the Ambient_IoT service provided by the 5G system, Mickey can find his wallet as soon as possible.

### 5.x.5	Existing features partly or fully covering the use case functionality

SA1 has performed various studies on IoT in previous releases. To enable extremely small, light-weight, low complexity, battery-less or low power consumption Ambient_IoT Devices that engage in basic IoT data transaction, new challenges to the 5G system are foreseen and need to be addressed.

### 5.x.6	Potential New Requirements needed to support the use case

### 5.x.6.1 Service requirements for use of Ambient_IoT services for personal belongings positioning

[PR.5.x.6.1-001] The 5G system shall support communication for an Ambient_IoT device.

[PR.5.x.6.1-002] The 5G system shall authorize a UE or a RAN to communicate with Ambient_IoT devices (e.g. performing inventory operations or transfer data for Ambient_IoT devices).

[PR.5.x.6.1-003] The 5G system shall support communication for Ambient_IoT Devices that are limited in their operation by the amount of power available.

[PR.5.x.6.1-004] The 5G system shall be able to provide indoor communication range for Ambient_IoT communication of at least 10 meters.

[PR.5.x.6.1-005] The 5G system shall be able to provide outdoor communication range for Ambient_IoT communication of at least 80 meters.

[PR 5.x.6.1-006] The 5G system shall be able to provide the 5G indoor positioning services to Ambient_IoT device with the horizontal positioning accuracy of e.g. 1-3 metres.

Editor's note:	It is FFS whether AmbientIoT Devices need to be identified in 5GC.

### 5.x.6.2 KPIs for the use of Ambient_IoT services for personal belongings positioning

[PR.5.x.6.2-001] The 5G system shall be able to provide AmbientIoT service with following KPIs:

Table 5.x.6.2-1 AmbientIoT service KPI for personal belongings finding

| Scenario | Max. allowed end-to-end latency | Max. instantaneous device power consumption | Service bit rate: user-experienced data rate | communication range | positioning accuracy |
| --- | --- | --- | --- | --- | --- |
| Personal belongings finding (indoor) | [1] s | [20 – 50] uW | [< 1 kbit/s] | [10] m | [1-3] m for 90% of AmbientIoT Devices |
| Personal belongings finding (ourdoor) | [1] s | [20 – 100] uW | [< 1 kbit/s] | [100] m | [5-10] m for 90% of AmbientIoT Devices |

************* End of Changes ***************