3GPP TSG-SA1 Meeting #98e 	S1-221061

Electronic Meeting, 09 - 19 May 2022	(revision of xx-yyxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.183 | CR | 0004 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CRS interaction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAT&CRS |  |  |  |  |  | Date: |  |  | 2022-04-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The interaction capability of CRS service can divert consumers to other service platform(e.g. restaurant, movie platform), through called party’s interaction during playing CRS(e.g. get coupons, reserve a movie, vote for stars). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Requirements are introduced in clause 4.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CRS service will not have the capabilities to support CRS interaction and lose the business chance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*********************************First change********************************************

4	CRS Service Requirements

4.1		Basic functionality of CRS service

The Customized Ringing Signal service (CRS service) is an operator specific service by which an operator enables the subscriber to customize the ringing signal which is played to the called party. The CRS content will be provided to the called party only it’s allowed by the called party. The called party does not need to be a CRS subscriber to experience the CRS set by calling subscriber.

-	The service shall support the CRS content types of audio, video, image, graphic, text, location, and electronic business card.

-	The service shall support interaction with the called party when playing CRS content.

-	CRS can contain one or several content types, and at most one element of each type.

-	The called party should be able to simultaneously experience multiple CRS contents, e.g. audio and video.

-	The service user shall be able to subscribe to the services, activate the services, update the settings, e.g. to change his active CRS.

-	The CRS service shall override the default ringing signal towards the called party provided that media components are compatible.

-	The called party shall be able to experience the CRS set by the called service subscriber.

-	The called party shall be able to experience the CRS set by the calling service subscriber.

-	The operator should have the capability to store multiple CRSs per service subscriber.

-	It should be possible to inform the user about status and changes in his service, e.g. service expiry date or a particular CRS content.

Note: In the CS domain, this information may be provided by existing means such as SMS and web access.

-	The operator shall be able to charge for the CRS service and the related contents on event, volume or media type basis.

-	It shall be possible for a subscriber to subscribe to CRS that is provided by a third party content provider.

-	CRS service should not negatively affect the conversation between calling and called parties, e.g. no voice /video clipping.

-	If the CRS can’t be played for some reason (for example, the CRS system fails, or the CRS content expiry is up, or the called user roams from 3G network to 2G network, or other reasons), the called party shall experience the default ringing signal instead of the CRS.

-	When the multi-media CRS fails to be played in some condition (such as the caller roams from 3G network to 2G network or network congestion occurs), the voice CRS may be played instead if possible.

-	The operator shall be able to verify that the party is authorised to store and update CRS content.

-	The operator shall allow the calling party to select from the pre-stored CRS content according to the called party on a per call basis.

-	As service option, if a specific text (object of the communication) is provided by the calling party, this information should replace any potential text information pre-stored in the network.