3GPP TSG-SA WG1 Meeting #98e 	S1-221090r8

Electronic Meeting, 9 – 19 May 2022	(revision of S1-22xxxx)

Source:	Qualcomm

pCR Title:	Ambient IoT devices for Smart Cities

Draft Spec:	3GPP TR 22.840

Agenda item:	7.3

Document for:	Approval

Contact:	Lola Awoniyi-Oteri <oawoniyi@qti.qualcomm.com>

Abstract: This document proposes a Smart City use case for Ambient IoT Communication

1. Introduction

Introducing Smart City use case for Ambient IoT devices

2. Reason for Change

Adding Smart City use case with service requirements for Ambient IoT devices

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840, V0.0.0

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1]	Rasara Hewa Lunuwilage, “Study of Pole-top Fire Development in Power Distribution Networks”, PhD thesis, RMIT University, 2013 (Study of pole-top fire development in power distribution networks - CORE Reader)

[x2]	https://cbsaustin.com/news/local/pollen-dirt-and-drought-causing-power-poles-to-catch-on-fire

[x3]                     Pole Monitoring by NimbleLink (Asset Tracking System | NimbeLink)

* * * End of First Change * * * *

* * * Second Change (All new Text) * * * *

## 5.y	Communication with Ambient IoT Devices for Smart Cities

### 5.y.1	Description

### Some cities experience power outages and lose millions of dollars annually to pole, wild or brush fires. Most of these fires are caused by accumulated debris mixed with moisture resulting in high leakage current, tilting of poles resulting in the arching of these power lines or falling poles which ignite brushes and dry vegetation [x1, x2, x3].

### To address this issue, the use of temperature sensors, as well as accelerometers for continuous monitoring of the electricity pole tilt and detection of fallen poles would allow quick detection of fires or even predict the potential of impinging pole, wild or brush fires before occurrence. The communication unit (e.g., IoT device) enables transmission of the sensor readings periodically or on-demand (in case of emergency) to the network or via the network to a server.

### Powering the sensors and the IoT device with commonly used energy sources (e.g., generators running on diesel) also require significant human intervention and are not eco-friendly. Therefore, the use of ambient energy sources (such as solar energy) is attractive because little to no human contact is not required, and they tend to cheaper than the non-ambient energy power sources.

### 5.y.2	Pre-conditions

The city of  Salmego which is a tropical city which has been experiencing many pole, wild and brush fires, therefore, they decided to invest in sensor devices which are attached electricity poles to monitor temperatures, titling of the poles and detect fallen poles. These Ambient IoT devices are battery-less or with limited energy storage capability using harvested solar energy. This is depicted in Figure 5.y.2-1.

Additionally, the Ambient IoT devices are capable of 5G communications, including direct communication with other devices, communication with 5G system via a base station, or communication with server via the 5G system.

Figure 5.y.2-1: 5G System Assisted Communication with Ambient IoT Devices

### 5.y.3	Service Flows

1) 5G system assisted Intermittent Communication with Ambient IoT Devices

Even though Salmengo is a tropical city, during the raining season, it experiences week-long gloomy weather with little to no sunshine. During these periods, the batteries on the electricity poles are not charged fully for days. During one such periods, the power level went below a pre-defined threshold configured by 5G system and based on the configuration and policies from the 5G system,  the Ambient IoT device on Pole A sends a message to the network notifying the network that it is going out of coverage for a certain duration (or until it is fully charged). This message was sent along with the latest sensor readings.

In a few hours, there was enough sunlight to charge the limited energy storage to a level where the Ambient IoT device could communicate with the network briefly. The Ambient IoT device sends a communication message with the latest readings from the temperature sensor and accelerometer. Also, based on the history of the power levels in the limited energy storage, the Ambient IoT device on Pole A also informs the network it would like to enter the intermittent communication mode.  In this mode, the network supports intermittent connection with the Ambient IoT device (which implies connection for short duration of time and no connection for longer duration).

The network provides proper configurations and/or enable mechanisms for the Ambient IoT device to support reliable communication during the intermittent communication mode. Network also facilitates low power reliable communication between ambient IoT devices and network. The communication between the network and Ambient IoT device is shown in Figure 5.y.3-1.

In the case that the application is supported outside of the network, the network would also notify the application about the intermittent connection so that the application may adjust appropriately. In the intermittent mode, the Ambient IoT device  on Pole A would be able to communicate with the network with low power level using the configurations provided by the network.

Once the sunny days return and the limited energy storage is fully charged, the Ambient IoT device in Pole A would then notify the network about exiting the intermittent communication mode since it has more steady power. The network would then revert to typical communication with this Ambient IoT device.

In case a pole failing (fall) is detected by sensors, the Ambient IoT device immediately wakes up and notifies the network of the fall incident with its current location.

Figure 5.y.3-1: 5G Assisted Ambient IoT Device Intermittent Communication

2)  5G System Assisted Communication with Ambient IoT devices

Again, there was intermittent sunlight and the power level of limited energy storage on Ambient IoT device on Pole A  partially charged and could support sensing operations and minimal communication with Ambient IoT devices on surrounding electric poles (e.g., on Pole B) which are closer distance to Pole A than the base station. There were also light showers of rain so the mixture of debris and moisture on the pole led to high leakage current, consequently resulting in a pole fire, the temperature sensor which was still operational obtained a reading that detected extreme temperatures on pole A. Also, the fire burnt the wooden pole and the accelerometer captured a reading indicating significant increase in the pole’s tilt angle.

Editor’s note: Feasibility of this use case is FFS.

Figure 5.y.3-2: 5GS Assisted Communication with Ambient IoT Devices using indirect Communication

### 5.y.4	Post-conditions

Whether in or out coverage of the base station, the network is able to forward sensor readings and location information for an Ambient IoT device on an electricity pole to the application before or during emergency to ensure that the appropriate action is taken to advert outages, fires or help resolve the issues quickly.

### 5.y.5	Existing features partly or fully covering the use case functionality

The existing features in the following specifications could be leveraged for this use case:

- TS 22.368: clause 7.2.7 (infrequent mobile terminated)

- TS 23.682: clause 4.5.4 (power saving mode)

Editor’s Notes: Detailed Gap analysis is FFS

### 5.y.6	Potential New Requirements needed to support the use case

[PR 5.y.6-1] The 5G system shall be able to support Ambient IoT devices operating battery-less or on limited energy storage capability and intermittently harvested energy (e.g., solar power).

[PR 5.y.6-2] The 5G system shall be able to support direct network connectivity for Ambient IoT devices operating battery-less or on limited energy storage capability and intermittent harvested energy (e.g., solar power).

Editor’s Notes: Requirement to support indirect device connection for Ambient IoT devices is FFS.

[PR 5.y.6-3] The 5G system shall be able to support mechanisms to monitor the status, configure and control operation of Ambient IoT devices operating battery-less or on limited energy storage capability and intermittent harvested energy (e.g., solar power) e.g., policies and configurations for connectivity, QoS, and reporting (based on certain power conditions or events).

[PR 5.y.6-4] The 5G system shall be able to provide the status, control and configuration of Ambient IoT devices operating battery-less or on limited energy storage capability and intermittent harvested energy (e.g., solar power) to a trusted 3rd-party application.

Editor’s Notes: The KPIs for this use case are FFS.

* * * End of Second Change * * * *