3GPP TSG-SA WG1 Meeting #98e 	S1-221165r02

Electronic Meeting, 9 – 19 May 2022	(revision of S1-22xxxx)

Source:	Intel

pCR Title:	FS_Sensing Use Case: Autonomous/Assisted Driving

Draft Spec:	3GPP TR 22.837

Agenda item:	7.2

Document for:	Approval

Contact:	Suresh Srinivasan	suresh.srinivasan@intel.com

Abstract: Abstract: This pseudo-CR provides an overview of Autonomous/Assisted Driving use case, service flows and potential new requirements.

1. Introduction

2. Reason for Change

Proposing new “Autonomous/Assisted Driving” use case for FS_Sensing proposal.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 version 0.0.0.

BEGINNING OF CHANGE

# 5	Use cases

## 5.A	Use case of Autonomous/Assisted Driving

### 5.A.1	Description

Autonomous/Assisted Driving. Assisted driving is enabled either by vehicle-to-vehicle (V2V) or/and vehicle-to-infrastructure communication (V2I) coordination. Advanced assisted driving such as V2V and V2I can reduce road accidents and traffic jams. High resolution and accuracy in object detection and estimation of range/angle/velocity is achieved using Imaging, mapping and localization and multiple sensors. Autonomous driving largely depends on sensing, driving and mapping technology. Sensing in AV are used to build an accurate 360deg environment model. Cameras, lidar, ultrasound, short-range and long-range radars are some of the sensors adopted by autonomous vehicles (AV). Cameras gives high resolution, long range and capability to recognize traffic signals and road signs but underperforms under bad weather and poor lighting. Whereas Radars are resilient to weather and lighting and also detects distant and speedy objects but lacks clarity. Lidars have the benefits of wide field of vision and accurate distance estimation but has poor resolution and underperforms due to bad weather. AV use different strategies with the data from a combination of these sensors to ensure safe and reliable driving. Network assisted driving applications such as Edge driving, remote driving, Extended driving, Advance driving, Platooning, co-operative awareness etc. are the typical end consumers of AV sensor measurement data. These applications use computing intense algorithms for autonomous driving, and the results of these applications are communicated back to the AV using ubiquitous 5G network.

The network-based driving assistance can also combine data from other AV to make driving decisions. For a safe and reliable network assisted autonomous driving, the 5G Network should provide the required data rate, latency and reliability.

5G Network Data rate. Offloading heavy computing to the network-based assistance requires significant uplink/downlink data rate. The aggregated data rate of up to 1Gps will be required for V2V and V2X communication.

5G Network Latency and Reliable communication. 5G Network Reliability is a vital part of network-based assisted driving for safe and reliable driving. The reliability should be from 90% for co-operative awareness to 99.99% in the network-based assisted driving. Along with reliability, low latency is another key factor for safe and reliable autonomous driving. The acceptable latency can vary between 3msec to 100msec depending on the mode of autonomous driving.

Mobility. Along with 5G Network reliability and low latency, providing required connectivity all the time even under mobility is a key factor for autonomous driving. The speed of the AV can vary from 1miles/hour to 80miles/hr.

Seamless and smooth 5G network horizontal handover  is also another key factor for safe and reliable AV driving.

### 5.A.2	Pre-conditions

Editor's note:	Pre-conditions is FFS.

### 5.A.3	Service Flows

Alice has an autonomous car with 5G network subscription. One fine morning, Alice decides to go on a road trip from San Francisco, CA to yellow-stone national park, Montana. The following would be the sequence of operations involved with the 5G networks

Step 1: [AV Configuration]

- Alice turns on the AV and  AV authenticates itself with the 5G Network..

- AV registers its data measuring sensors with the 5G network (In this case, it is Lidars, radars and camera).

- Alice programs the AV to operate in Joint Sensing and communication mode.  (Mode is decided by the network-based applications such as Edge driving, remote driving, Extended driving, Advance driving, Platooning, co-operative awareness etc.,)

- AV initializes and uploads the configuration to the network-based assistance applicationsusing 5G network, and the server acknowledges the AV with the configured mode.

Step 2: [Registration of Autonomous Vehicle and Update for services provided by the 5G Network]

- When the AV is turned on, it registers and identifies with the 5G network. Based on the type of subscription and configured mode of driving, the AV initializes itself.

- AV initializes its sensors and actuators such as camera, lidar, short- and long-range radars and registers data measurement sensors with the 5G system.

- AV performs a Build in Test on the sensors and actuators along with network-based driving assistance to check if they are functional

- Upon successful registration and Initialization, the AV is ready with the navigation map to get the destination address.

Step 3: [Edge Assisted Navigation to the Destination]

- Once the destination address is keyed into the navigation system, the AV communicates the destined address to the network-based driving assistance server .

- The AV will start uploading the sensor data to the network-based driving assistance

- The network-based driving assistance will start assisting the AV with driving commands such as drive left, right, stop etc.,

- The network-based driving assistance uses the connected vehicles information to collective perform a safe and reliable driving assistance of the AV.

Step 4: [Cross-Traffic Left-Turn Assist]

- Network-based driving will assist AV to turn left across the traffic based on the planned future trajectory.

- Based on the measurement data from the sensors, collective information from other connected AVs. Network-based driving assistance will instruct AV to stop in case of an incoming traffic.

- Once the cross-traffic is safe to turn left based on sensors and other connected AVs. Network-based driving assistance will instruct AV to turn left.

### 5.A.4	Post-conditions

Editor's note:	Post-conditions is FFS.

### 5.A.5	Existing feature partly or fully covering use case functionality

Editor's note:	Existing feature covering use case is FFS.

### 5.A.6	Potential New Requirements needed to support the use case

[PR 5.A.6-1] The 5G system shall support obtaining sensing measurement data from the registered AV..

[PR 5.A.6-2] The 5G system shall Authenticate autonomous vehicles requiring 5G system sensing support.

[PR 5.A.6-3] The autonomous vehicles will register types of sensors requiring 5G system sensing support.

[PR 5.A.6-4] The 5G system shall support mechanism to export data measured from the AV sensors to application layer.

[PR 5.A.6-5] The 5G system shall support required network QoS support between network assisted driving applications and network assisted autonomous vehicles as per the KPI table-5.A.6-1.

[PR 5.A.6-6] The 5G system shall support enable/disable sensing measurement data from the registered AV.

[PR 5.A.6-7] The 5G system shall support the following KPI to support sensing data collection and/or autonomous driving.

Table- 5.A.6-1 Key Performance Metrics to support AV sensor data and support network-based driving assistance.

| Network Latency (msec) | Data Rate (Mb/sec) with dense multiuser | Reliability | 5G Range (meters) of communication |
| --- | --- | --- | --- |
| 3 – 10 | 10 -1000 | 90 - 99.99% | 50-1000 |

END OF CHANGE