3GPP TSG-SA WG1 Meeting #99e 	S1-222241

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-22xxxx)

Title:	New use case on distributed wireless sensing

Agenda Item:	7.2 - FS_Sensing

Source:	Philips International B.V.

Contact:	Walter Dees <walter.dees@philips.com>

Abstract: This contribution proposes a new use case for TR 22.837. All text is new.

---------- Use Case template ----------

5.X     Distributed wireless sensing

### 5.X.1 	Description

Tom is an elderly person living in his house. Since he has become weaker, he has subscribed to a wireless sensing service of his MNO so that his health state (breathing, hearth rate, falls) can be monitored 24/7 when he is at his home.

A single base station is not capable of covering Tom’s home with good coverage. Thus, multiple base stations capable of acting as wireless transmitters and/or receivers cooperate to ensure excellent coverage. Furthermore, the received reflected radar signal is sometimes weak, and thus, the MNO offers the possibility of using a phone with wireless sensing receiving capabilities. The usage of the phone also allows more accurate measurements of certain vital signs since the phone is close to Tom. The usage of the phone also allows the MNO to offload the workload from the base station to the phone.

Fig. 5.x.1.1 shows a schematic illustration of how such system could look like, whereby the blue arrow indicates transmitted wireless sensing signals from Base Station A, and the green dashed arrows indicate reflected wireless sensing signals received by Base Station B and Tom’s phone.

Figure 5.x.1-1 Example of a distributed sensing system (incl. two base stations, a UE and a Sensing server).

### 5.X.2	Pre-conditions

- Tom has subscribed to the sensing service offered by an MNO.

- The MNO has deployed two RAN entities (e.g. base station A and base station B) that are capable of wireless communication and sensing. The base stations can act as wireless sensing transmitters and/or wireless sensing receivers.  These two base stations are sufficiently close to Tom's house to provide good coverage in and around Tom's house in the frequency bands used for wireless sensing. Tom’s subscription includes a phone with wireless sensing capabilities for more accurate sensing.

- Tom is currently located in the living room. At this location, base station A can successfully transmit a strong sensing signal, however, base station A can hardly receive the reflection of its transmitted sensing signal. However, base station B can receive a strong reflection of that sensing signal.

- Tom has a mobile phone that is capable of detecting wireless sensing signals. Tom can use it to directly sense his health state.

### 5.X.3	Service Flows

The service flows are as follows:

- Based on Tom’s sensing subscription, information about a user (in this case Tom) is obtained including information about where he lives and sensing requirements that are needed (e.g. sensing of movements which can be used to detect falls or sufficient activity of Tom).

- Base station A starts transmitting the wireless sensing signal.

- Base station A and B coordinate with each other  so that Base Station B is capable of processing the received reflected wireless sensing signal, generating sensing measurement and/or sensing results that are sent to a sensing function for further processing.

- Tom decides to measure his health state. Tom picks up his phone and uses it as a wireless sensing receiver capable of picking up and processing the reflected wireless sensing signal transmitted by Base station A. This requires the phone to coordinate wireless sensing with Base station A, which may include for example exchanging of capabilities (since the sensing capabilities can differ per phone), and coordinating of timing/frequencies of sensing signals. Since Tom’s phone is very close to Tom, Tom can use his phone for more accurate sensing of certain vital signs. The phone sends measurements to a sensing function for further processing.

- When Tom’s health state is determined to be in danger, e.g., when Tom falls or stops moving, the family or emergency services may get alerted of such event.

### 5.X.4	Post-conditions

The sensing service/application receives accurate sensing measurements about Tom and can generate alerts if an adverse event happens to Tom.

### 5.X.5	Existing features partly or fully covering the use case functionality

### 5.X.6	Potential New Requirements needed to support the use case

[PR 5.X.6-1] The 5G system shall be able to coordinate wireless sensing between two RAN entities or between a RAN entity and a UE.

[PR 5.X.6-2] The 5G system shall be able to retrieve the wireless sensing capabilities from UEs and RAN entities.

[PR 5.X.6-3] The 5G system shall be able to configure two or more authorized UEs and/or RAN entities to take part in distributed wireless sensing of a target.

[PR 5.X.6-4] The 5G system shall be able to configure wireless sensing capable UEs and RAN entities with information of which network entity or destination server to send the sensing measurement data to.