3GPP TSG SA WG 1 Meeting #99e 	S1-222364

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-222123)

Source:	ZTE Corporation

pCR Title:	Pseudo-CR to resolve editor’s notes in clause 5.2

Draft Spec:	3GPP TR 22.840 v0.1.0

Agenda item:	7.3

Document for:	Approval

Contact:	bian.lanjian@zte.com.cn,

xu.ling@zte.com.cn,

dai.bo@zte.com.cn

Abstract: This contribution proposes KPIs and remove editor notes in TR22.840 clause 5.2 “medical instruments inventory management and positioning”.

1. Introduction

During SA1 #98e meeting, the use case of Medical instruments inventory management and positioning is accepted. In the potential requirements clause, there are three editor’s notes which need to be resolved.

In this pCR, it is proposed to resolve the editor notes.

2. Reason for Change

(1) It is suggested to remove the first editor’s note related to the following potential requirement :

[PR 5.2.6-2] The 5G network shall be able to provide group communication for a group of Ambient-IoT devices.

The reason is that in the description and service flow, it can be observed that the medical instruments inventory is operated based on different instrument type, e.g. orthopaedic forceps is one type of orthopaedic instrument. So, a group communication for this kind of Ambient-IoT devices which attached in these orthopaedic forceps is needed. The group management is out of scope of 5G network.

(2) It is suggested to remove the second editor’s note and revise the related potential requirement as follows:

[PR 5.2.6-3] The 5G system shall be able to provide a mechanism to collect the information stored in an Ambient-IoT device.

[PR 5.2.6-4] The 5G system shall be able to provide a mechanism to expose the information collected from an Ambient-IoT device to a trusted 3rd party.

The reason is that in the use case, the Ambient-IoT devices attached in the orthopaedic instrument are with very simple capability and not application installed on them. So, the command of collecting the inventory/status information of the orthopaedic instruments need to be delivered through the network layer e.g. 5G communication links to translate and deliver to help the Ambient IoT device understand the command then operate accordingly. So, the editor note is removed and further the original PR is split into two ones to make them more clearly.

(3) It is suggested to remove the third editor’s note and add the related KPIs.

3. Proposal

It is proposed to agree the following changes to 3GPP TR22.840 V0.1.0.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	International Energy Agency (IEA), "World Energy Outlook 2020". Available online: https://www.iea.org/reports/world-energy-outlook-2020

[3] 	Motlagh, N.H.; Mohammadrezaei, M.; Hunt, J.; Zakeri, B. "Internet of Things (IoT) and the Energy Sector." Energies 2020, 13, 494.

[4] 	Pereira F., Correia R., Carvalho N.B. "Passive Sensors for Long Duration Internet of Things Networks." Sensors 2017;17:2268. doi: 10.3390/s17102268.

[5] 	https://www.gs1.org/standards/rfid.

[6]	3GPP TS 22.368: "Service requirements for Machine-Type Communications (MTC)".

[7]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.011: “Service accessibility”.

[10]	5G for smart manufacturing: https://www.gsma.com/iot/wp-content/uploads/2020/04/2020-04_GSMA_SmartManufacturing_Insights_On_How_5G_IoT_Can_Transform_Industry.pdf

[11] 	 https://forcetechnology.com/en/articles/batteryless-electronics-energy-harvesting

[12] 	R. Ding and B. Xing, "Comparative Research on the Way of Energy Harvesting of the Wireless Sensor Network Nodes," 2013 6th International Conference on Intelligent Networks and Intelligent Systems (ICINIS), 2013

[13] 	P. D. Hung et al., "A Self-Powered Wireless Gas Sensor Node Based on Photovoltaic Energy Harvesting," 2021 Symposium on VLSI Circuits, 2021.

[14] 	GS1: "EPC Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard (Release 2.1) "

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

5.2	Medical instruments inventory management and positioning

5.2.1	Description

More and more medical instruments are utilized in hospital. They always need to be cleaned and sterilized, and shall withstand certain conditions e.g., high temperature, high pressure or humidity. Traditional inventory management for the medical instrument is usually operated manually, which is inefficient and even in some cases, causes serious accident e.g., lost or invalidity. To improve safe and efficient utilization of the medical instrument, remote medical instrument inventory and online maintenance are being developed.

For the remote inventory and online maintenance, the medical instrument is needed to be supplied with Ambient IoT device. Considering the working condition of the medical instrument, this kind of Ambient IoT device should be battery free or with limited energy storage capability, maintenance-free and should have long service life time. Through 5G network and the IoT device, the medical instrument exchange inventory management and positioning information with medical instrument management platform.

Following is an example of service flow to illustrate an inventory and positioning operation for medical instruments.

5.2.2	Pre-Conditions

Network operator UU deploys a new service “Ambient IoT” through its 5G system. Hospital Z has subscribed the new service for its orthopaedic instrument inventory management.

Orthopaedic instruments generally refer to professional medical instruments specially used for orthopaedic surgery. According to the usage purpose, it can be classified as orthopaedic knives, orthopaedic scissors, orthopaedic forceps, orthopaedic hooks, orthopaedic needles, orthopaedic scrapers, orthopaedic cones, orthopaedic drills, orthopaedic saws, orthopaedic chisels, orthopaedic files / shovels, orthopaedic active instruments, etc.

A number of Ambient-IoT devices recording different orthopaedic instrument information are stuck on these orthopaedic instruments. They are usually indoor stored in the instrument warehouse or medical instrument storage room or special storage cabinet.

The Ambient-IoT device is battery free or with limited energy storage capability.

These Ambient-IoT devices attached in the orthopaedic instrument are with very simple capability and not applications installed on them.

Belle is a nurse of Hospital Z. She has the authorization to remotely manage orthopaedic instrument through the inventory management platform of the hospital. She can operate this work in the hospital or out of the hospital.

5.2.3	Service Flows

Belle wants to acquire the inventory information of orthopaedic forceps. She uses her phone to send inventory request to the inventory management platform of the hospital. The platform requires the 5G network to collect all the inventory information of orthopaedic forceps in predefined duration, e.g. 10 seconds.

The 5G network then sends signals to the Ambient-IoT devices in Hospital Z. After receiving the signal, the ambient-IoT devices are active and can receive messages from 5G network.

Considering the Ambient-IoT devices attached on the repaired orthopaedic forceps are without application software, the 5G network translates application platform command and asks the Ambient-IoT devices attached on the orthopaedic forceps to report its status information.

Ambient-IoT devices on the orthopaedic forceps report the corresponding inventory information (e.g., the serial number of the orthopaedic forceps, usage status, usage records, years of use, integrity, etc.) to the inventory management platform of the hospital via 5G network.

The inventory management platform distributes the inventory information to Belle's phone.

After receives the status list of the orthopaedic forceps, Belle selects a pair of orthopaedic forceps from the list of orthopaedic instruments and clicks on “positioning request”. The request is passed to the inventory management platform. The inventory management platform asks 5G network to collect the position of the selected orthopaedic forceps.

The selected orthopaedic forceps is now being transported in the hospital handcart from Hospital Outpatient building to Hospital Inpatient building. Its moving speed is less than 6km/h.

The 5G network calculates the position of the selected orthopaedic forceps after receives response of the Ambient-IoT device attached on the orthopaedic forceps.

The 5G network then delivers the position information to the inventory management platform.

The platform responds the position information to Belle’s phone.

Thus, Belle acquires the position of the selected orthopaedic forceps on her phone.

5.2.4	Post-Conditions

Hospital Z utilizes Ambient-IoT service to support the remote inventory management of medical instrument. Belle can read the information of medical instrument. She can also find a medical instrument through the positioning information provided by ambient-IoT service.

5.2.5	Existing features partly or fully covering the use case functionality

None

5.2.6	Potential New Requirements needed to support the use case

[PR 5.2.6-1] The 5G system shall be able to communicate with an Ambient-IoT device.

[PR 5.2.6-2] The 5G system shall be able to communicate with a group of Ambient-IoT devices over a predefined duration.

[PR 5.2.6-3] The 5G system shall be able to provide a mechanism to collect the information stored in an Ambient-IoT device.

Editor’s Note: This requirement need to be revisited to clarify how the device can store information without application and what kind of data that can be stored.

[PR 5.2.6-4] The 5G system shall be able to provide a mechanism to expose the information collected from an Ambient-IoT device to a trusted 3rd party.

[PR 5.2.6-5] The 5G system shall be able to support positioning for an Ambient-IoT device.

[PR 5.2.6-6] The 5G system shall be able to provide communication service with KPIs listed in Table 5.2.6-1 for the Ambient IoT device/s.

Table 5.2.6-1  KPIs for use case of Medical Instrument Inventory management

| User experienced data rate (Kbps) | Communication service availability: (%) | End to end latency (ms) (note) | Device Density (#/Km2) | Communication range (m) | Moving speed (km/h) | Use cases |
| --- | --- | --- | --- | --- | --- | --- |
| DL: FFS UL: FFS | FFS | ≤ FFS | ≥ 1000 | FFS | Static or walking speed <6 | Medical instrument inventory management and positioning |

Editor’s Note: KPIs is FFS.

Editor’s Note: The KPI term and table format will be updated.

Editor’s Note: Other potential new requirements are FFS.