3GPP TSG SA WG 1 Meeting #99e 	S1-222227

Electronic Meeting, 22 August – 1 September 2022

Title:	Description of store and forward operation

Agenda item:	7.8

Source:	Sateliot, GateHouse, Novamint

Document for:	Approval

Contact:	Ramon Ferrús (ramon.ferrus@sateliot.space); René Brandborg (rbs@gatehouse.com)

Abstract: This contribution proposes to add, in an annexe, a description of store and forward operation.

* * * First Change * * *

Annex B (informative):
Store and forward operation

The support of S&F service delivery and S&F operation in a 3GPP system with satellite access is intended to provide the network with the ability to maintain a level of communications with UEs reachable from satellites in coverage areas where, or time periods when, the satellites are not connected (e.g. via a feeder link or via ISL) to the ground network when interacting with the UE over the service link and, therefore, there is no a continuous end-to-end connectivity path between the UE, the satellite and the ground network.

This is in-line with the scope and goals of the “Service Ubiquity” use-case categories and to some extent the “Service Continuity” use-case category identified in TR22.822. “Service Ubiquity” is attained by providing coverage in areas without satellite-ground network coverage while “Service Continuity” allows UEs to obtain service - albeit perhaps at reduced QoS or with reduced capabilities - when moving into areas without (a terrestrial network and) satellite-ground network coverage.

The concept of “S&F operation” is illustrated in Figure 1, in contrast to what could be considered the current assumption for the “normal/default operation” of a 3GPP network with satellite access.

As shown in Figure 1:

- Under “normal/default operation” mode, signalling and data traffic exchange between a UE with satellite access and the remote ground network requires the service and feeder links to be active simultaneously, so that, at the time that the UE interacts over the service link with the satellite, there is a continuous end-to-end connectivity path between the UE, the satellite and the ground network.

- In contrast, under “S&F operation” mode, the end-to-end exchange of signalling/data traffic is now handled as a combination of two steps not concurrent in time (Step A and B in Figure 1). In Step A, signalling/data exchange between the UE and the satellite takes place, without the satellite being simultaneously connected to the ground network (i.e. the satellite is able to operate the service link without an active feeder link connection). In Step B, connectivity between the satellite and the ground network is established so that any control information (e.g. user contexts) and user data traffic (messages gathered from, or addressed to devices) necessary for service delivery to the UE during Step A can be uploaded/downloaded to/from the satellite in Step B.

| “Normal/default operation” mode: Service link and feeder link are both active simultaneously for signalling/data exchange between the UE and the ground network. |  |
| --- | --- |
| “S&F operation” mode: Signalling/data exchange between the UE and the satellite can occur without the satellite being simultaneously connected to the ground network (i.e. service link can be kept operational even when there is no active feeder link connection) |  |

Figure 1.- Illustration of “normal/default operation” and “S&F operation” modes in a 3GPP network with satellite access.

Alternatively, or complementarily to the definition of the “S&F operation” mode, one may simply rely on the definition of “S&F services”, understood as information exchange services that do not rely on the existence of an end-to-end connectivity path between service end-points but allow information to be sent to an intermediate network node where it is kept and sent at a later time to the final destination or to another intermediate network node.

The concept of “S&F service” is widely used in the fields of delay-tolerant networking and disruption-tolerant networking. In 3GPP context, a service that could be assimilated to an S&F service is SMS, for which there is no need to have an end-to-end connectivity between the end-points (e.g. an end-point can be a UE and the other an application server) but only between the end-points and the SMSC which acts as an intermediate node in charge of storing and relying.

The support of S&F operation is especially suited for the delivery of delay-tolerant/non-real-time IoT NTN services with NGSO satellites.

* * * End of 1st Change * * *