3GPP TSG-SA1 Meeting #100	S1-223016

Toulouse, France, 14 Nov – 18 November 2022	(Revision of S1-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.989 | CR | 0022 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement of two use cases of Railway emergency communications: Termination of Railway Emergency Voice Communication and Initiation of Data communication during railway emergency alert |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2022-10-19 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on latest FRMCS Use cases requirements (reference MG-7900 - https://uic.org/rail-system/frmcs/), enhancements or clarification have been made by the Railways on the termination of the railway emergency voice communication and initiation of Data communication during railway emergency alert use cases. Alignment is necessary in the TR 22.989. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The main enhancements of the termination of the railway emergency voice communication are the followings: - capability for an authorised user (and not only a Controller) to put on hold, leave, or join an ongoing railway emergency voice communication - clarification on the possibilities to terminate an ongoing railway emergency voice communication automatically or by an entitled controller. The main enhancement of the initiation of Data communication during railway emergency is the presentation of the existence of data communications and read/unread status to all participants in the railway emergency alert. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Railway needs are not fully covered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.9., 6.4.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st change * * *

### 6.4.9	Use case: Termination of railway emergency voice communication

#### 6.4.9.1	Description

Based on operational rules, the configuration determines whether a FRMCS user within an ongoing railway emergency alert can put on hold, leave, or join an ongoing railway emergency voice communication.

During a railway emergency alert, only entitled Controllers are able to terminate the railway emergency voice communication.

After the termination of railway emergency alert, the railway emergency voice communication is automatically terminated (case of automatic initiation) or only entitled Controllers are able to terminate it (case of manual initiation).

A railway emergency voice communication is terminated when the last entitled Controller leaves it.

#### 6.4.9.2	Pre-conditions

The railway emergency voice communication is ongoing.

#### 6.4.9.3	Service flows

In the case a FRMCS user no longer meets the conditions of the railway emergency alert and if there is an ongoing railway emergency voice communication, the FRMCS user will remain in the ongoing railway emergency voice communication.

On hold

Based on configuration, a FRMCS user is able to put the railway emergency voice communication on hold.

After the FRMCS user has put the railway emergency voice communication on hold, the communication remains active in the FRMCS System. The FRMCS user is able to revert back to the communication again.

Leaving

Based on configuration, a FRMCS user in an ongoing railway emergency voice communication is able to leave the railway emergency voice communication.

Joining

A FRMCS user in an ongoing railway emergency alert is able to join an ongoing railway emergency voice communication.

Termination

An entitled Controller is able to terminate the railway emergency voice communication although the railway emergency alert is still ongoing.

After the termination of the railway emergency alert, in the case of an automatic initiation of a railway emergency voice communication, the communication is automatically terminated (see 6.4.7.3).

After the termination of the railway emergency alert, in the case of a manual initiation of a railway emergency voice communication, an entitled Controller of the ongoing railway emergency voice communication is able to terminate the communication (see 6.4.7.3).

The railway emergency voice communication is terminated when the last entitled Controller leaves it.

#### 6.4.9.4	Post-conditions

A FRMCS user has left or joined the railway emergency voice communication, or the railway emergency voice communication is terminated.

#### 6.4.9.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.4.9-001] | In the case a FRMCS User no longer meets the conditions of railway emergency voice communication, the FRMCS system shall remove the FRMCS user from the ongoing railway emergency voice communication. | A | 22.280 | R-5.3-XXX R-5.6.2.4.1-004 R-6.8.8.4.1-006 (Using User regroup, not 5.6.2.4.1-005) R-6.6.4.1-XXX User regroup MCX service Ad hoc Group Communication as an alternative to User regroup: R-6.15.5.2-014 |
| [R-6.4.9-002] | For a railway emergency voice communication, only authorised FRMCS user based on configuration shall be able to put the voice communication on hold. The railway emergency voice communication between the remaining FRMCS users shall not be affected. The authorised FRMCS user is able to take up the communication again. All involved FRMCS users are informed. | A | 22.280 | 5.4.1, 5.4.2, 5.1.5 6.4.4-003, 6.4.4-004 |
| [R-6.4.9-003] | For a railway emergency voice communication, only authorised user based on configuration shall be able to leave the communication. All involved FRMCS users are informed. | A | 22.280 | 6.4.4-002; 6.4.5-001; 5.4.2, 5.1.5 003-008 6.4.4-003, 6.4.4-004 6.4.9-001 |
| [R-6.4.9-004] | For a railway emergency voice communication, only an entitled Controller shall be able to terminate the communication. All involved FRMCS users are informed. | A | 22.280 | 6.4.9-001 6.8.8.4.2-XXX MCX service Ad hoc Group Communication as an alternative: 6.15.5.2-003 |
| [R-6.4.9-005] | The FRMCS system shall terminate a railway emergency voice communication when the last entitled Controller leaves it. All involved FRMCS users are informed. | A | 22.280 | 6.4.9-001 MCX service Ad hoc Group Communication as an alternative: 6.15.5.2-003 |

* * * End of 1st change * * *

* * * 2nd change * * *

### 6.4.10	Use case: Initiation of Data communication during railway emergency alert

#### 6.4.10.1	Description

Based on operational rules, additional information about the emergency can be exchanged using data communication. A system or a FRMCS user is able to send the additional information (e.g. text, voice prompts).

If real time video is required during the railway emergency communication, the FRMCS user is able to use the (Non-) Critical real time video application.

#### 6.4.10.2	Pre-conditions

The railway emergency alert is active.

#### 6.4.10.3	Service flows

A FRMCS User, Controller or an External System is able to send additional information (e.g. text, voice prompts).

All the FRMCS Users shall receive the additional information.

The existence of data communications and the read/unread status are indicated to all FRMCS users involved in the related Railway emergency alert.

The FRMCS System establishes the data communication within a setup time specified as IMMEDIATE (see 12.10). The information from the Role management and presence application is used to present the identities to the users involved in communication.

The location of the initiator of the data communication is presented to a mobile FRMCS User, Controller or to an External System. The location information is retrieved from the location services application.

The handling of the incoming data communication is managed by the prioritisation application. The alert has the priority which matches the application category of CRITICAL DATA (see 12.10) within the FRMCS System.

It shall be possible to receive data communication during railway emergency voice communication.

In the case of an ongoing Data communication during railway emergency alert, the additional user is added to the ongoing Data communication during railway emergency alert.

The communication is recorded by the FRMCS System Data recording and access to recorded data application.

#### 6.4.10.4	Post-conditions

Data communication is initiated.

#### 6.4.10.5	Potential requirements and gap analysis

Independently on any actions between the user and the end user device, the additional information provided by data communication is presented to the user.

The user shall be able to send additional information by data communication using simple user actions e.g. by selecting predefined messages.

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.4.10-001] | The FRMCS System shall allow a FRMCS User, Controller or an External System to send additional information (e.g. text, voice prompts, video) to FRMCS Users involved in a railway emergency alert or railway emergency voice communication. | A | 22.282 | R-5.2.2-014 |
| [R-6.4.10-002] | The FRMCS System shall be able to deliver the additional data to a railway emergency alert or railway emergency voice communication within a time specified as IMMEDIATE (see 12.10). | A | N/A | See section 12.10 below |
| [R-6.4.10-003] | The location of the initiator of the data communication shall be presented to all participants in the railway emergency alert or railway emergency voice communication. | A | 22.280 | 5.11 001, 008, 015 6.12 001,006,007 6.4.5 001, 003, 004 |
| [R-6.4.10-004] | For railway emergency alert /railway emergency voice communication the application layer priority of the communication shall be managed by the prioritisation application. | A | 22.280 | 5.4.2-XXX 5.6.X, 6.8.7.X |
| [R-6.4.10-005] | The FRMCS System shall be able to make available the content and communication related data of the railway emergency data communication for recording purposes. | A | 22.280 | 6.15.4-001 - 010 |
| [R-6.4.10-006] | The existence of data communications shall be presented to all participants in the railway emergency alert. | A | 22.280 | 5.6.2.4.1-003: consider adding presentation of additional data in MCX Service Emergency Alert content |
| [R-6.4.10-007] | The read/unread status shall be indicated to all participants in the railway emergency alert | A | 22.282 | 5.2.2-009: MCData SDS using the read receipt indication |

* * * End of 2nd change * * *