3GPP TSG-SA WG1 Meeting #100 	S1-223041

Toulouse, France, 14-18 November 2022	(revision of S1-22xxx)

Title:	Geofencing for Visual Line-of-Sight UAV missions

Agenda Item:	7.8

Source:	Orange

Contact:	philippe.lottin(a)orange.com

Abstract: The present contribution proposes to add, in the draft 3GPP TR 22.843, a new use case concerning geofencing for Visual Line of sight (VLoS).

1. Introduction

3GPP has specified a wide range of aerial features, activated through an "aerial subscription". Three types of C2 communications have been defined for R17, in TS.22.125:

- Direct C2 communications (when the UAV controller and UAV establish a direct C2 link to communicate with each other)

- Network-Assisted C2 communications (when the UAV controller and UAV communicate with each other via 5G network)

- UTM-Navigated C2 communication (for pre-scheduled autonomous flights)

3GPP features for Network-Assisted C2 communications and UTM-Navigated C2 communication mostly cover Beyond Visual Line-of-Sight (BVLoS) UAV operations, and Direct C2 communications are constrained by their radio coverage range, which is generally much wider than the VLoS constraints imposed by airspace authorities.

In all three cases, it is thus technically possible that an UAV controller does not comply with airspace regulatory requirements related to VLoS operations.

Ensuring that an aerial UE is complying with regulatory Visual Line-of-Sight constraints, by supporting pre-flight preparations and inflight operation flight monitoring and control, enhances the safety and security of drone operations.

2. Reason for Change

There is a need to enable the 5G system to be aware if VLoS constraints should be applied to UAV operations, and if so, to monitor the relative positions of an aerial UE with respect to its pilot to ensure that this aerial UE is complying with Line-of-Sight constraints.

3. Conclusions

The 5G system shall be able to ensure that an aerial UE is complying with Line-of-Sight constraints.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.843.

Begin of 1st Change

## 5.x	Use case: Geofencing for Visual Line-of-Sight UAV missions

### 5.x.1	Description

3GPP has specified a wide range of aerial features, activated through an "aerial subscription". Three types of C2 communications have been defined for R17, in TS.22.125:

- Direct C2 communications (when the UAV controller and UAV establish a direct C2 link to communicate with each other)

- Network-Assisted C2 communications (when the UAV controller and UAV communicate with each other via 5G network)

- UTM-Navigated C2 communication (for pre-scheduled autonomous flights)

3GPP features for Network-Assisted C2 communications and UTM-Navigated C2 communication mostly cover Beyond Visual Line-of-Sight (BVLoS) UAV operations, and Direct C2 communications are constrained by their radio coverage range, which is generally much wider than the VLoS constraints imposed by airspace authorities.

In all three cases, it is thus technically possible that an UAV controller does not comply with airspace regulatory requirements related to VLoS operations.

Ensuring that an aerial UE is complying with regulatory Visual Line-of-Sight constraints, by supporting pre-flight preparations and inflight operation flight monitoring and control, enhances the safety and security of drone operations.

There is a need to enable the 5G system to be aware if VLoS constraints should be applied to UAV operations, and if so, to monitor the relative positions of an aerial UE with respect to its pilot to ensure that this aerial UE is complying with Line-of-Sight constraints.

### 5.x.2	Pre-conditions

The UAV is associated with an aerial UE profile and a “VLoS” flag is activated together with its subscription. The UAV is associated with an UAV controller. The network is informed of the maximal distance authorized between the UAV and the UAV-C (VLoS distance threshold, usually around 200 or 300m), depending on airspace regulation.

### 5.x.3	Service Flows

1.	The UAV registers in 5GS as an aerial UE with the VLoS flag activated.

2.	The UAV is associated with an UAV-C. The UAV takes off.

2.	The network tracks the geolocalization of both the UAV and UAV-C, and deduces the distance between both.

3.	In case this distance exceeds the VLoS threshold, the network takes action, e.g. alerts the UTM and/or lets the UTM takes over the communication used to bring back the UAV within the authorized VLoS constraints, or sends a new waypoint within the authorized VLoS constraints.

### 5.x.4	Post-conditions

The UTM may take over the communication used to control the UAV and force the UAV to land safely.

Later, the UTM and/or network operator informs or reminds the user that a specific USIM needs to be used for proper operation of a UE on board a UAV.

### 5.x.5	Existing features partly or fully covering the use case functionality

-	"Aerial subscription" flag in subscription.

-	UAV / UAV-C association.

-	Tracking of the geolocalization of the UAV.

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] The 5G system shall be able to be informed of VLoS regulatory requirements (e.g. maximal distance threshold) and shall be informed if such requirements need to be met for a given UAS.

[PR 5.x.6-2] The 5G system shall be able to track the UAV-Controller, e.g. by receiving information from the UTM or through UE geolocalization.

End of Change