3GPP TSG-SA WG1 Meeting #102 	S1-231382

Berlin, Germany, 22 - 26 May 2023	(revision of S1-231086)

Title:	Consolidation proposal for TR 22.865

Agenda Item:	7.7

Source:	Novamint, Huawei, ETRI, Nokia, Gatehouse, China Telecom

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This document discusses the use cases and the associated potential requirements captured in TR 22.865 v1.0.0, and proposes the way forward for consolidation.

1. Introduction

The study on satellite access phase 3 has made good progress with 16 use cases and associated potential requirements captured in the TR 22.865. While new use cases and potential requirements may be proposed, this document discusses and proposes the way forward for consolidation.

The present document has been updated to take into account the following contributions:

- for consolidation:

- S1-231071 => now merged with the present document

- S1-231223 => now merged with the present document

- with revision of existing potential requirements:

- S1-231235 (1 potential requirement revised for Store & Forward Satellite operation)

- S1-231141 (1 potential requirement revised for UE-Satellite-UE communication)

- S1-231122 (1 potential requirement revised for General aspects)

- with potential new requirements:

- S1-231117 (1 new potential requirement for Store & Forward Satellite operation)

- S1-231118 (2 new potential requirements for Store & Forward Satellite operation)

- S1-231119 (2 new potential requirements for Store & Forward Satellite operation)

- S1-231120 (1 new potential requirement for Store & Forward Satellite operation)

- S1-231143 (1 new potential requirement for Store & Forward Satellite operation)

- S1-231121 (1 new potential requirement for UE-Satellite-UE communication)

- S1-231142 (1 new potential requirement for positioning and GNSS independent operation)

2. Discussion

The captured use cases can be summarised into 7 categories as listed in the table below.

| Category | Use cases |
| --- | --- |
| Store & Forward Satellite operation | 5.1 Use case on store and forward – MO 5.2 Use case on store and forward – MT 5.3 Use case on store and forward - Inter-satellite 5.4 Use case on data transfer for IoT devices in remote areas 5.16 Use case on store and forward – emergency report 5.6 Use case on Information Exchange between Ships at sea (new requirements related to S&F) |
| UE-Satellite-UE communication (without going through the ground network) | 5.6 Use case on Information Exchange between Ships at sea 5.7 Use case on the support of UE-satellite-UE phone call 5.8 Use case for enabling multiple communication services between UEs 5.10 Use case on vehicle fleet management in the desert 5.14 Use case on service continuity for UE-to-UE communication between satellites 5.15 Use case on service continuity for UE-to-UE communication in case of mobility between satellite and terrestrial network |
| GNSS independent operation & positioning enhancements for satellite access | 5.11 Use case on service differentiation for UEs via satellite access 5.12 Use case on UAVs using satellite access 5.13 Use case on Enhanced Positioning Service using Satellite Access |
| General and other aspects | 5.5 Use case on LAN using Satellite Access 5.15 Use case on service continuity for UE-to-UE communication in case of mobility between satellite and terrestrial network 5.9 Use case on usage of satellite connectivity for collection of information to aid terrestrial network planning |
| Security aspect | Related aspect in 5.1 Use case on store and forward – MO 5.2 Use case on store and forward - MT 5.3 Use case on store and forward - Inter-satellite 5.4 Use case on data transfer for IoT devices in remote areas 5.6 Use case on Information Exchange between Ships at sea 5.10 Use case on vehicle fleet management in the desert |
| Charging aspect | Related aspect in 5.4 Use case on data transfer for IoT devices in remote areas 5.6 Use case on Information Exchange between Ships at sea 5.7 Use case on the support of UE-satellite-UE phone call |

The table below lists the use cases and the associated potential requirements in each category.

| Category: Store & Forward Satellite operation |  |
| --- | --- |
| Use cases | Potential requirements |
| 5.1 Use case on store and forward - MO |  | [PR 5.1.6-001] The 5G system with satellite access shall be able to support store and forward operation. [PR 5.1.6-002] The 5G system with satellite access shall be able to inform a UE that store and forward operation is applied. [PR 5.1.6-003] Subject to operator policy, the 5G system with satellite access supporting store and forward operation shall be able to allow the operator or a trusted 3rd party to set and enforce, on a per UE basis, a S&F data retention period. [PR 5.1.6-004] Subject to operator policy, the 5G system with satellite access supporting store and forward operation shall be able to allow the operator or a trusted 3rd party to set and enforce, on a per UE basis, a S&F data storage quota. [PR 5.1.6-005] The 5G system with satellite access supporting store and forward operation shall be able to support a mechanism to configure and provision specific required QoS and policies for S&F operation (e.g. forwarding priority, acknowledgment policy). Potential New requirement from S1-231120 [PR.5.1.6-00x] Subject to operator’s policies, a 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support forwarding of the stored data from one satellite to another satellite, which has an available feeder link to the ground network, through Inter-Satellite Links. Potential New requirement from S1-231117 [PR 5.1.6-00y] The 5G system with satellite access supporting the S&F operation shall be able to support a mechanism to optimize the accumulated and stored MO messages delivery once the feeder link is available (e.g., priority, timer, timestamp, message size, etc). |  |
| 5.2 Use case on store and forward - MT | [PR 5.2.6-001] The 5G system with satellite access shall be able to inform a trusted application server whether store and forward operation is applied for communication with a UE. [PR 5.2.6-002] Subject to operator policy, the 5G system with satellite access supporting store and forward operation shall be able to allow the operator or a trusted 3rd party to set and enforce, on a per UE basis, a S&F data retention period. [PR 5.2.6-003] Subject to operator policy, the 5G system with satellite access supporting store and forward operation shall be able to allow the operator or a trusted 3rd party to set and enforce, on a per UE basis, a S&F data storage quota. [PR 5.2.6-004] The 5G system with satellite access supporting store and forward operation shall support a mechanism to configure and provision specific required QoS and policies for S&F operation (e.g. forwarding priority, acknowledgment policy). Potential New requirement from S1-231143 [PR 5.2.6-00x] The 5G system with satellite access shall be able to provide the information about the store and forward operation applied to a UE (e.g how long the data will be stored before being delivered) to a trusted third party application. Potential New requirements from S1-231118 [PR 5.2.6-00y] The 5G system with satellite access supporting the S&F operation shall be able to support a mechanism to optimize the stored MT messages delivery once the feeder link is available (e.g. priority, timer, timestamp, message size, etc). [PR 5.2.6-00z] The 5G system with satellite access supporting the S&F operation shall be able to support an optimized paging mechanism to deliver stored MT messages to UEs once the service link is available. |
| 5.3 Use case on store and forward - Inter-satellite | [PR.5.3.6-001] Subject to operator’s policies, a 5G system with satellite access shall be able to store data received from authorized UEs using delay-tolerant communication service while the feeder link is unavailable. [PR.5.3.6-003] Subject to operator’s policies, a 5G system with satellite access shall be able to define the maximum amount of data storage per satellite per authorized UEs using delay-tolerant communication service. |
| 5.4 Use case on data transfer for IoT devices in remote areas | [PR 5.4.6-001] The 5G system with satellite access shall support mechanisms to store user data, received from UEs via satellite access, on the satellite and forward it when feeder link between the satellite and the ground segment is available. [PR 5.4.6-002] The 5G system with satellite access shall support mechanisms for a user to securely register a UE to use the Store & Forward Satellite operation when satellite connectivity is intermittently/temporarily unavailable. NOTE: The user could be a human user using a UE with a certain subscription or a third party that is typically a business customer having service level agreement with the operator and interacting with the 5G network via an application server. [PR 5.4.6-004] The 5G system with satellite access shall be able to limit the total amount of the stored data received from a UE when using the Store & Forward Satellite operation. |
| 5.16 Use case on store and forward – emergency report | [PR.5.16.6-001] The 5G system with satellite access, and when the satellite access is operating in store and forward mode, shall be able to inform an authorized UE that currently the communication can be done without any storage. Editor's Note: this requirement is FFS. =>Revised in S1-231235 to [PR.5.16.6-001] The 5G system with satellite access, and when the satellite access is operating in store and forward mode, shall be able to know the report sent from an IoT device is for emergency purpose, shall forward the emergency report from an IoT device as soon as possible, and shall immediately notify the IoT device about the successful forwarding. [PR.5.16.6-002] The 5G system with satellite access, and when the satellite access is operating in store and forward mode, shall be able to inform an authorized UE about how long the data will be stored before being delivered. |
| 5.6 Use case on Information Exchange between Ships at sea | New requirements coming out of S1-231119 [PR 5.6.6-005] The 5G system with satellite access supporting the S&F operation shall support a mechanism to inform an active UE with ongoing satellite services of the feeder link status when the feeder link is becoming unavailable. [PR 5.6.6-006] The 5G system with satellite access supporting the S&F operation shall support a mechanism to handle ongoing on board satellite traffic without going through the ground when the feeder link is becoming unavailable (e.g., notifying UE of the feeder link status, then release traffic, or keep traffic). |
| Category: UE-satellite-UE communication (without going through the ground network) |  |
| 5.6 Use case on Information Exchange between Ships at sea | [PR 5.6.6-003] Subject to regulatory requirements and operator’s policy, the 5G system shall support communication between UEs using satellite access without going through the ground network. [PR 5.6.6-004] Subject to regulatory requirements and operator’s policy, the 5G system shall maintain service continuity with minimum service interruption of the communication between UEs using satellite access without going through the ground network when a UE changes from the coverage of one satellite to another (due to the movement of the UE and/or the satellites). |
| 5.7 Use case on the support of UE-satellite-UE phone call | [PR 5.7.6-001] Subject to regulatory requirements and operator’s policy, the 5G system with satellite access shall support providing connectivity between UEs without going through the ground network regardless if they are registered in the HPLMN or a VPLMN. |
| 5.8 Use case for enabling multiple communication services between UEs | [PR 5.8.6-001] Subject to regulatory requirements and operator’s policy, the 5G system with satellite access shall be able to provide a mechanism for QoS control of the communication between UEs using satellite access without going through the ground network. [PR 5.8.6-002] Subject to regulatory requirements and operator’s policy, the 5G system with satellite access shall be able to support different types of communication (e.g. services including unicast, multicast, broadcast) using satellite access without going through the ground network. |
| 5.10 Use case on vehicle fleet management in the desert | [PR 5.10.6-002] Subject to the operator’s policy, the 5G system shall support the traffic data exchanged between two UEs with different network connection paths (e.g. a direct network connection path and an indirect network connection path) to use satellite access without going through the ground network. Editor’s Note: It’s FFS to reword the description about direct/indirect network connection. =>Revised in S1-231141 to [PR 5.10.6-002] Subject to the operator’s policy, the 5G system with satellite access shall be able to maintain service continuity with minimum service interruption for a UE when the communication path changes between indirect network connection via satellite access and direct network connection via terrestrial access. |
| 5.14 Use case on service continuity for UE-to-UE communication between satellites | [PR 5.14.6-001] Subject to regulatory requirements and operator’s policy, the 5G system with satellite access shall be able to support service continuity of a communication between UEs without going through the ground network when the UE communication path moves between serving satellites. [PR 5.14.6-002] Subject to regulatory requirements and operator’s policy, the 5G system with satellite access shall support service continuity of a communication between UEs without going through the ground network when the communication path between UEs via one or multiple serving satellites extends across several satellites (through inter satellite links). Potential new requirement from S1-231121: [PR 5.14.6-00x] Subject to regulatory requirements and operator’s policy, the 5G system with satellite access shall be able to support the establishment of a communication path between UEs via one or multiple serving satellites without going through the ground network. |
| Category: GNSS independent operation & positioning enhancements for satellite access |  |
| 5. 11 Use case on service differentiation for UEs via satellite access | [PR 5.11.6-001] Subject to the regulatory requirements and operator’s policy, the 5G system with satellite access shall be able to provide services to an authorized UE independently of the UE’s GNSS capability. [PR 5.11.6-002] Subject to the regulatory requirements and operator’s policy, the 5G system with satellite access shall be able to provide services to an authorized UE registered to VPLMN independently of the UE’s GNSS capability. [PR 5.11.6-003] Subject to the operator’s policy, the 5G system with satellite access shall be able to determine the location of a UE using only satellite access (e.g. based on 3GPP positioning technologies, based on the information from reliable and trusted sources) in order to provide services in accordance with the governing national or regional regulatory requirements applicable to that UE. |
| 5.12 Use case on UAVs using satellite access | [PR 5.12.6-001] The 5G system with satellite access shall be able to support suitable positioning mechanisms for UAV services. [PR 5.12.6-002] The 5G system with satellite access shall be able to support positioning services and to provide information to a UE on delivered performance of positioning services. |
| 5.13 Use case on Enhanced Positioning Service using Satellite Access | [PR 5.13.6-001] Subject to regulatory requirements and operator’s policy, the 5G system with satellite access shall be able to support 3GPP positioning methods for UEs using only satellite access. [PR 5.13.6-002] The 5G system with satellite access shall be able to negotiate the positioning methods according to 3GPP RAT and UE positioning capability, the availability of non-3GPP positioning technologies (e.g. GNSS) and shall support mechanisms to allow the network or UE to trigger the negotiation. New requirement and KPIs coming out of S1-231142 [PR 5.13.6-003] The 5G system with satellite access shall be able to provide positioning services in compliance with regulatory requirements (e.g. for public safety services) independently of UE’s GNSS capability when the UE is using only satellite access. The minimum performance shall be compliant with the following KPIs |
| Category: General and other aspects |  |
| 5.5 Use case on LAN using Satellite Access | [PR 5.5.6-001] Subject to regulatory requirements and operator’s policies, the 5G system with satellite access shall be able to support an efficient communication path and resource utilization for a UE using only satellites access, e.g. to minimize the latencies introduced by satellite links involved. [PR 5.5.6-002] Subject to regulatory requirements and operator policy, the 5G system with satellite access shall be able to support service continuity when the UE communication path moves between satellite access network and terrestrial access network. |
| 5.15 Use case on service continuity for UE-to-UE communication in case of mobility between satellite and terrestrial network | [PR 5.15.6-001] Subject to regulatory requirements and operator’s policy, the 5G system with satellite access shall support service continuity, when the UE communication path moves between 5G terrestrial access network and 5G satellite access network owned by the same operator or owned by different operators having an agreement. =>Revised in S1-231122 to [PR 5.15.6-001] Subject to regulatory requirements and operator’s policy, the 5G system with satellite access shall support service continuity, when the communication path between UEs via satellite(s) changes to going through the satellite and the terrestrial network, and vice versa. |
| 5.9 Use case on usage of satellite connectivity for collection of information to aid terrestrial network planning | [PR 5.9.6-001] Subject to regulatory requirements and operator’s policies, the 5G system with satellite access shall be able to support collection of information on usage statistics and location of the UEs that are connected to the satellite, for network (e.g. terrestrial) planning. |
| Category: Security aspect |  |
| 5.1 Use case on store and forward – MO | [PR 5.1.6-006] The 5G system with satellite access shall be able to provide integrity protection and confidentiality for communications between an authorized UE and the network when store and forward operation is applied. Editor’s Note: this requirement is FFS. |
| 5.2 Use case on store and forward – MT | [PR 5.2.6-005] The 5G system with satellite access shall be able to provide integrity protection and confidentiality for communications between an authorized UE and the network when store and forward operation is applied. Editor’s Note: this requirement is FFS. |
| 5.3 Use case on store and forward – Inter-satellite | [PR.5.3.6-002] Subject to operator’s policies, a 5G system with satellite access shall be able to support forwarding of the stored data received from authorized UEs using delay-tolerant communication service from one satellite to another satellite through Inter-Satellite Links while preserving integrity protection, confidentiality and security of the data. [PR.5.3.6-004] The 5G system with satellite access shall be able to authorize the communication of a UE when the satellite access is operating in store and forward mode. |
| 5.4 Use case on data transfer for IoT devices in remote areas | [PR 5.4.6-003] The 5G system with satellite access shall support mechanisms to authenticate and authorize a UE for the Store & Forward Satellite operation. |
| 5.6 Use case on Information Exchange between Ships at sea | [PR 5.6.6-001] The 5G system shall support mechanisms to authorize the communication between UEs using satellite access (without going through the ground network) based on e.g., location information and subscription. |
| 5.10 Use case on vehicle fleet management in the desert | [PR 5.10.6-001] The 5G system shall support mechanisms to authorize a UE to use indirect network connection to communicate with another UE via satellite access without going through the ground network. |
| Category: Charging aspect |  |
| 5.4 Use case on data transfer for IoT devices in remote areas | [PR 5.4.6-005] The 5G system with satellite access shall be able to collect charging information per UE for use of the Store & Forward Satellite operation (e.g., data volume, duration, involved satellites). [PR 5.4.6-006] The 5G system with satellite access shall be able to collect charging information per application for use of the Store & Forward Satellite operation (e.g., number of UEs, data volume, duration, involved satellites). |
| 5.6 Use case on Information Exchange between Ships at sea | [PR 5.6.6-002] The 5G system shall support mechanisms to collect charging information for the traffic data exchanged using satellite access without going through the ground network. |
| 5.7 Use case on the support of UE-satellite-UE phone call | [PR 5.7.6-002] The 5G system with satellite access shall support collection of charging information for a UE registered to the HPLMN or a VPLMN, without going through the ground network. |

3. Proposal

Following this 2-step approach, please consider the consolidation of the potential requirements in each category summarised in the tables below. This is intended as the base for the consolidation discussion, and no much merging or revising of the original PRs is done.

# 7	Consolidated Potential Requirements

## 7.1	Store & Forward Satellite operation

The potential requirements corresponding to the support of Store & Forward Satellite information are listed in the table below

Table 1 – Consolidated Requirements for Store & Forward Satellite operation

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 1.1-1 | Subject to operator’s policies, a 5G system with satellite access shall be able to support Store & Forward Satellite operation for authorized UEs e.g. store data on the satellite when the feeder link is unavailable; and forward the data when feeder link between the satellite and the ground segment is available. | [PR 5.1.6-001] [PR.5.3.6-001] [PR 5.4.6-001] | Used the term “Store and Forward Satellite operation” as defined. Modified to merge 2 PR |
| CPR 1.1-2 | A 5G system with satellite access shall be able to inform a UE whether Store & Forward Satellite operation is applied. | [PR 5.1.6-002] | Used the term “Store & Forward Satellite operation” |
| CPR 1.1-3 | A 5G system with satellite access shall be able to inform a trusted application server or 3rd party application whether Store & Forward Satellite operation is applied for communication with a UE. | [PR 5.2.6-001] | Use the term “Store & Forward Satellite operation” |
| CPR 1.1-4 |  | Subject to operator’s policies, a 5G system with satellite access supporting Store and Forward Satellite operation shall be able to allow the operator or a trusted 3rd party to set and enforce, on a per UE and/or satellite basis, an S&F data retention period. |  | [PR 5.1.6-003] [PR 5.2.6-002] |  | Merged of these two PRs, applying to S&F data retention period. Used the term “Store and Forward Satellite operation” |  |
| CPR 1.1-5 | Subject to operator’s policies, a 5G system with satellite access supporting Store & Forward Satellite operation shall be able to allow the operator or a trusted 3rd party to set and enforce, on a per UE and/or satellite basis, an S&F data storage quota. | [PR 5.1.6-004] [PR 5.2.6-003] [PR.5.3.6-003] [PR 5.4.6-004] | Merged of these four PRs, applying to S&F data storage quota + adding “and per satellite basis” Used the term “Store & Forward Satellite operation” |
| CPR 1.1-6 | A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support a mechanism to configure and provision specific required QoS and policies for UE’s data subject to store and forward operation (e.g. forwarding priority, acknowledgment policy). | [PR 5.1.6-005] [PR 5.2.6-004] | Merged of these two PRs, applying to policies for S&F operation Used the term “Store & Forward Satellite operation” and avoid repetition |
| CPR 1.1-7 | A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to inform an authorized UE about how long the data will be stored before being delivered. | [PR.5.16.6-002] | Used the term “Store & Forward Satellite operation” |
| CPR 1.1-8 | A 5G system with satellite access supporting Store & Forward Satellite operation shall support mechanisms for a user to securely register a UE to use the S&F operation when satellite connectivity is intermittently/temporarily unavailable. NOTE: The user could be a human user using a UE with a certain subscription or a third party that is typically a business customer having service level agreement with the operator and interacting with the 5G network via an application server. | [PR 5.4.6-002] | Minor rewording. Used the term “Store & Forward Satellite operation” To be added in security requirements even to merge with CPR 1.6-2? Moved to security |

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 1.1-9 | Subject to operator’s policies, a 5G system with satellite access supporting Store & Forward Satellite operation shall be able to store, on the satellite, data received from authorized UEs when the feeder link is unavailable; and forward the data when feeder link between the satellite and the ground segment is available. | [PR.5.3.6-001] [PR 5.4.6-001] | Merge of these two PRs, applying to authorised UEs Use the term “Store & Forward Satellite operation” Minor rewording Merged with CPR 1.1-1 |

## 7.2	UE-Satellite-UE communication

The potential requirements corresponding to the support of UE-Satellite-UE communication (without going through the ground network) are listed in the table below

Table 2 – Consolidated Requirements for UE-Satellite-UE communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 1.2-1 |  | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support communication between UEs via one or multiple serving satellites without going through the ground network. NOTE: UEs can be connected via their HPLMN or a VPLMN. |  | [PR 5.6.6-003] [PR 5.7.6-001] |  | Using “5G system with satellite access” |  |
| CPR 1.2-2 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support service continuity (with minimum service interruption) of the communication between UEs not going through the ground network, when the UE communication path moves between serving satellites (due to the movement of the UE and/or the satellites). | [PR 5.6.6-004] [PR 5.14.6-001] | Merged of the two PRs, Using “5G system with satellite access” Small rephrasing |
| CPR 1.2-3 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support service continuity (with minimum service interruption) of the communication between UEs not going through the ground network, when the communication path between UEs extends from one to several satellites (through inter satellite links). | [PR 5.14.6-002] | Small rephrasing |
| CPR 1.2-4 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support service continuity, when the UE communication path moves between 5G terrestrial access network and 5G satellite access network owned by the same operator or owned by different operators having an agreement. | [PR 5.15.6-001] | Already addressed in CPR 1.4-2 |

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 1.2-5 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support communication between UEs without going through the ground network regardless if they are registered in the HPLMN or a VPLMN. | [PR 5.7.6-001] | Minor change to the PR Using “5G system with satellite access - To be merged in CPR 1.2-1 |
| CPR 1.2-4 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to provide QoS control of the communication between UEs using satellite access without going through the ground network. | [PR 5.8.6-001] | Rephrased |
| CPR 1.2.5 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support different types of communication (e.g. services including unicast, multicast, broadcast) using satellite access without going through the ground network. | [PR 5.8.6-002] |  |
| CPR 1.2-6 |  | Subject to the operator’s policy, a 5G system shall be able to support two UEs using different satellite access paths to communicate with each other without going through the ground network. (e.g. one UE using a direct satellite connection path, and one UE using a connection via a terrestrial relay with satellite access) |  | [PR 5.10.6-002] |  | EN needs to be resolved This requirement will be aligned based on the final discussion. Revision resolving issue + rewording |  |

## 7.3	GNSS independent operation & positioning enhancements for satellite access

The potential requirements corresponding to the support of GNSS independent operation & positioning enhancements for satellite access are listed in the table below

Table 3 – Consolidated Requirements for GNSS independent operation & positioning enhancements for satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 1.3-1 | Subject to the regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to provide services to an authorized UE registered to a HPLMN or a VPLMN independently of the UE’s GNSS capability. | [PR 5.11.6-001] [PR 5.11.6-002] | Merge of these two PRs |
| CPR 1.3-2 | Subject to the regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to determine the location of a UE using only satellite access (e.g. based on 3GPP positioning technologies, based on the information from reliable and trusted sources) in order to provide services. | [PR 5.11.6-003] | Simplified and aligned the wording |
| CPR 1.3-3 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support 3GPP positioning methods for UEs using only satellite access. | [PR 5.13.6-001] [PR 5.12.6-001] | Merged of these two PRs Simplify and align the wording |
| CPR 1.3-4 | A 5G system with satellite access shall be able to provide to a UE using only satellite access with information on positioning services (e.g. supported positioning performance). | [PR 5.12.6-002] | Rephrased |
| CPR 1.3-5 |  | A 5G system with satellite access shall be able to support negotiation of positioning methods, between UE or network, according e.g. to 3GPP RAT and UE positioning capability, the availability of non-3GPP positioning technologies (e.g. GNSS). |  | [PR 5.13.6-002] |  | Rephrased |  |

## 7.4	General and other aspects for satellite access

The potential requirements corresponding to the support of enhancements of general and other aspects of satellite access are listed in the table below

Table 4 – Consolidated Requirements for general and other aspects of satellite access

| CPR # |  | Consolidated Potential Requirement |  | Original PR # |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CPR 1.4-1 |  | Subject to regulatory requirements and operator’s policies, a 5G system with satellite access shall be able to support an efficient communication path and resource utilization for a UE using only satellites access, e.g. to minimize the latencies introduced by satellite links involved. |  | [PR 5.5.6-001] [PR 5.5.6-002] [PR 5.15.6-001] |  |  |  |
| CPR 1.4-2 | Subject to regulatory requirements and operator’s policies, a 5G system with satellite access shall be able to support collection of information on usage statistics and location of the UEs that are connected to the satellite, for network (e.g. terrestrial) planning. | [PR 5.9.6-001] |  |

## 7.5	Network Planning

The potential requirements corresponding to the support of network planning with satellite access are listed in the table below

Table 5 – Consolidated Requirements for network planning

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 1.5-1 | Subject to regulatory requirements and operator’s policies, a 5G system with satellite access shall be able to support collection of information on usage statistics and location of the UEs that are connected to the satellite, for network (e.g. terrestrial) planning. | [PR 5.9.6-001] |  |

## 7.5	Security aspects

The potential requirements corresponding to the security aspects are listed in the table below.

Table 5 – Consolidated Requirements for security aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 1.5-1 | Subject to operator’s policies, a 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support forwarding of the stored data received from authorized UEs using S&F operation from one satellite to another satellite through Inter-Satellite Links while preserving integrity protection, confidentiality and security of the data. | [PR.5.3.6-002] | Aligned with the defined term. |
| CPR 1.5-2 | A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support mechanisms to authorize a UE to use the S&F operation when satellite connectivity is intermittently/temporarily unavailable. NOTE: The authorizing entity could be an operator, a user/subscriber (UE owner) or a third party (e.g., a business customer having service level agreement with the operator and interacting with the 5G network via an application server). | [PR.5.3.6-004] [PR 5.4.6-003] [PR 5.4.6-002] | Merged these PRs and aligned with the defined terms. Simplify the wording incorporated [PR 5.4.6-002] in |
| CPR 1.5-3 | A 5G system with satellite access shall be able to support mechanisms to authorize the communication between UEs using satellite access without going through the ground network, based on e.g., location information and subscription. NOTE: UEs can use satellite access directly or via a terrestrial relay. | [PR 5.6.6-001] [PR 5.10.6-001] [PR 5.1.6-006] [PR 5.2.6-005] | Minor changes. incorporated [PR 5.10.6-001], [PR 5.1.6-006], [PR 5.2.6-005] in |

## 7.6	Charging aspects

The potential requirements corresponding to the charging aspects are listed in the table below.

Table 6 – Consolidated Requirements for charging aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 1.6-1 | A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to collect charging information per UE or per application (e.g., number of UEs, data volume, duration, involved satellites). | [PR 5.4.6-005] [PR 5.4.6-006] | Used the term “Store & Forward Satellite operation” |
| CPR 1.6-2 | A 5G system with satellite access shall be able to collect charging information for a UE registered to a HPLMN or a VPLMN, without going through the ground network. | [PR 5.6.6-002] [PR 5.7.6-002] | Merged of these two PRs Aligned the wording |

4. Potential addendum to the proposal

Below are potential new PR coming of new requirements and potential CPRs associated which will be considered by the consolidation only once the corresponding PR have been agreed in their respective contribution.

| CPR 1.1-x | Potential revised requirement from S1-231235 Subject to regulatory requirement, a 5G system with satellite access supporting Store & Forward Satellite operation shall be able to know the data sent from a UE is for emergency purpose, shall forward the emergency report from an UE as soon as possible, and shall immediately notify the UE about the successful forwarding. | Potential revised PR [PR.5.16.6-001] | Potential revised PR from S1-231235 |
| --- | --- | --- | --- |
| CPR 1.1-y | Potential New requirement from S1-231120 Subject to operator’s policies, a 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support forwarding of the stored data from one satellite to another satellite, which has an available feeder link to the ground network, through Inter-Satellite Links. | Potential new PR [PR.5.1.6-00x] | Potential new PR from S1-231120 Is a complement to CPR 1.1-1 Or addressed by CPR 1.5-1? |
| CPR 1.1-11 | Potential New requirement from S1-231117 Subject to operator’s policies, a 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support a mechanism to optimize the accumulated and stored MO messages delivery once the feeder link is available (e.g., priority, timer, timestamp, message size). | Potential new PR [PR.5.1.6-00y] | Potential new PR from S1-231117 To be discussed |
| CPR 1.1-12 | Potential New requirement from S1-231143 [PR 5.2.6-00x] A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to provide the information about the store and forward operation applied to a UE (e.g. how long the data will be stored before being delivered) to a trusted application server or third party application. | Potential new PR [PR 5.2.6-00x] | Potential new PR from S1-231143 If it is stored it means that the feeder link is not available hence the application server is not reachable |
| CPR 1.1-13 | Potential New requirement from S1-231118 [PR 5.2.6-00y] A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support a mechanism to optimize the stored MT messages delivery once the feeder link is available (e.g. priority, timer, timestamp, message size). | Potential new PR [PR 5.2.6-00y] | Potential new PR from S1-231118 To be discussed |
| CPR 1.1-14 | Potential New requirement from S1-231118 [PR 5.2.6-00z] A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support an optimized paging mechanism to deliver stored MT messages to UEs once the service link is available. | Potential new PR [PR 5.2.6-00z] | Potential new PR from S1-231118 To be discussed |
| CPR 1.1-15 | Potential New requirement from S1-231119 A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support a mechanism to inform an active UE with ongoing satellite services of the feeder link status when the feeder link is becoming unavailable. | Potential new PR [PR 5.6.6-005] | Potential new PR from S1-231119 To be discussed Partially already covered by CPR 1.1-1? |
| CPR 1.1-16 | Potential New requirement from S1-231119 A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support a mechanism to handle ongoing on board satellite traffic without going through the ground when the feeder link is becoming unavailable (e.g., notifying UE of the feeder link status, then release traffic, or keep traffic). | Potential new PR [PR 5.6.6-006] | Potential new PR from S1-231119 To be discussed Partially already covered by CPR 1.1-7? |
| CPR 1.2-7 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support the establishment of a communication path between UEs via one or multiple serving satellites without going through the ground network. | Potential new PR [PR 5.14.6-00x] | Potential new PR from S1-231121 May be merged in CPR 1.2-1 |
| CPR 1.3-x | Potential new PR from S1-231142 A 5G system with satellite access shall be able to provide positioning services in compliance with regulatory requirements (e.g. for public safety services) independently of UE’s GNSS capability when the UE is using only satellite access. The minimum performance shall be compliant with the following KPIs. | Potential new PR [PR 5.13.6-003] | Potential new PR from S1-231142 To decide where to put this PR as there is KPIs To incorporate in CPR 1.3-1? |