3GPP TSG SA WG1 Meeting #104	S1-233319

# Chicago, USA, 13-17 November 2023                                    revision of S1-233131,3306

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.837 | CR | 0019 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update sensing consolidated KPI table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Sensing |  |  |  |  |  | Date: |  |  | 2023-11-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There has a great progress on the sensing KPI table during the SA103. But there has still some open issues need to be addressed. For example, the principle of the KPI valuses for one category is to choose the strictest requirements. But some of KPI values in one categoria are diversity, so it is suggested to add a new principle to use a range in the case that the KPI values have a big difference, i.e. ≥10 times. |  |  |  |  |  |  |  |  |
| Summary of change: |  | it is suggested to revise the consolidated KPI tables: - To align the KPI values with the use cases in the TR. - To use value range when the KPI values of refreshing rate are very large, i.e. ≥10 times. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The KPI tables are not correct and inconsistency |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

7.2	Consolidated potential KPIs of sensing results

| Scenario | Sensing service category | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 1 (use cases 5.1; 5.13 – level1) | Object to be detected indoor: Human, object to be detected outdoor: UAV | 95 | 10 | 10 | N/A | N/A | 10 NOTE 2 | 5 NOTE 3 | 1000 | 1 | 5 | 2 | intruder detection in smart home, UAV intrusion detection |
|  | 2 (use cases 5.13 – level2, 5.6, 5.14) | Object to be detected outdoor: Human, UAV | 95 | 2 | 5 | 1 | N/A | 1 NOTE 2 | 1 NOTE 3 | 1000 | 0.2 | 0.1~5 | 5 | UAV flight route intrusion detection, intruder detection in surroundings of smart home, tourist spot monitoring |
|  | 3 (use cases 5.2, 5.7, 5.10, 5.11, 5.12, 5.23) | Factory (100m2), crossroad, highway, railway [air] NOTE 4 Object to be detected: Animal, Human, UAV, Vehicle | 95 | 1 | 1 | 1 NOTE 5 | 1 | 1 NOTE 5 NOTE 8 | 1 x 1 NOTE 9 | 100 NOTE 6 1000 NOTE 10 5000 for detection in highway | 0.05~1 NOTE 11 | 2 | 2 | pedestrian/animal intrusion detection on a highway/railway, sensing at crossroads with/without obstacle, UAV flight trajectory tracing UAV collision avoidance, AMR collision avoidance in smart factories |
|  | 4 (use cases 5.20, 5.22, 5.25, 5.32,5.27) | Factory and public safety Object to be detected: Animal, Human, UAV, AGV/AMR, Vehicle | 95, Public safety: 99 | 0.5 | 0.5 | 0.1; Vehicle: 15; Pedestrian: 1.5 | N/A; Pedestrian: 1.5 | 0.5m | factories: 0.5 x 0.5 | 100~5000 | 0.1 | 1 | 3 | Parking Space Determination, UAVs/vehicles/pedestrians detection near Smart Grid equipment (NOTE 7), immersive experience based on sensing, integrated sensing and positioning in factory hall |
|  | 5 (use cases 5.28) | ADAS Object to be detected: Vehicle Public area safety, | 95 | short-range radar:2.6 Long range radar: 1.3 | 0.5 | 0.12 | N/A; | 0.4 | 0.6 | Short range radar: 20; Long range radar: 50 | Short range radar: 0.05; Long range radar: 0.2 | 10 | 1 | public safety search and rescue or apprehend, ADAS |
| Environment monitoring | 6 (use cases 5.3 and 5.5.) | Rainfall monitoring and flooding NOTE 14 Object to be detected: Rain | 95 | 10 | 0.2 NOTE 15 | N/A | N/A | N/A | N/A | 60000 | 1<10min, | 0.1~5 | 3 | rainfall monitoring, flooding monitoring |
| Motion monitoring | 7 (use cases 5.15, 5.24) | Indoor human motion -sleep monitoring NOTE 12, sports monitoring NOTE 13, | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 60000 | 60 | 5 | 5 | sleep monitoring, sports monitoring |
|  | 8 (use case 5.29) | Hand gesture recognition | 95 | 0.2 | 0.2 | 0.1 | 0.1 | 0.375 | 0.3 | 5~50 | 0.1 | 5 | 5 | Hand gesture recognition |
| NOTE 1: The terms in Table 7.2-1 are found in Section 3.1. NOTE 2: To detect the UAV existance (e.g., for intrusion detection), the sensing resolution of distance is 10m [25]. NOTE 3: To detect the UAV existence, the sensing resolution of velocity is 10m/s [25]. NOTE 4: The typical size (Length x Width x Height) of UAV is 1.6m x 1.5m x 0.7m, the typical size of pedestrian is 0.5m x 0.5m x 1.75m, and the typical size of engineering vehicle is 7.5m x 2.5m x 3.5 m. NOTE 5: The KPI values for UAVs are sourced from [25] and [40] and for factories are sourced from [47]. NOTE 6: The value 100 ms is sourced from [28] and is valid for sensing at crossroads. NOTE 7: The safe distance between pedestrian/vehicle and transmission station/line is 0.7m/0.95m [46]. The size of the park of Smart Grid depends on the real environment. NOTE 8: To track the UAV flying (e.g., for collision detection and warning), the sensing resolution of distance is 1m [25]. NOTE 9: To track the UAV flying, the sensing resolution of velocity is 1m/s [25]. NOTE 10: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1s. NOTE 11: Echodyne MESA-DAATM has approximate 1Hz scan rate [40]. NOTE 12: Additional KPI on human motion rate accuracy of 2 times/min (0.033 Hz). NOTE 13: Additional KPI on human motion rate accuracy of 3 times/min (0.05Hz) and 4 times/min (0.07 Hz) NOTE 14: Rainfall estimation accuracy is1 mm/h[39] and describes the closeness of the measured rainfall estimation to its true rainfall value. NOTE 15: This value is for the water level. Description related to NOTE in clause 5.5.1 suggests 0.01 m. [≤0.2] is derived from the water level where people feel difficulty in walking. NOTE 16: The KPI value with range is configurable based on the application demand. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |