3GPP TSG-SA WG1 Meeting #106	S1-241190

Jeju , Korea, 27-31 May 2024													(revision of S1-xxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.989 | CR | 0031 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update and Gap analysis of Transfer (Divertion) of an incoming voice communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_FRMCS_Ph6 |  |  |  |  |  | Date: |  |  | 2024-05-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce call divertion for Ad hoc Group communications to fulfiil functional requirements of FRMCS FRS (FU-7120) |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Define conditions of automatic call divertion, 2.Update of potential requirements and gap analysis for Ad hoc Group communications |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing requirements in the stage 1 specifications, to allow subsequent groups implement call divertion with Ad hoc Group communications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.1, 6.22.2, 6.22.2.1, 6.22.2.2, 6.22.2.3, 6.22.2.4, 6.22.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

## 6.22	Voice communication transfer related use cases

### 6.22.1	Introduction

In this chapter the use cases related to the  divertion of incoming or transfer of ongoing voice communication are defined. The following use cases are defined:

-	Divertion of an incoming voice communication

-	Transfer of an ongoing voice communication

-	Service interworking and service continuation with GSM-R

To provide some background, a description of one use case based on operational rules is described: a railway station could temporarily be closed for any operational reasons. In that case, railway staff (e.g., controller) could not be joined anymore. A mechanism of transferring calls to railway staff located in another railway stations shall be provided.

The use cases are applicable for user-to-user and multi-user voice communications.

### 6.22.2	Use case: Divertion of an incoming voice communication

#### 6.22.2.1	Description

Prior to the presentation of a communication to an FRMCS user, the FRMCS system shall support automatic divertion of the communication to another FRMCS user of the same FRMCS domain, if one of the divertion conditions activated by an authorised user, are fulfilled.

The FRMCS system shall also support divertion of call, if activated by an authorised user, in case of rejecting or ignoring a presented call.

#### 6.22.2.2	Pre-conditions

FRMCS User is registered to a single or multiple FRMCS Equipment.

FRMCS User is or is not part of an ongoing communication.

#### 6.22.2.3	Service flows

The FRMCS System provides and activates a mechanism to automatically divert communications.

FRMCS User is authorised to configure the mechanism to divertcommunications.

The FRMCS system shall enable automatic transfer of communication to another FRMCS user of the same FRMCS domain, even before presentation of the communication to the destinated FRMCS user, if one of the following conditions of diverting communications is fulfilled:

- All incoming calls are diverted

- If the user is already connected to another communication

- If the user doesn’t reply during a configurable time or number of rings

- If the user is not reachable.

- If the user rejects the call.

FRMCS User has already configured the new FRMCS User out of a list (e.g. telephone book) or entered manually based on either FRMCS User Identity, FRMCS Functional Identity.

The arbitration is managed by the arbitration application.

#### 6.22.2.4	Post-conditions

The incoming voice communication is diverted to the new FRMCS User by the FRMCS System.

6.22.2.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.22.2-001] | The FRMCS System shall enable authorised FRMCS User to configure divertion of incoming voice communications to another FRMCS user of the same FRMCS domain, based on different criteria | A/T | 22.280 22.179 | Limited to private calls because group communication already provides other meachnisms [R-5.6.3-015], [R-6.7.4-016] a) Call Divertion is missing for Ad hoc Group calls. b) Authorisation of Call Divertion for Ad hoc Group calls to another FRMCS User of the same FRMCS domain based on FRMCS user identity, functional identity, or subparts of functional identities is missing. Editor’s note 1: How to handle Functional Alias(es) activated by multiple users? Editor’s note 2: CR is needed |
| [R-6.22.2-002] | The FRMCS System shall support the divertionof incoming voice communications based on one of the following criteria: - All incoming communications are diverted; - When FRMCS user is not reachable (e.g. out of coverage); - Busy (i.e., the FRMCS user is connected in another call); - When the FRMCS user is not answering the incoming call within the set (time) interval; - When the FRMCS user rejects the presented communication | A/T | 22.179 22.280 | Partly covered (Busy excluded). Limited to private calls because group communication already provides other meachnisms [R-5.6.3-015], [R-6.7.4-016]. Busy concept no more applicable to MCX: an MCX UE/client can be simultaneously engaged in multiple private calls, or even mixed in private and group calls (see 22.179 5.5 & 22.280 5.4 & 6.14). a) All the divertion conditions mentionned, are missing for Ad hoc Group calls b) Busy for Ad hoc Group call can be handled at application level, so out of scope of 3GPP. (e.g., based on arbitration rules) b) Divertion of presented and rejected call by the user is missing for Private calls and Ad hoc Group calls. Editor’s note: CR is needed |
| [R-6.22.2-003] | The FRMCS System shall be able to support configurable time or number of rings when the FRMCS User is not answering | A/T | 22.179 22.280 | Partly covered (Busy excluded). Limited to private calls because group communication already provides other meachnisms [R-5.6.3-015], [R-6.7.4-016]. Busy concept no more applicable to MCX: an MCX UE/client can be simultaneously engaged in multiple private calls, or even mixed in private and group calls (see 22.179 5.5 & 22.280 5.4 & 6.14). Call divertion of Ad hoc Group call based on configurable time or number of rings for not answering, is missing. Editor’s note: CR is needed |
| [R-6.22.2-004] | The FRMCS System shall support the transfer of incoming voice communications for user-to-user and multi-user. | A/T | 22.280 22.179 | Indicated by sub-clause 5.6.3 Private Call (with Floor control) commencement requirements 6.7.4 Private Call (without Floor control) commencement requirements. Call divertion is missing for Ad hoc Group call. Same comment as for [R-6.22.2-001]. |
| [R-6.22.2-005] | The FRMCS System shall be aware of the busy condition of the targeted FRMCS user to forward the incoming voice communication to another FRMCS User. | A/T | 22.179 | Partly covered (Busy excluded). Limited to private calls because group communication already provides other meachnisms [R-5.6.3-015], [R-6.7.4-016]. Busy concept no more applicable to MCX: an MCX UE/client can be simultaneously engaged in multiple private calls, or even mixed in private and group calls (see 22.179 5.5 & 22.280 5.4 & 6.14). Busy for Ad hoc Group call can be handled at application level, as per comment of [R-6.22.2-002]. |
| [R-6.22.2-006] | The FRMCS system shall provide information to the initial destinator of the call that the incoming call has been suceesfully diverted sending back the new FRMCS User’s identity or functional identity. | A/T | 22.280 22.179 | Notification to the initial destinator about the divertion call status and new FRMCS User’s Identity or functional identity is missing in Ad hoc Group calls. |

End of changes