3GPP TSG SA WG 1 Meeting #109 	S1-250185

Athens, Greece, 17-21 February 2025	(revision of S1-25xxxx)

Title:	Use Case on enhanced user experience with mega LEO constellation

deployment

Agenda item:	8.1.4

Source:	China Telecom

Contact:	Wen Qi (qiw@chinatelecom.cn)

Xu Xia (xiaxu@chinatelecom.cn)

Abstract: This contribution provides a new use case for 3GPP system with regenerative LEO satellite access and the corresponding requirements, where the LEO satellites deployed are from the mega-constellation.

1. Introduction

“Ubiquitous Connectivity” has been identified as one of key areas for 6G study. This contribution proposes a new use case on .

2. Reason for Change

The method of obtaining satellite connection information through configuration-based approaches poses significant challenges for large-scale low Earth orbit constellations, leading to high call setup delays in UE-to-Satellite-to-UE services. Furthermore, the inconsistency in network functions deployed on satellites makes it difficult to ensure uninterrupted voice quality, especially during satellite changes.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change (All new)* * * *

8.x	Use case on enhanced user experience with mega LEO constellation deployment

8.x.1	Description

Currently, the method used by 5G system to obtain satellite connection information relies on configuration-based approaches. This method poses significant challenges, particularly for large-scale LEO constellations. Due to this, and the way IMS obtains user satellite connection information, it takes a considerable amount of time for the network to determine whether two users can establish a UE-to-Satellite-to-UE communication. As a result, the delay in call setup becomes quite high, especially when there are changes in the satellites the users are connected, which results frequent interactions between the satellite and the remote terrestrial network during the process. Furthermore, considering that the network functions deployed on satellites may not be fully consistent, maintaining voice quality without degradation during the call faces a difficult challenge.

8.x.2	Pre-conditions

Operator#A is a satellite operator with launch plans for a large-size constellation of LEO satellite.

Alice has subscribed the UE-Satellite-UE service of Operator#A and her smartphone is capable to directly connect with Operator#A’s satellites.

Bob has subscribed the UE-Satellite-UE service of Operator#A and his smartphone is capable to directly connect with Operator#A’s satellites.

8.x.3	Service Flows

- Operator#A completes the deployment of a large-size LEO constellation.

- Alice travels to the island with her friend Bob. Both Alice's and bob's phones have switched to Operator#A's satellite network. Alice follows the boat out for fishing, while Bob stays on the island to enjoy the scenery. Alice wants to call her friend Bob to ask him to wait for her for dinner.

- The 6G network quickly verifies whether the current satellite connections of Alice and Bob can support UE-to-Satellite-to-UE service, even if their connected satellites change during the verification process.

- After the verification is successful, Bob’s phone rings and he answers the call. They talk for several minutes and throughout the call, the UE-Satellite-UE service remains available and to different ones, their call without any interruption, even though their accessing satellites changes to different ones.

8.x.4	Post-conditions

Thanks to the support of UE-Satellite-UE service with mega-constellation deployment, the users can have a smooth experience of UE-Satellite-UE service, quickly completing the verification processing, and maintaining same voice quality throughout the call, even with frequent satellite changes.

8.x.5	Existing features partly or fully covering the use case functionality

Clause 6.46.9 of TS 22.261 defines the requirements for 5G systems with satellite access.

This use case proposes new requirements for satellite access in case of mega LEO constellation deployment.

8.x.6	Potential New Requirements needed to support the use case

[PR 8.x.6-1]: The 6G system using satellite access, shall be able to provide a mechanism to obtain and manage the connection information of a mega-constellation.

[PR 8.x.6-2]: Subject to regulatory requirements, the 6G system using satellite access, shall be able to provide a mechanism to quickly verify that the users’ current access conditions meet the UE-Satellite-UE service requirements, especially when the access information keeps changing.

[PR 8.x.6-3]: Subject to regulatory requirements, the 6G system using satellite access, shall be able to provide a mechanism to ensure the voice quality of UE-Satellite-UE service, especially when the satellite deployes different network functions and the satellite connection information keeps changing.