3GPP TSG SA WG 1 Meeting #109 	S1-250321

Athens, Greece, 17-21 February 2025	(revision of S1-25xxxx)

Source:	Deutsche Telekom

Title:	Discussion paper on UE and (Ambient) IoT device definitions

Agenda item:	2

Document for:	Endorsement

Contact:	kurt.bischinger@magenta.at

1. Introduction

During discussions on the definition of Ambient IoT device in SA#106 based on discussion paper SP-241876, it was clarified that there are two separate definitions for UE:

Definition in TS 22.261 not mentioning UICC:

User Equipment: An equipment that allows a user access to network services via 3GPP and/or non-3GPP accesses.

Definition in TR 21.905 mentioning UICC:

User Equipment (UE): Allows a user access to network services. […]Currently the User Equipment is subdivided into the UICC domain and the ME Domain. […]

TSG SA asked SA WG1 to review both definitions and to provide the necessary clarification CR(s) to either TS 22.261, TR 21.905 or both.

2. Background

The trigger for this was a discussion concerning Ambient IoT. An Ambient IoT device is defined as a type of IoT device, which in turn is a type of UE. Therefore, by inference an Ambient IoT device can be seen as a type of UE according  to TS 22.261. It was also discussed that the definition in TS 22.261 would take precedence for the requirements in TS 22.261 and the related requirements in TS 22.369. Hence, the definition of Ambient IoT device does not automatically imply that Ambient IoT device has a UICC, even if TR 21.905 mentions otherwise.

A potential issue was identified: downstream working groups (e.g. SA2, SA3) may use the definition of User Equipment of TR 21.905 instead of TS 22.261. Hence, clarification of the definitions is needed.

3. Discussion

At the beginning of the work for 5G there was the desire to have a more generic definition of UEs (as quoted above) to allow looking into new types of devices. As a result of the studies IoT devices were defined as follows in 22.261:

IoT Device: a type of UE which is dedicated for a set of specific use cases or services and which is allowed to make use of certain features restricted to this type of UEs.

Which features this would be, is specified in TS 22.261. One of the most prominent features is in clause 8.3 Authentication:

The 5G system shall support operator-controlled alternative authentication methods (i.e. alternative to AKA) with different types of credentials for network access for IoT devices in isolated deployment scenarios (e.g. for industrial automation).

Observation 1:

It should be noted that the use of alternative authentication methods, which covers authentication without USIM, is only allowed in isolated deployment scenarios. It is by no means a blank cheque for IoT devices to be operated without USIM in e.g. PLMNs.

Clause 6.14 Susbcription aspects further clarifies that

An IoT device which is able to access a 5G PLMN in direct network connection mode using a 3GPP RAT shall have a 3GPP subscription.

and

An IoT device which is able to connect to a UE in direct device connection mode shall have a 3GPP subscription, if the IoT device needs to be identifiable by the core network (e.g. for IoT device management purposes or to use indirect network connection mode).

Observation 2:

It should also be clear that although the definition of the User Equipment in 22.261 is more generic this does not mean that a UE is not required to have a USIM.

E.g. 22.101 requires in clause 14 Types of features of UEs:

The basic mandatory UE requirements are:

- Support for USIM. […]

- An unalterable equipment identification; IMEI, see 3GPP TS 22.016 [12]; […]

Also, many features exist that rely on functionalities of the USIM (e.g. network selection, unified access control, selection of relay UEs, emergency number lists, and many more) which makes clear that a UE requires a USIM – with the only exception as given above for IoT devices in isolated deployments.

However, as can be seen by the ongoing discussion, it is obvious that the different definitions can cause confusion. There are two possible ways to solve the issue:

- Change the definition in 21.905 to the generic one of 22.261.

There is the danger that such a change would be misinterpreted by some people to open up the possibility for UEs, IoT devices and Ambient IoT devices without USIM. Especially as the definition in 21.905 has been existing since Rel-99, this could be a source of even more confusion.

- Decouple the definition of IoT device in 22.261 from the definition of UE in 22.261 and delete the definition of UE in 22.261.

Making the definition of IoT device in 22.261 more independent would allow to keep the definition of the UE in 21.905.

In a further step, if agreeable, the definition of Ambient IoT devices could be decoupled from the definiton of IoT devices.

A possible change of the IoT device definition could look like this:

IoT Device: a device similar to a UE which is dedicated for a set of specific use cases or services and which is allowed to make use of certain features restricted to this type of devices. All requirements for UEs also apply to IoT devices except for the herein specified exceptions.

NOTE 5:	An IoT device may be optimized for the specific needs of services and application being executed (e.g. smart home/city, smart utilities, e-Health and smart wearables). Some IoT devices are not intended for human type communications.

4. Proposal

It is proposed to agree on Option 2) above – change the definition of IoT device as proposed, and delete the definiton of UE in 22.261.

If agreed, the author will prepare the corresponding CR to 22.261 for Release 19.