3GPP TSG-SA WG1 Meeting #109	S1-250354

Athens, Greece, 17-21 February 20225								        (revision of S1-250315)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.179 | CR |  | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ambient listening requirements – pause and resume operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UK Home Office, FirstNet, Netherland Police, AT&T and Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | ALPR-REQ |  |  |  |  |  | Date: |  |  | 2025-01-31 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adds new requirements to ambient listening to enable pause and resume of ambient listening MCPTT calls. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to introduce a pause and resume feature for ambient listening calls when the "MCPTT User being ambient listened to" starts an MCPTT Private Call or an MCPTT Group Call. New requirements are added to TS 22.179 clause 6.16.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mission critical community would lack these important operational requirements for ambient listening. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.16.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S1-250069, S1-250315 |  |  |  |  |  |  |  |  |

Start of changes

### 6.16.2    Ambient listening

#### 6.16.2.1        Overview of ambient listening

Ambient Listening is a feature that allows an authorized MCPTT User, typically a dispatcher, to cause an MCPTT UE to initiate a call which results in no indication on the MCPTT UE that it is transmitting. Ambient Listening can be initiated by an authorized MCPTT User who wants to be listened to by another remote authorized MCPTT User or can be initiated by a remote authorized MCPTT User who wants to listen to another MCPTT User. The purpose of this feature allows a dispatcher to listen to activities at the Location of the remote MCPTT UE to find out what is happening around that MCPTT UE without providing an indication to the MCPTT User or people around the user (whom the MCPTT User does not want to make aware of this action) that this is happening. This type of call is different from other types of call, as for Ambient Listening audio is only transmitted to one party in the call (i.e., a dispatcher or an authorized MCPTT User that is acting in a similar role to a dispatcher).

This is used for stolen MCPTT UEs, monitoring officers, officer safety and particular operations, where it is important that the MCPTT UE does not indicate what is happening.

#### 6.16.2.2        Ambient listening requirements

##### 6.16.2.2.1            General Ambient Listening requirements

[R-6.16.2.2.1-001] Void

[R-6.16.2.2.1-002] Void

[R-6.16.2.2.1-003] Void

[R-6.16.2.2.1-004] The MCPTT Service shall either terminate Ambient Listening if the MCPTT User being listened to starts to transmit or receive  an MCPTT Private Call or an MCPTT Group Call or shall execute one of the two following options.

NOTE:    An authorized MCPTT User could initiate Discreet Listening at this point if needed

[R-6.16.2.2.1-005] The MCPTT service shall pause an Ambient Listening call if the MCPTT User being listened to starts to transmit or receive  an MCPTT Private Call or an MCPTT Group Call and resume the paused Ambient Listening call once the interrupting call that caused the pause has concluded.

[R-6.16.2.2.1-006] The MCPTT service shall continue the ambient listening call without pause or termination, in parallel with a transmission or reception of an MCPTT Private Call or an MCPTT Group Call, if the MCPTT service allows two parallel calls.

##### 6.16.2.2.2            Remotely initiated Ambient Listening requirements

[R-6.16.2.2.2-001] Void

[R-6.16.2.2.2-002] Void

##### 6.16.2.2.3            Locally initiated Ambient Listening requirements

[R-6.16.2.2.3-001] Void

[R-6.16.2.2.3-002] Void

End of changes