3GPP TSG SA WG 1 Meeting #109 	S1-250364

Athens, Greece, 17-21 February 2025	(revision of S1-250211)

Source:	CEWiT

pCR Title:	New use case on UE assisted UE Energy Efficiency Improvement and Service Optimization by the Network

Draft Spec:	3GPP TR 22.870 v0.1.1

Agenda item:	8.1.1

Document for:	Approval

Contact:	Ankita Lachhwani, ankita.lachhwani@cewit.org.in

Abstract: This contribution proposes a new case UE Energy Efficiency Improvement and Service Optimization use case where the UE(s) have SLA with the network, to monitor and customize its services for extending its battery life dynamically based on its battery and radio conditions.

1. Introduction

This contribution proposes a new UE Energy Efficiency Improvement and Service Optimization use case.

2. Reason for Change

With the existing use case in TR 22.870, Clause 5.5 “Supporting end-to-end energy efficiency is an important design target for the sustainability of 6G system.” The UE energy efficiency as service was proposed where UE will assist the network by indicating its power saving mode based on the specific scenarios and on user’s decision-making capabilities. The use case in clause 5.5 is restricted to specific users who have the knowledge and decision-making capabilities as mentioned above.

4. Proposal

It is proposed that agree the following changes to 3GPP TR 22.870

* * * First Change * * * *

5.2	Use case on UE assisted UE Energy Efficiency Improvement and Service Optimization by the Network

5x.2.1	Description

With the increasing adoption of immersive, sensing, and other resource-intensive applications, the battery consumption rate of User Equipment (UE) is expected to rise exponentially. This heightened demand for power will inevitably shorten the battery life within each charging cycle. Moreover, as UEs are expected to remain continuously connected and available, the challenge of balancing service quality with battery availability becomes even more critical.

To enable comprehensive energy efficiency improvements in the 6G system, striking a trade-off between service delivery and battery preservation is essential. One potential solution involves establishing a Service Level Agreement (SLA) between the UE and the network. Through this SLA, the UE can grant permission for the network to actively participate in extending its battery life. This is achieved by enabling the transfer of battery consumption data to the network.

By collecting and monitoring detailed battery usage statistics, the network can analyse consumption patterns and customize its services accordingly. Based on the agreed SLA, the network can optimize resource allocation, service quality, and power management strategies to enhance the battery life of the UE. This approach not only ensures and promotes energy efficiency but also ensures uninterrupted service availability across the 6G ecosystem. This aligns with the system's sustainability goals.

5.2.2	Potential New Requirements needed to support the use case

[PR 5.x.6.1] Procedures to support collection and monitoring of detailed battery usage statistics from UE to the Networks.