3GPP TSG SA WG 1 Meeting #109	S1-250413

Athens, Greece, 17-21 February 2025	(revision of S1-250300)

Source:	Novamint

pCR Title:	Pseudo-CR on consolidation of TR22887

Draft Spec:	3GPP TR 22.887 v1.0.0

Agenda item:	7.3.1

Document for:	Approval

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This contribution proposes a final consolidation for TR.22887

1. Introduction

The consolidation of the potential requirements of TR 22.887 is to be concluded

2. Reason for Change

To provide consolidation of potential requirements for TR 22.887 for the sections & topics which have not been consolidated

- Emergency communications and mission critical services using satellite access including PWS

- multi-orbit satellite access

- broadcast services with satellite access

- charging aspects

- others aspects

The consolidation proposed can be done as following

Use cases on emergency communications and mission critical services using satellite access

| 5.3 Use case on Resilient Satellite Communication for Public Safety | [PR 5.3.6-001] Subject to operator’s policy, the 5G network with satellite access shall be able to maintain the connection to an available core network with the same service level agreement (SLA) for resilient satellite communication. NOTE: The resilient communication can be achieved through a single satellite or multiple satellites over different types of orbits. | General requirement |
| --- | --- | --- |
| 5.9 Use case on Public Warning Service | The following requirement could be added to TS 22.268 [15]: [PR 5.9.6-001] If a 5G System provides satellite access, the PWS shall be able to broadcast geo-targeted Warning Notifications via Non-Terrestrial Networks. | Already supported (CR changed this in Rel-19) |
| 5.10 Use case on Mission Critical Services using Satellite Access with Mobile Base Station Relay | [PR 5.10.6-001] A 5G system with satellite access, shall support delivery of a cached content from a content caching application via a broadcast/multicast service for mission critical services. [PR 5.10.6-002] A 5G system with satellite access shall support configurations of content caching applications in the network (e.g., access network, core network), that provide content close to the UE for mission critical services. [PR 5.10.6-003] A 5G network with satellite access shall be able to support multicast services via Mobile Base Station Relays using satellite access. [PR 5.10.6-004] A 5G system with satellite access shall be able to support C2 communication of UAV operation with required QoS via a mobile base station relay using satellite access . NOTE: QoS is end-to-end QoS, with respect to KPIs for command and control as defined in table 7.2-1 TS 22.125. + KPIs | Content caching TBD MBRS TBD KPIs |
| 5.11 Use case on emergency communication using satellite access | [PR 5.11.6-001] Subject to the regulatory requirements and operator’s policy, the 5G system should be able to support means to provide smaller granularity location information (compared with original cell information in serving satellite) of emergency communication user to the trusted 3rd party when the user triggers the emergency communication via satellite access. | General |

Use cases on Multi-orbit

| 5.4 Use case on service continuity through multi-orbit satellite access | [PR 5.4.6-001] Subject to regulatory requirements and operator’s policy, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able, if applicable, to support service continuity and provide suitable QoS control when the UE communication path moves between satellites in different orbit types with different characteristics.. NOTE: Service continuity across different orbits might not always be possible/applicable depending on the service characteristics (e.g., service continuity for a low-latency service is not applicable across LEO and GEO orbits). [PR 5.4.6-002] A 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support charging mechanisms for communication services for the access based on the different orbit types with different characteristics. | Service continuity Charging |
| --- | --- | --- |
| 5.5 Use case on support for Mobile base station relays (MBSRs) through multi-orbit satellite networks | [PR 5.5.6-001] Subject to operator’s policy, the 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support mobile base station relays to access different orbit types with different characteristics considering e.g., availability of the coverage, required QoS. | MBRS |
| 5.6 Use case on switching between multi-orbits satellite networks in remote area applications | [PR 5.6.6-001] A 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support minimum service interruption when a relay UE is switching connection between different orbit types with different characteristics. (e.g., GEO, MEO or LEO) of the same network. NOTE: Relay UE as specified in TS 22.261 [2] [PR 5.6.6-002] Subject to operator’s policy, the 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to switch a relay UE connection between different orbit types with different characteristics considering e.g. QoS, availability of the coverage. | Service continuity Which orbit to use |
| 5.7 Use case on assisting vehicular communications via multi-orbits satellite access | [PR 5.7.6-001] Subject to regulatory requirement and operator’s policy, a 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support mechanisms to allow a UE connecting to one satellite with a given characteristics considering e.g., QoS, satellite availability. [PR 5.7.6-002] Subject to regulatory requirement and operator’s policy, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall minimize the service interruption when UE communication connection moves between different orbit types of satellite access networks belonging to the same PLMN. [PR 5.7.6-003] A 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to collect charging information for a UE served by satellites in different orbit types with different characteristics of the satellite access network. | Which orbit to use Service Continuity Charging |
| 5.8 Use case on traffic over different orbit satellites | [PR 5.8.6-001] The 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support a mechanism to satisfy the QoS when user data traffic moves across satellite orbit types with different characteristics.. [PR 5.8.6-002] The 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to collect charging information related to traffic of user data when user data traffic moves across satellite orbit types with different characteristics. | QoS Charging |
| 5.12 Use case on supporting different services with multi-orbit satellites | [PR 5.12.6-001] Based on regulatory requirements and operators’ policy, the 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support Service Hosting Environment (e.g., edge application server) on satellites with different satellite orbit types with different characteristics.. [PR 5.12.6-002] Based on regulatory requirements, operators’ policy and agreement with 3rd party, the 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support a mechanism to provide most suitable service hosting environment across different satellite orbit types with different characteristics.. NOTE: One example is an operator can choose the most suitable service hosting environment on-board of satellites, based on the topology of satellites as they are moving on the orbits. | Service Hosting Service Hosting |
| 5.13 Use case on wide-area disaster control support using multi-orbit satellite networks | [PR 5.13.6-001] Subject to regulatory requirements and operator policies, the 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support switching between satellite orbit types with different characteristics depending on the latency requirements. NOTE: The switching between satellites of a multi-orbit satellite network can be triggered by latency requirements. | Which orbit to use |
| 5.14 Use case on broadband services through multi-orbit satellite access | [PR 5.14.6-001] Subject to regulatory requirements and operator’s policy, the 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support a UE to connect with NGSO satellite access for data transmission, but be served via a GSO satellite of the same network before and after the data transmission. | Which orbit to use |
| 5.15 Use case on multi-orbit access for cross-hybrid service | [PR 5.15.6-001] A 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be capable of selecting communication paths across different satellites with given characteristics as well as terrestrial access, based on QoS requirement, access capabilities and availability to support effective management of capacity and quality, aligned with the anticipated level of service. | Which orbit to use |
| 5.16 Use case on using GEO for initial network entry and service initialization | [PR 5.16.6-001] A 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support establishing the initial UE connection to the network through satellites with a given characteristics and then having the UE data traffic using satellite with different characteristics. [PR 5.16.6-002] A 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall enable the UE to select the relevant satellite with a given characteristics for the initial connection to the network. | Inform about Multi orbit? Which orbit to use |
| 5.17 Use case on resource and energy efficiency in a multi-orbit Satellite Access System | [PR 5.17.6-001] A 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to improve the network’s energy efficiency, by e.g., switching different power saving modes considering user density and data demand. [PR 5.17.6-002] A 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall select satellite access type for each UE to optimize the resource usage of the system, by e.g., considering connectivity state of the UE and data demand. | General? Which orbit to use |
| 5.18 Use case on using GEO to land data in a desirable geographical location | [PR 5.18.6-001] A 5G system with satellite access shall be able to support a mechanism for the network to deliver specific data traffic of a UE using preferred satellite ground station location. NOTE: The link between satellite and satellite ground station location is out of 3GPP scope Editor's Note: it is FFS how it is related to multi-orbit. | General requirement – other aspects |
| 5.19 Use case on using GEO for sleep/power management | [PR 5.19.6-001] Subject to regulatory requirement and operator’s policy, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support a mechanism for the network to inform a UE when coverage is available with which satellite orbit types and characteristics.. | Inform about Multi orbit |
| 5.20 Use case on multi-orbit satellite connectivity to mobile base station relay (MBSR) | [PR 5.20.6-001] The 5G system with satellite access supporting simultaneous connectivity over multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to take into account the respective capabilities (e.g., latency, data rate) and availability of the satellite accesses (e.g., over GEO, MEO, LEO) to provision required aggregated QoS for traffic relayed via a mobile base station relay. [PR 5.20.6-002] The 5G system with satellite access supporting simultaneous connectivity over multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to collect and distinguish charging information related to the different satellite access traffic carried via a mobile base station relay. Editor’s note: whether the use of simultaneous connectivity to multi orbit satellite for MBSR is within the scope of the FS_5GSAT_Ph4 is FFS | MBRS Charging |
| 5.21 Use case on combined terrestrial and multi-orbit satellite access for connecting MBSR and 5GC | [PR 5.21.6-001] The 5G system shall support connectivity between a mobile [PR 5.21.6-001] The 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support connectivity between a mobile base station relay (MBSR) and the 5G Core through simultaneously using terrestrial and at least two multi orbit satellite accesses taking into account the respective capabilities (e.g., latency, data rate) and availability of the different satellite access (e.g., over GEO, MEO, LEO) to map the traffic with the aggregated QoS required at the MBS. [PR 5.21.6-002] The 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to collect and distinguish charging information per radio access path related to traffic between MBSR and 5GC that is simultaneously going over terrestrial radio access path and multiple satellite access paths. Editor’s note: whether the use of simultaneous connectivity to multi orbit satellite for MBSR is within the scope of the FS_5GSAT_Ph4 is FFS | TBD/MBRS Charging |
| 5.22 Use case on aircraft communication services in Joint TN/NTN and multi-orbit deployments | [PR 5.22.6-001] Subject to regulatory requirements and operator’s policies, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall maintain user experience or QoE for a UE (e.g., Relay or CPE) mounted and integrated in a high-speed platform. [PR 5.22.6-002] Subject to regulatory requirements and operator’s policies, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support minimum service interruption when the communication path of a UE (e.g., Relay or CPE) mounted and integrated in a high-speed platform is moving between terrestrial access and satellite access networks or between satellites networks potentially supporting different orbit types with different characteristics. [PR 5.22.6-003] Subject to regulatory requirements and operator’s policies, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support service continuity with no interruption when the communication path of a UE (e.g., Relay or CPE) mounted and integrated in a high-speed platform is moving between satellites of the same network operating in any orbit type with given characteristics). [PR 5.22.6-004] Subject to regulatory requirements and operator’s policies, a 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to provide high throughput performances to the UE (e.g., Relay or CPE) mounted and integrated in a high-speed platform ensuring consistent QoE to the end users. +KPIs | QOE Service continuity Service continuity QoE/QoS KPIs |
| 5.23 Use case on Connectivity served by multi satellite operators | [PR 5.23..6-001] Subject to regulatory requirements and operator’s policy, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to provide the UE with the necessary information to acquire and access the suitable satellite among those available. NOTE 1: The information provided to the UE can include satellites information operated by roaming partners. NOTE 2: Acquisition of satellites may be required prior to the initial access procedure, including, e.g. the determination of the antenna pointing direction. In this use case, network selection procedures follow the existing procedures. | General Multi -orbit – inform the UE |

Use Case on Broadcast services with satellite access for unregistered UEs

| 5.24 Use Case on Broadcast services with satellite access for unregistered UEs | [PR 5.24..6-001] Subject to regulatory requirements and operator’s policy, a 5G System supporting satellite access and Broadcast Services shall be able to deliver Media Broadcast to a UE which is not registered. NOTE: Subject to an agreement (SLA) between the MNO and the SNO the above requirement can be applied | Broadcast services with satellite access |
| --- | --- | --- |

Consolidation proposed

- PWS with satellite access

| PR # | Potential Requirement | Potential CPRs |
| --- | --- | --- |
| [PR 5.9.6-001] | If a 5G System provides satellite access, the PWS shall be able to broadcast geo-targeted Warning Notifications via a satellite access network | CPR 6.4-1 If a 5G System provides satellite access, the PWS shall be able to broadcast geo-targeted Warning Notifications via a satellite access network This is already supported in Rel-19 in TS 22.268 – no normative work to be conducted |

- MBRS

| PR # | Potential Requirement | Potential CPRs |
| --- | --- | --- |
| [PR 5.10.6-003] | A 5G network with satellite access shall be able to support multicast services via Mobile Base Station Relays using satellite access. | CPR A 5G network with satellite access shall be able to support multicast services via Mobile Base Station Relays using satellite access. |
| [PR 5.10.6-004] | A 5G system with satellite access shall be able to support C2 communication of UAV operation with required QoS via a mobile base station relay using satellite access . NOTE: QoS is end-to-end QoS, with respect to KPIs for command and control as defined in table 7.2-1 TS 22.125. | CPR A 5G system with satellite access shall be able to support C2 communication of UAV operation with required QoS via a mobile base station relay using satellite access . NOTE: QoS is end-to-end QoS, with respect to KPIs for command and control as defined in table 7.2-1 TS 22.125. |
| [PR 5.5.6-001] | Subject to operator’s policy, the 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support mobile base station relays to access different orbit types with different characteristics considering e.g., availability of the coverage, required QoS. | CPR Subject to operator’s policy, the 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support mobile base station relays to access different orbit types with different characteristics considering e.g., availability of the coverage, required QoS. |
| [PR 5.20.6-001] | The 5G system with satellite access supporting simultaneous connectivity over multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to take into account the respective capabilities (e.g., latency, data rate) and availability of the satellite accesses (e.g., over GEO, MEO, LEO) to provision required aggregated QoS for traffic relayed via a mobile base station relay. | CPR The 5G system with satellite access supporting simultaneous connectivity over multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to take into account the respective capabilities (e.g., latency, data rate) and availability of the satellite accesses (e.g., over GEO, MEO, LEO) to provision required aggregated QoS for traffic relayed via a mobile base station relay. |
| [PR 5.21.6-001] | The 5G system shall support connectivity between a mobile [PR 5.21.6-001] The 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support connectivity between a mobile base station relay (MBSR) and the 5G Core through simultaneously using terrestrial and at least two multi orbit satellite accesses taking into account the respective capabilities (e.g., latency, data rate) and availability of the different satellite access (e.g., over GEO, MEO, LEO) to map the traffic with the aggregated QoS required at the MBRS. | CPR The 5G system shall support connectivity between a mobile [PR 5.21.6-001] The 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support connectivity between a mobile base station relay (MBSR) and the 5G Core through simultaneously using terrestrial and at least two multi orbit satellite accesses taking into account the respective capabilities (e.g., latency, data rate) and availability of the different satellite access (e.g., over GEO, MEO, LEO) to map the traffic with the aggregated QoS required at the MBRS. |

- Charging

| PR # | Potential Requirement | Potential CPRs |
| --- | --- | --- |
| [PR 5.4.6-002] | A 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support charging mechanisms for communication services for the access based on the different orbit types with different characteristics. | CPR 6.6-1 A 5G system with satellite access supporting multiple satellite orbits types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to collect and distinguish charging information related to user data traffic or mobile base station relay traffic over the different satellite access types. |
| [PR 5.7.6-003] | A 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to collect charging information for a UE served by satellites in different orbit types with different characteristics of the satellite access network. | CPR 6.6-1 |
| [PR 5.8.6-002] | The 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to collect charging information related to traffic of user data when user data traffic moves across satellite orbit types with different characteristics.. | CPR 6.6-1 |
| [PR 5.20.6-002] | The 5G system with satellite access supporting simultaneous connectivity over multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to collect and distinguish charging information related to the different satellite access traffic carried via a mobile base station relay. | CPR 6.6-1 |
| [PR 5.21.6-002] | The 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to collect and distinguish charging information per radio access path related to traffic between MBSR and 5GC that is simultaneously going over terrestrial radio access path and multiple satellite access paths. | CPR 6.6-1 |

- Service Continuity

| PR # | Potential Requirement | Potential CPRs |
| --- | --- | --- |
| [PR 5.4.6-001] | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able, if applicable, to support service continuity and provide suitable QoS control when the UE communication path moves between satellites in different orbit types with different characteristics.. NOTE: Service continuity across different orbits might not always be possible/applicable depending on the service characteristics (e.g., service continuity for a low-latency service is not applicable across LEO and GEO orbits). | CPR 6.5.2-2 |
| [PR 5.6.6-001] | A 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support minimum service interruption when a relay UE is switching connection between different orbit types with different characteristics. (e.g., GEO, MEO or LEO) of the same network. NOTE: Relay UE as specified in TS 22.261 [2] | CPR 6.5.2-2 |
| [PR 5.7.6-002] | Subject to regulatory requirement and operator’s policy, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall minimize the service interruption when UE communication connection moves between different orbit types of satellite access networks belonging to the same PLMN. | CPR 6.5.2-1 |
| [PR 5.22.6-002] | Subject to regulatory requirements and operator’s policies, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support minimum service interruption when the communication path of a UE (e.g., Relay or CPE) mounted and integrated in a high-speed platform is moving between terrestrial access and satellite access networks or between satellites networks potentially supporting different orbit types with different characteristics. | CPR 6.5.2-2 Subject to regulatory requirements and operator’s policies, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support minimum service interruption when the communication path of a UE (e.g., Relay or CPE) is moving between terrestrial access and satellite access networks or between satellites networks potentially supporting different orbit types with different characteristics. |
| [PR 5.22.6-003] | Subject to regulatory requirements and operator’s policies, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support service continuity with no interruption when the communication path of a UE (e.g., Relay or CPE) mounted and integrated in a high-speed platform is moving between satellites of the same network operating in any orbit type with given characteristics. | CPR 6.5.2-1 Subject to regulatory requirements and operator’s policies, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support service continuity with no interruption when the communication path of a UE (e.g., Relay or CPE) is moving between satellites of the same network operating in any orbit type with given characteristics. |

- Multi orbit

| PR # | Potential Requirement | Potential CPRs |
| --- | --- | --- |
| [PR 5.17.6-001] | A 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to improve the network’s energy efficiency, by e.g., switching different power saving modes considering user density and data demand. | CPR 6.5.1-1 A 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to improve the network’s energy efficiency, by e.g., switching different power saving modes considering user density and data demand. |
| [PR 5.17.6-002] | A 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall select satellite access type for each UE to optimize the resource usage of the system, by e.g., considering connectivity state of the UE and data demand. | CPR 6.5.1-2 |
| [PR 5.23..6-001] | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to provide the UE with the necessary information to acquire and access the suitable satellite among those available. NOTE 1: The information provided to the UE can include satellites information operated by roaming partners. NOTE 2: Acquisition of satellites may be required prior to the initial access procedure, including, e.g. the determination of the antenna pointing direction. In this use case, network selection procedures follow the existing procedures. | CPR 6.5.1-2 Subject to regulatory requirements and operator’s policy, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to provide the UE with the necessary information to acquire and access the suitable satellite among those available. NOTE 1: The information provided to the UE can include satellites information operated by roaming partners. NOTE 2: Acquisition of satellites may be required prior to the initial access procedure, including, e.g. the determination of the antenna pointing direction. In this use case, network selection procedures follow the existing procedures. |
| [PR 5.19.6-001] | Subject to regulatory requirement and operator’s policy, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support a mechanism for the network to inform a UE when coverage is available with which satellite orbit types and characteristics.. | CPR 6.5.1-2 |
| [PR 5.6.6-002] | Subject to operator’s policy, the 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to switch a relay UE connection between different orbit types with different characteristics considering e.g. QoS, availability of the coverage. | CPR 6.5.1-3 |
| [PR 5.7.6-001] | Subject to regulatory requirement and operator’s policy, a 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support mechanisms to allow a UE connecting to one satellite with a given characteristics considering e.g., QoS, satellite availability. | CPR 6.5.1-2 |
| [PR 5.13.6-001] | Subject to regulatory requirements and operator policies, the 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support switching between satellite orbit types with different characteristics depending on the latency requirements. NOTE: The switching between satellites of a multi-orbit satellite network can be triggered by latency requirements. | CPR 6.5.1-3 Subject to regulatory requirements and operator policies, the 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support switching between satellites with orbit types with different characteristics depending on the latency requirements or QoS. NOTE: The switching between satellites of a multi-orbit satellite network can be triggered by latency requirements. |
| [PR 5.14.6-001] | Subject to regulatory requirements and operator’s policy, the 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support a UE to connect with NGSO satellite access for data transmission, but be served via a GSO satellite of the same network before and after the data transmission. | CPR 6.5.1-4 |
| [PR 5.15.6-001] | A 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be capable of selecting communication paths across different satellites with given characteristics as well as terrestrial access, based on QoS requirement, access capabilities and availability to support effective management of capacity and quality, aligned with the anticipated level of service. | CPR 6.5.1-3 |
| [PR 5.16.6-001] | A 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support establishing the initial UE connection to the network through satellites with a given characteristics and then having the UE data traffic using satellite with different characteristics. | CPR 6.5.1-4 A 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support establishing the initial UE connection to the network through satellites with a given characteristics and then having the UE data traffic using satellite with different characteristics. |
| [PR 5.16.6-002] | A 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall enable the UE to select the relevant satellite with a given characteristics for the initial connection to the network. | CPR 6.5.1-2 |
| [PR 5.12.6-002] | Based on regulatory requirements, operators’ policy and agreement with 3rd party, the 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support a mechanism to provide most suitable service hosting environment across different satellite orbit types with different characteristics. NOTE: One example is an operator can choose the most suitable service hosting environment on-board of satellites, based on the topology of satellites as they are moving on the orbits. | CPR 6.5.1-5 |

Remains to be treated

| [PR 5.8.6-001] | The 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support a mechanism to satisfy the QoS when user data traffic moves across satellite orbit types with different characteristics.. | CPR The 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support a mechanism to satisfy the QoS when user data traffic moves across satellites with orbit types with different characteristics.. |
| --- | --- | --- |
| [PR 5.12.6-001] | Based on regulatory requirements and operators’ policy, the 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support Service Hosting Environment (e.g., edge application server) on satellites with different satellite orbit types with different characteristics.. | CPR Based on regulatory requirements and operators’ policy, the 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support Service Hosting Environment (e.g., edge application server) on satellites with orbit types with different characteristics. |
| [PR 5.22.6-001] | Subject to regulatory requirements and operator’s policies, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall maintain user experience or QoE for a UE (e.g., Relay or CPE) mounted and integrated in a high-speed platform | CPR as below |
| [PR 5.22.6-004] | Subject to regulatory requirements and operator’s policies, a 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to provide high throughput performances to the UE (e.g., Relay or CPE) mounted and integrated in a high-speed platform ensuring consistent QoE to the end users | CPR Subject to regulatory requirements and operator’s policies, a 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to provide high throughput performances to the UE (e.g., Relay or CPE) mounted and integrated in a high-speed platform ensuring consistent QoE to the end users |

- Broadcast Services with satellite access for unregistered UEs

| [PR 5.24..6-001] | Subject to regulatory requirements and operator’s policy, a 5G System supporting satellite access and Broadcast Services shall be able to deliver Media Broadcast to a UE which is not registered. NOTE: Subject to an agreement (SLA) between the MNO and the SNO the above requirement can be applied | CPR Subject to regulatory requirements and operator’s policy, a 5G System supporting satellite access and Broadcast Services shall be able to deliver Media Broadcast to a UE which is not registered. NOTE: Subject to an agreement (SLA) between the MNO and the SNO the above requirement can be applied |
| --- | --- | --- |

- Other aspects

| [PR 5.18.6-001] | A 5G system with satellite access shall be able to support a mechanism for the network to deliver specific data traffic of a UE using preferred satellite ground station location. NOTE: The link between satellite and satellite ground station location is out of 3GPP scope | CPR A 5G system with satellite access shall be able to support a mechanism for the network to deliver specific data traffic of a UE using preferred satellite ground station location. |
| --- | --- | --- |
| [PR 5.3.6-001] | Subject to operator’s policy, the 5G network with satellite access shall be able to maintain the connection to an available core network with the same service level agreement (SLA) for resilient satellite communication. NOTE: The resilient communication can be achieved through a single satellite or multiple satellites over different types of orbits. | CPR Subject to operator’s policy, the 5G network with satellite access shall be able to maintain the connection to an available core network with the same service level agreement (SLA) for resilient satellite communication. NOTE: The resilient communication can be achieved through a single satellite or multiple satellites in different orbit types with different characteristics |

3. Conclusions

None

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.887 v1.0.0.

* * * First Change * * * *

# 6	Consolidated requirements

## 6.1	Introduction

The following requirements represent a consolidation of the various potential requirements captured in the above use cases related to a 5G system with satellite access.

The consolidation is organised by topics as following:

- IMS voice calls using GEO satellite access

- Resilient notification

- emergency communications and mission critical services using satellite access

- multi-orbit

- charging aspects

- others (if needed)

## 6.2	IMS voice calls using GEO satellite access

### 6.2.1	Consolidated functional requirements

The potential requirements corresponding to the support of IMS voice calls using GEO satellite access are listed in the table below.

Table 6.2.1-1 – Consolidated Requirements for IMS voice calls using GEO satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.2.1-1 | The 5G system with GEO satellite access shall be able to support IMS voice communication as defined in TS 22.228 [3]. | [PR 5.1.6-001] |  |
| CPR 6.2.1-2 | The 5G system with GEO satellite access shall be able to provide mechanisms to optimize IMS voice (e.g., call setup, transmission overhead) and support a codec for the transfer of the voice considering the transmission data rate, latency and packet size. | [PR 5.1.6-002] [PR 5.1.6-003] |  |

### 6.2.2	Consolidated potential KPIs

Table 6.2.2-1	Consolidated Performance Requirements for IMS voice call using GEO satellite access

| Scenario | UE type | Transmission data rate |  | Call setup time NOTE 1 |
| --- | --- | --- | --- | --- |
|  |  | UL | DL |  |
| IMS voice call using GEO | Handheld | [1-3] kbit/s | [1-3] kbit/s | ≤30 s NOTE 2 |
| NOTE 1: call set up time refers to [4]; NOTE 2: 30s is the upper bound that is derived based on the user’s patience suggestions (30s) in [12]; |  |  |  |  |

## 6.3	Resilient Notification

The potential requirements corresponding to the support of resilient notification are listed in the table below.

Table 6.3-1 – Consolidated Requirements for resilient notification

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.3.1-1 | Subject to operator’s policies, a 5G System with satellite access shall be able to support Resilient Notification Service to notify users, with valid subscription to the notification service, about a missed mobile terminated service when the user is unreachable via satellite access. NOTE: The Resilient Notification service can provide the user with service information e.g. caller’s information and service type. | [PR 5.2.6-001] |  |

## 6.4	Emergency communications and mission critical services using satellite access

## 6.4.2	Other emergency communications and mission critical services using satellite access

The potential requirements corresponding to the support of emergency communications and mission critical services using satellite access are listed in the table below.

Table 6.4-2 – Consolidated Requirements for emergency communications and mission critical services using satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

### 6.4.3.	Consolidated potential KPIs

Table 6.4-3 – Consolidated Performance Requirements for Video data over satellite access

| Scenario | Type of data | Function | Experienced data rate | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | End-to-end Latency | Max UE ground speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UAV | Video data over satellite access (used for Search and rescue and aid to UAV non-VLOS C2) | UAV terminated C2 | 300 kbit/s | TBD | TBD | TBD | 40ms | 60km/h | UAV mounted |
|  |  | UAV originated C2 | 6 Mbit/s | TBD | TBD | TBD | 140ms |  |  |
| Note 1: Area capacity is averaged over a satellite beam |  |  |  |  |  |  |  |  |  |

## 6.5	Multi-orbit

## 6.5.1	General aspects

The potential requirements corresponding to the support of multi-orbit general aspects are listed in the table below.

Table 6.5.1-1 – Consolidated Requirements for multi-orbit general aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.5.1-1 | A 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to improve the network’s energy efficiency, by e.g., switching different power saving modes considering user density and data demand. | [PR 5.17.6-001] |  |
| CPR 6.5.1-2 | CPR 6.5.1-2 Subject to regulatory requirements and operator’s policy, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to provide the UE with the necessary information to acquire and access the suitable satellite among those available. NOTE 1: The information provided to the UE can include satellites information operated by roaming partners. NOTE 2: Acquisition of satellites may be required prior to the initial access procedure, including, e.g. the determination of the antenna pointing direction. In this use case, network selection procedures follow the existing procedures. | [PR 5.17.6-002] [PR 5.23..6-001] [PR 5.19.6-001] [PR 5.7.6-001] [PR 5.16.6-002] |  |
| CPR 6.5.1-3 | Subject to regulatory requirements and operator policies, the 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support switching between satellites with orbit types with different characteristics depending on the latency requirements or QoS. NOTE: The switching between satellites of a multi-orbit satellite network can be triggered by latency requirements. | [[PR 5.6.6-002] [PR 5.13.6-001] [PR 5.15.6-001] |  |
| CPR 6.5.1-4 | A 5G network with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to support establishing the initial UE connection to the network through satellites with a given characteristics and then having the UE data traffic using satellite with different characteristics. | [PR 5.14.6-001] [PR 5.16.6-001] |  |

## 6.5.2	Service continuity aspects

The potential requirements corresponding to the support of service continuity aspects related to multi-orbit are listed in the table below.

Table 6.5.2-1 – Consolidated Requirements for service continuity related to multi-orbit

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.5.2-1 | Subject to regulatory requirements and operator’s policies, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support service continuity with no interruption when the communication path of a UE (e.g., Relay or CPE) is moving between satellites of the same network operating in any orbit type with given characteristics. | [PR 5.7.6-002] [PR 5.22.6-003] |  |
| CPR 6.5.2-2 | Subject to regulatory requirements and operator’s policies, a 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall support minimum service interruption when the communication path of a UE (e.g., Relay or CPE) is moving between terrestrial access and satellite access networks or between satellites networks potentially supporting different orbit types with different characteristics. | [PR 5.4.6-001] [PR 5.6.6-001] [PR 5.22.6-002] |  |

### 6.5.3.	Consolidated potential KPIs

It is proposed to modify the following KPIs defined in the table 7.4.2-1 (performance requirements for satellite access) in TS 22.261 [2] as listed in the table 6.4.3-1

Table 6.5.3-1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Airplanes connectivity (note 4) | [1] Gbit/s per plane | [300] Mbit/s per plane | N/A | N/A | N/A | 40% | Up to [1500] km/h | Airplane mounted |
| Note 4: - Required experienced peak data rate corresponding to the aggregated passenger traffic at aircraft level - Based on an assumption of 450 seats, average take rate of 75% (free model) and load factor of 85% - Assumption of 3:1 Downlink / Uplink ratio, anticipating future usages - The Downlink & Uplink throughput can be achieved using one or multiple links |  |  |  |  |  |  |  |  |

## 6.6	Charging aspects

The potential requirements corresponding to the charging aspects are listed in the table below.

Table 6.6-1 – Consolidated Requirements for charging aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.6-1 | A 5G system with satellite access supporting multiple satellite orbits types with different characteristics (e.g.; altitude, orbital characteristics, satellite capabilities) shall be able to collect and distinguish charging information related to user data traffic or mobile base station relay traffic over the different satellite access types. | [PR 5.4.6-002] [PR 5.7.6-003] [PR 5.8.6-002] [PR 5.20.6-002] [PR 5.21.6-002] |  |

## 6.7	Broadcast Services with satellite access for unregistered UEs

The potential requirements corresponding to the support of Broadcast Services with satellite access for unregistered UEs are listed in the table below.

Table 6.7-1 – Consolidated Requirements for Broadcast Services with satellite access for unregistered UEs

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.7.1-1 | Subject to regulatory requirements and operator’s policy, a 5G System supporting satellite access and Broadcast Services shall be able to deliver Media Broadcast to a UE which is not registered. NOTE: Subject to an agreement (SLA) between the MNO and the SNO the above requirement can be applied | [PR 5.24..6-001] |  |

## 6.8	Other aspects for satellite access

The potential requirements corresponding to the support of enhancements of other aspects of satellite access are listed in the table below.

Table 6.8-1 – Consolidated Requirements for other aspects of satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.8.1-1 | A 5G system with satellite access shall be able to support a mechanism for the network to deliver specific data traffic of a UE using preferred satellite ground station location. | [PR 5.18.6-001] |  |
| CPR 6.8.1-2 | Subject to operator’s policy, the 5G network with satellite access shall be able to maintain the connection to an available core network with the same service level agreement (SLA) for resilient satellite communication. NOTE: The resilient communication can be achieved through a single satellite or multiple satellites in different orbit types with different characteristics | [PR 5.3.6-001] |  |

# 7	Conclusions and recommendations

This technical report identifies gaps, including potential requirements and KPIs, for enhancing existing use cases and defining new requirements for emerging use cases related to 5G systems with satellite access.

The resulting service requirements have been already consolidated in clause 6.

It is recommended to consider the consolidated requirements identified in this TR as the baseline for the subsequent normative work.