3GPP TSG-SA1 Meeting #109	S1-250416

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0832 | rev | 1 | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of normative input based on FS_EnergyServ_Ph2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergyServ_Ph2-REQ |  |  |  |  |  | Date: |  |  | 2025-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the conclusions and recommendations of FS_EnergyServ_Ph2 documented in TR22.883, new considerations and normative requirements & KPIs are proposed to be added to TS 22.261. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In sub-clause 6.15a.1, generic considerations are added to clarify the attribution & accuracy of energy-related information, and other clarifications. - In sub-clause 6.15a.2, normative requirements corresponding to the CPRs of TR22.883 cl 6.1.2/3/4 are added. - In sub-clause 6.15a.5, normative requirements corresponding to the CPRs of TR22.883 cl 6.1.1 are added - In clause 7, a new sub-clause is added to add KPIs for energy-related characteristics Editor’s Notes are for DRAFT version only and will be removed from the submitted CR Rev1: - Revisions agreed during drafting sessions - Incorporation of CPRs agreed during drafting sessions - Removal of ENs placeholder for CPRs - Addition of EN on considerations related to net energy savings - Addition of new definitions and abbreviations from TR 22.883 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FS_EnergyServ_Ph2 study result in no normative work being provided on time by SA1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.15a and 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S1-250202 |  |  |  |  |  |  |  |  |

FIRST CHANGE

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G enhanced positioning area: a subset of the 5G positioning service area that is assumed to be provided with additional infrastructure or deploy a particular set of positioning technologies to enhance positioning services.

NOTE 1: 	The enhanced positioning service area represents for example a factory plant, a dense urban area, an area along a road or railway track, a tunnel and covers both indoor and outdoor environments.

5G LAN-type service: a service over the 5G system offering private communication using IP and/or non-, i.e. UEs that are members of the same 5G LAN-VN IP type communications.

5G LAN-virtual network: a virtual network capable of supporting 5G LAN-type service.

5G satellite access network: 5G access network using at least one satellite.

5G positioning service area: a service area where positioning services would solely rely on infrastructures and positioning technologies that can be assumed to be present anywhere where 5G is present (e.g. a country-wide operator-supplied 5G network, GNSS, position/motion sensors).

NOTE 2: 	This includes both indoor and any outdoor environments.

active communication: a UE is in active communication when it has one or more connections established. A UE may have any combination of PS connections (e.g. PDP contexts, active PDN connections).

activity factor: percentage value of the amount of simultaneous active UEs to the total number of UEs where active means the UEs are exchanging data with the network.

Ambient IoT device: An ambient power-enabled Internet of Things device is an IoT device powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g., using a capacitor).

aggregated QoS: QoS requirement(s) that apply to the traffic of a group of UEs.

area traffic capacity: total traffic throughput served per geographic area.

authorised administrator: a user or other entity authorised to partially configure and manage a network node in a CPN (e.g. a PRAS, or eRG) or a PIN element in a PIN.

carbon emission: quantity of equivalent carbon dioxide emitted (e.g. kg of CO2 equivalent).

carbon intensity: quantity of CO2 equivalent emission per unit of final energy consumption for an operational period of use [46]

communication service availability: percentage value of the amount of time the end-to-end communication service is delivered according to a specified QoS, divided by the amount of time the system is expected to deliver the end-to-end service.

NOTE 3:	The end point in "end-to-end" is the communication service interface.

NOTE 4:	The communication service is considered unavailable if it does not meet the pertinent QoS requirements. For example, the communication service is unavailable if a message is not correctly received within a specified time, which is the sum of maximum allowed end-to-end latency and survival time.

Customer Premises Network: a network located within a premise (e.g. a residence, office or shop), which is owned, installed and/or (at least partially) configured by the customer of a public network operator.

direct device connection: the connection between two UEs without any network entity in the middle.

direct network connection: one mode of network connection, where there is no relay UE between a UE and the 5G network.

Disaster Condition: This is the condition that a government decides when to initiate and terminate, e.g. a natural disaster. When this condition applies, users may have the opportunity to mitigate service interruptions and failures.

Disaster Inbound Roamer: A user that (a) cannot get service from the PLMN it would normally be served by, due to failure of service during a Disaster Condition, and (b) is able to register with other PLMNs.

Disaster Roaming: This is the special roaming policy that applies during a Disaster Condition.

DualSteer device: A device supporting traffic steering and switching of user data (for different services) across two 3GPP access networks; it can be a single UE, in case of non-simultaneous data transmission over the two networks, or two separate UEs in case of simultaneous data transmission over the two networks.

end-to-end latency: the time that it takes to transfer a given piece of information from a source to a destination, measured at the communication interface, from the moment it is transmitted by the source to the moment it is successfully received at the destination.

energy availability: the remaining amount of energy (e.g. in kWh) locally available for consumption. For devices, network elements and functions, energy availability may be limited and/or intermittent, in particular when relying on batteries and/or renewable energy sources (e.g. off-grid base stations, satellites etc) or during power grid heavy load or disruptions.

energy capacity: the maximum amount of energy (e.g. in kWh) that can be locally available for consumption (either locally produced and/or stored) by a device or a network element or function.

energy charging rate: a means of determining the energy consumption consequence (use of energy credit) associated with charging events.

energy credit: a quantity of energy credit associated with the subscriber that can be used for credit control by the 5G system.

energy rationing: A situation in which the availability of energy either across the network or at a particular network element or function is limited or reduced.

energy-related characteristics: information which characterize the energy to power the operator’s network in terms of energy consumption, energy supply mix, carbon footprint, energy capacity and availability conditions (e.g. type, value or ratio).

NOTE:	Which energy-related characteristics are relevant depends on the scenario.

energy state: state of a cell, a network element and/or a network function with respect to energy, e.g. (not) energy saving states, which are defined in TS 28.310 [47].

energy supply mix: the combination of the various energy sources (i.e. renewable and not) used to meet energy needs of a device or a network element or function.

evolved Residential Gateway: a gateway between the public operator network (fixed/mobile/cable) and a customer premises network.

holdover: A clock A, previously synchronized/syntonized to another clock B (normally a primary reference or a Master Clock) but whose frequency is determined in part using data acquired while it was synchronized/syntonized to B, is said to be in holdover or in the holdover mode as long as it is within its accuracy requirements.

NOTE 4a:	holdover is defined in [31]

Holdover time: the time period that is available to repair the first priority timing source when it is lost (e.g., when the primary GNSS reference is lost). During this period the synchronization accuracy requirement should be guaranteed, e.g., by means of defining multiple synchronization references.

Hosted Service: a service containing the operator's own application(s) and/or trusted third-party application(s) in the Service Hosting Environment, which can be accessed by the user.

Hosting NG-RAN Operator: the operator that has operational control of a Shared NG-RAN.

NOTE 4b:	Hosting NG-RAN Operator is a Hosting RAN Operator.

Hosting RAN Operator: as defined in 3GPP TS 22.101 [6].

hybrid access: access consisting of multiple different access types combined, such as fixed wireless access and wireline access.

indirect network connection: one mode of network connection, where there is a relay UE between a UE and the 5G network.

Indirect Network Sharing: a type of NG-RAN Sharing in which the communication between the Shared NG-RAN and the Participating Operator’s core network is routed through the Hosting NG-RAN Operator’s core network.

IoT device: a type of UE which is dedicated for a set of specific use cases or services and which is allowed to make use of certain features restricted to this type of UEs.

NOTE 5:	An IoT device may be optimized for the specific needs of services and application being executed (e.g. smart home/city, smart utilities, e-Health and smart wearables). Some IoT devices are not intended for human type communications.

maximum energy consumption: a policy establishing an upper bound on the quantity of energy consumption [47] by the 5G system in a specific period of time, or space, e.g. energy consumption inside a given service area.

maximum energy credit limit: a policy establishing an upper bound on the aggregate quantity of energy consumption by the 5G system to provide services to a specific subscriber, e.g. in kilowatt hours.

NOTE:	The terms maximum energy credit limit is distinct from 'maximum energy consumption' because the credit limit is a total amount of energy consumed, where maximum energy consumption is a limit to the consumption in a given interval of time.

network slice: a set of network functions and corresponding resources necessary to provide the required telecommunication services and network capabilities.

NG-RAN: a radio access network connecting to the 5G core network which uses NR, E-UTRA, or both.

NG-RAN Sharing: the sharing of NG-RAN among a number of operators.

non-public network: a network that is intended for non-public use.

NR: the new 5G radio access technology.

Participating NG-RAN Operator: authorized operator that is using Shared NG-RAN resources provided by a Hosting NG-RAN Operator.

NOTE 5a: Participating NG-RAN Operator is a Participating Operator.

Participating Operator: as defined in 3GPP TS 22.101 [6].

Personal IoT Network: A configured and managed group of at least one UE PIN Element and one or more PIN Element that communicate with each other.

PIN Element: UE or non-3GPP device that can communicate within a PIN.

PIN direct connection: the connection between two PIN Elements without any 3GPP RAN or core network entity in the middle.

NOTE 5A:	A PIN direct connection could internally be relayed by other PIN Elements.

NOTE 5B:	When a PIN direct connection is between two PIN Elements that are UEs this direct connection is typically known as a direct device connection.

PIN Element with Gateway Capability: a UE PIN Element that has the ability to provide connectivity to and from the 5G network for other PIN Elements.

NOTE 5C:	A PIN Element can have both PIN management capability and Gateway Capability.

PIN Element with Management Capability: A PIN Element with capability to manage the PIN.

positioning service availability: percentage value of the amount of time the positioning service is delivering the required position-related data within the performance requirements, divided by the amount of time the system is expected to deliver the positioning service according to the specification in the targeted service area.

proximity-based work task offloading: a relay UE receives data from a remote UE via direct device connection and performs calculation of a work task for the remote UE. The calculation result can be further sent to network server.

positioning service latency: time elapsed between the event that triggers the determination of the position-related data and the availability of the position-related data at the system interface.

Premises Radio Access Station: a base station installed at a customer premises network.

priority service: a service that requires priority treatment based on regional/national or operator policies.

private communication: a communication between two or more UEs belonging to a restricted set of UEs.

private network: an isolated network deployment that does not interact with a public network.

private slice: a dedicated network slice deployment for the sole use by a specific third-party.

ProSe UE-to-UE Relay: a Public Safety ProSe-enabled UE that acts as a relay between two other Public Safety ProSe-enabled UEs.

Ranging: refers to the determination of the distance between two UEs and/or the direction of one UE from the other one via direct device connection.

relative positioning: relative positioning is to estimate position relatively to other network elements or relatively to other UEs.

reliability: in the context of network layer packet transmissions, percentage value of the packets successfully delivered to a given system entity within the time constraint required by the targeted service out of all the packets transmitted.

renewable energy: energy from renewable sources as energy from renewable non-fossil sources, namely wind, solar, aerothermal, geothermal, hydrothermal and ocean energy, hydropower, biomass, landfill gas, sewage treatment plant gas and biogases

NOTE 2:	This definition was taken from [48].

satellite: a space-borne vehicle embarking a bent pipe payload or a regenerative payload telecommunication transmitter, placed into Low-Earth Orbit (LEO) typically at an altitude between 300 km to 2 000 km, Medium-Earth Orbit (MEO) typically at an altitude between 8 000 to 20 000 k m, or Geostationary satellite Earth Orbit (GEO) at 35 786 km altitude.

satellite access: direct connectivity between the UE and the satellite.

satellite NG-RAN: a NG-RAN which uses NR in providing satellite access to UEs.

service area: geographic region where a 3GPP communication service is accessible.

NOTE 6:	The service area can be indoors.

NOTE 7:	For some deployments, e.g. in process industry, the vertical dimension of the service area can be considerable.

service continuity: the uninterrupted user experience of a service that is using an active communication when a UE undergoes an access change without, as far as possible, the user noticing the change.

NOTE 8:	In particular service continuity encompasses the possibility that after a change the user experience is maintained by a different telecommunication service (e.g. tele- or bearer service) than before the change.

NOTE 9:	Examples of access changes include the following. For EPS: CS/PS domain change. For EPS and 5G: radio access change, switching between a direct network connection and an indirect network connection.

Service Hosting Environment: the environment, located inside of 5G network and fully controlled by the operator, where Hosted Services are offered from.

serving satellite: a satellite providing the satellite access to an UE. In the case of NGSO, the serving satellite is always changing due to the nature of the satellite constellation.

Shared NG-RAN: as defined in 3GPP TS 22.101 [6].

NOTE: Shared NG-RAN can be a shared satellite NG-RAN.

Stand-alone Non-Public Network: A non-public network not relying on network functions provided by a PLMN

SNPN Credential Provider: Entity within the 5G system that creates and manages identity information and provides authentication services for those identities for the purpose of accessing a SNPN

NOTE:	The SNPN Credential Provider can also authorize access to a non-public network for a subscriber associated with an identity handled by this SNPN Credential Provider.

S&F Satellite operation: S&F (Store and Forward) Satellite operation is an operation mode of a 5G system with satellite-access where the 5G system can provide some level of service (in storing and forwarding the data) when satellite connectivity is intermittently/temporarily unavailable, e.g. to provide communication service for UEs under satellite coverage without a simultaneous active feeder link connection to the ground segment.

S&F data retention period: it is the data storage validity period for a 5G system with satellite access supporting store and forward operation (e.g. after which undelivered data stored is being discarded).

synchronization threshold: A synchronization threshold can be defined as the maximum tolerable temporal separation of the onset of two stimuli, one of which is presented to one sense and the other to another sense, such that the accompanying sensory objects are perceived as being synchronous.

NOTE 10:	This definition is based on [41].

survival time: the time that an application consuming a communication service may continue without an anticipated message.

Time to First Fix (TTFF): time elapsed between the event triggering for the first time the determination of the position-related data and the availability of the position-related data at the positioning system interface.

Traffic steering: the procedure that selects an access network and transfers traffic over the selected access network. This can apply to traffic of one or multiple services/applications across two 3GPP access networks, including scenarios where all services use the same network connection (no simultaneous data over the two networks) or different services are steered across different networks (with simultaneous data over the two networks).

Traffic switching: the procedure that moves all traffic from one access network to another access network in a way that minimizes service interruption. This can apply to traffic of one or multiple services/applications across two 3GPP access networks, including scenarios where all services use the same network connection (no simultaneous data over the two networks) or different services are moved to different networks (with simultaneous data over the two networks).

UE-Satellite-UE communication: for a 5G system with satellite access, it refers to the communication between UEs under the coverage of one or more serving satellites, using satellite access without the user traffic going through the ground segment.

User Equipment: An equipment that allows a user access to network services via 3GPP and/or non-3GPP accesses.

user experienced data rate: the minimum data rate required to achieve a sufficient quality experience, with the exception of scenario for broadcast like services where the given value is the maximum that is needed.

wireless backhaul: a link which provides an interconnection between 5G network nodes and/or transport network using 5G radio access technology.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5G LAN-VN	5G LAN-Virtual Network

A/S	Actuator/Sensor

CO2	Carbon dioxide

CO2e	Carbon dioxide equivalent

CPN	Customer Premises Network

eFMSS	Enhancement to Flexible Mobile Service Steering

eRG	Evolved Residential Gateway

eV2X	Enhanced V2X

FL	Federated Learning

FMSS	Flexible Mobile Service Steering

GEO	Geostationary satellite Earth Orbit

ICP	Internet Content Provider

ID	Identification

IMU	Inertial Measurement Unit

IOPS	Isolated E-UTRAN Operation for Public Safety

ISL	Inter-Satellite Link

LEO	Low-Earth Orbit

MBS	Metropolitan Beacon System

MCS	Mission Critical Services

MCX	Mission Critical X, with X = PTT or X = Video or X = Data

MEO	Medium-Earth Orbit

MIoT	Massive Internet of Things

MMTEL	Multimedia Telephony

MOCN	Multi-Operator Core Network

MPS	Multimedia Priority Service

MSGin5G	Message Service Within the 5G System

MSTP	Multiple Spanning Tree Protocol

NGSO	Non-Geostationary Satellite Orbit

NPN	Non-Public Network

PIN	Personal IoT Network

PRAS	Premises Radio Access Station

RSTP	Rapid Spanning Tree Protocol

RVAS	Roaming Value-Added Service

SEES	Service Exposure and Enablement Support

SNPN	Stand-alone Non-Public Network

S&F	Store and Forward

SST	Slice/Service Type

TBS	Terrestrial Beacon System

TTFF	Time To First Fix

UAV	Uncrewed Aerial Vehicle

UTC 	Coordinated Universal Time

XR	eXtended Reality

SECOND CHANGE

## 6.15a	Energy Efficiency as a Service Criteria

### 6.15a.1	Description

Climate change and the rising consumption of energy motivate increased energy efficiency. Energy efficiency is a strategic priority for telecom operators around the world.

Energy efficiency as (a) service criteria allows services to be delivered with diverse energy efficiency and energy consumption policies. Such criteria are expected to be subject to regulatory requirements, in particular to keep delivering critical services such as emergency calls, PWS, MPS and MCS to the extent possible. Energy consumption and efficiency information and network energy states can be exposed to third parties and energy consumption can be constrained (e.g. energy rationing).

Energy related information can include ratio of renewable energy and carbon emission information when available. Calculation of energy related information as described in the following requirements can be done by means of averaging or applying a statistical model to a combination of measurements, service configuration, statistics, computations and estimates. The requirements do not imply that some form of 'real time' monitoring, nor exact measurement is required (e.g. at flow level). Instead, they can be met by means of attribution as accurate as possible, e.g. based on data volume and network’s carbon and/or energy KPIs or metrics of the communication service, which could be collected either from the network or from sources external to 3GPP (e.g. energy supply mix provided by electricity grid providers).

The level of accuracy of attribution and associated time granularity can impact what can be done with the energy-related information.

The energy-related information considered in the following requirements only relates to the use of the communication service provided by a telecom operator to its home network subscribers, as a single part of the end-to-end service chain. The following requirements do not apply to the energy-related information of other providers outside of 3GPP scope potentially involved in the same service chain (e.g. video streaming providers).

Editor’s Note: considerations targeting net energy savings are FFS

### 6.15a.2	Energy related information as a service criteria

#### 6.15a.2.1	Description

Energy consumption can be monitored and considered through O&M as part of network operations [47], as well as a service criteria.

For best-effort traffic, that is, without QoS criteria, policies can be defined to limit energy use for services. This is not in conflict with the principle that performance policies will not be traded off for energy efficiency, since best-effort service has no performance guarantees.

Specifically, best-effort traffic can be subject to a policy that limits the maximum energy consumption over time, or further constrained by location (so that the energy consumption limit only applies when used in a specified service area.)

Additionally, policies can be defined with a maximum energy credit limit, e.g. for best-effort services to limit the total amount of energy consumption according to an energy charging rate. These policies expand the options of subscription policies to control energy consumption in the 5G system.

#### 6.15a.2.2	Requirements

Subject to operator’s policy, the 5G system shall support subscription policies that define a maximum energy credit limit for services without QoS criteria.

Subject to operator’s policy, the 5G system shall support a means to associate energy consumption information with charging information based on subscription policies for services without QoS criteria.

Subject to operator’s policy, the 5G system shall support a mechanism to perform energy consumption credit limit control for services without QoS criteria.

NOTE 1: The result of the credit control is not specified by this requirement.

NOTE 2:	Credit control [49] compares against a credit control limit. It is assumed charging events are assigned a corresponding energy consumption and this is compared against a policy of energy credit limit. It is assumed there can be a new policy to limit energy consumption allowed.

Subject to operator’s policy, the 5G system shall support a means to define subscription policies and means to enforce the policy that define a maximum energy consumption (i.e. quantity of energy for a specified period of time) for services without QoS criteria.

NOTE 3:	The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services.

The 5G system shall provide a mechanism to include Energy related information as part of charging information.

Subject to operator policy and agreement with 3rd party, the 5G system shall provide a mechanism to support the selection of an application server based on energy related information associated with a set of application servers.

Subject to user consent and operator policy, 5G system shall be able to provide means to modify a communication service based on energy related information criteria based on subscription policies.

Subject to user consent, operator policy and regulatory requirements, the 5G system shall be able to provide means to operate part or the whole network according to energy consumption requirements, which may be based on subscription policies or requested by an authorized 3rd party.

Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall enable the operator to provide means to degrade service performance (e.g. QoS, bitrate) to meet energy rationing constraints.

NOTE 4: 	This assumes that the degradation in service performance will reduce energy consumption in the 5G network to meet energy rationing constraints.

Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall support – and allow authorized 3rd parties to provision - subscription policies that include alternative (i.e. degraded) service performance (e.g. QoS parameters, maximum bitrate) of services with QoS criteria for energy saving reasons.

NOTE 5:	This requirement implies that the policies could disallow some network energy saving action to be performed for some service with QoS criteria e.g. based on applicability conditions (e.g. slice, application).

Subject to operator’s policy, the 5G system shall be able to support a means for the UE to request network actions targeting per UE energy saving, based on pre-agreed subscription policies.

Subject to operators’ policy, regulatory requirements, 5G network shall provide mechanisms to adjust communication service (e.g. user plane path, suitable Service Hosting Environment, defer background traffic delivery) considering the change of energy supply mix of the network as one of the factors.

NOTE 6:	It is assumed that 5G network can obtain energy supply mix information. How to obtain this information is out of scope of this requirement. The adjustment is not assumed to be real-time. It is up to the operators’ policy to define the time difference between obtaining the change on energy supply mix information and the actual adjustment of the communication service.

NOTE 7:  Degradation of service experience resulting from the adjustment is subject to subscription-based user consent.

Subject to operator’s policy, regulatory requirements and user consent, the 5G system should enable a UE to take action (e.g. defer background traffic, change access type, disable video service etc) based on the energy-related characteristics received from their serving network.

Subject to operator’s policy and regulatory requirements, the 5G network shall be able to trigger charging events corresponding to an impacted UE when degrading performance of services with QoS criteria (e.g. to a lower bitrate) in order to achieve energy saving.

Subject to user consent, operator policy and regulatory requirements, the 5G network shall be able to assist an authorized 3rd party to identify a set of target UEs for whom to adjust the provided application service, considering criteria such as the current and future (e.g. predicted) energy-related characteristics of their serving network.

### 6.15a.3	Support of different energy states

#### 6.15a.3.1	Description

Supporting different energy states is beneficial for verticals and operators to save energy according to different working status of telecommunication equipment and manufacturing.

#### 6.15a.3.2	Requirements

The 5G system shall support different energy states of network elements and network functions.

5G system shall support dynamic changes of energy states of network elements and network functions.

NOTE: This requirement also includes the condition when providing network elements or functions to an authorised 3rd party, the dynamic changes can be based on pre-configured policy (the time of changing energy states, which energy state map to which level of load, etc.)

The 5G system shall support different charging mechanisms based on the different energy states of network elements and network functions.

### 6.15a.4	Monitoring and measurement

#### 6.15a.4.1	Description

Different levels of monitoring and measurement related to energy consumption and efficiency bring more support in energy efficiency and energy saving. In this section, monitoring and measurement related to energy consumption and efficiency include network functions in NPN and also all kinds of NG-RAN deployment scenarios.

#### 6.15a.4.2	Requirements

Subject to operator's policy, the 5G network shall support energy consumption monitoring at per network slice and per subscriber granularity.

NOTE 1:	Energy consumption monitoring as described in the preceding requirement is done by means of averaging or applying a statistical model. The requirement does not imply that some form of 'real time' monitoring is required. The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services.

Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to monitor energy consumption for serving this 3rd party.

NOTE 2: The granularity of energy consumption measurement could vary according to different situations, for example, when several services share a same network slice, etc.

NOTE 3: The energy consumption information can be related to the network resources of network slice, NPNs, etc.

Subject to operator policy and regulatory requirements, the 5G system shall be able to monitor the energy consumption for serving the 3rd party, together with the network performance statistic information for the services provided by that network, related to same time interval e.g. hourly or daily.

NOTE 4: The network performance statistic information could be the data rate, packet delay and packet loss, etc.

### 6.15a.5	Information exposure

#### 6.15a.5.1	Description

Information related to energy consumption and efficiency is not only necessary for network internal optimization, but also will benefit the service adjustment for 3rd party.

The performance requirements related to the exposure of network energy-related characteristics such as energy consumption and CO2e emissions depending on the target use case are documented in clause 7.13.

#### 6.15a.5.2	Requirements

Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to expose information on energy consumption for serving this 3rd party.

NOTE 1: Energy consumption information can include ratio of renewable energy and carbon emission information when available. The reporting period could be set, e.g., on monthly or yearly basis and can vary based on location.

NOTE 2: The energy consumption information can be related to the network resources of network slice, NPNs, etc.

Subject to operator’s policy, agreement with 3rd party and consent by the customer, the 5G system shall be able to expose the network performance statistic information (e.g. the data rate, packet delay and packet loss) together with energy consumption information resulting from service provided to the customer, to the authorized third party, related to the same time interval e.g. hourly or daily.

Subject to operator’s policy, the 5G system shall support a means to expose energy consumption to authorized third parties for services, including energy consumption information related to the condition of energy credit limit (e.g. when the energy consumption is reaching the energy credit limit).

Subject to operator policy, the 5G system shall provide means for the trusted 3rd party, to configure which network performance statistic information (e.g. the data rate, packet delay and packet loss) for the communication service provided to the 3rd party, needs to be exposed along with the information on energy consumption for serving this 3rd party.

Based on operator’s policy and agreement with 3rd party, the 5G system shall be able to expose energy consumption information and prediction on energy consumption of the 5G network per application service to the 3rd party.

Subject to operator’s policy and agreement with 3rd party, the 5G system shall support a mechanism for the 3rd party to provide current or predicted energy consumption information over a specific period of time.

Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall be able to collect, and expose to authorized 3rd parties, the carbon equivalent emissions resulting from the use of the communication service, related to one or more specific UEs of home network subscribers (e.g. fleet of vehicles, IoT devices, company phones etc), over a specific time period (e.g. month etc).

NOTE 3:	This requirement does not apply to the use of the communication service when roaming.

NOTE 4:	Exposing carbon equivalent emissions can be based on data volume and operator’s energy consumption and carbon intensity information. In particular, it is expected such exposed information to be provided on demand, not more granularly than on a per-day basis.

NOTE 5:	The exposed carbon equivalent emissions can further include information to allow to compare subscribers of e.g. the same PLMN, NPN or slice over the same time period. Such information can take the form of e.g. a range (i.e. minimum and maximum) or average of carbon equivalent emissions per subscriber, or the form of a category, class or score of carbon emissions (e.g. A to F, 1 to 5).

Subject to operator’s policy and user consent, the 5G network shall be able to expose to an authorized 3rd party information about network energy consumption and carbon equivalent emissions for a specific service data flow.

NOTE 6:	One of the goals of exchanging Energy Consumption data and CO2e data for a specific individual service data flow is to enable applications to provide an indication to the user during the communication service. The frequency of such indications should be meaningful to the user and is assumed to be configurable, e.g. frequency or updates of thresholds for reporting (e.g. based on e.g. a session, down to once per minute).

Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall be able to expose to an authorized 3rd party the current and expected energy-related characteristics of the network resources used in serving a UE.

NOTE 7:	This information can be used by applications to schedule non time-critical data transfer to or from the UE.

Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall be able to provide UEs with information related to the current energy-related characteristics of their serving network.

NOTE 8:	A subset of target UEs can be selected, for example, based on their location, their energy consumption in the network, their service experience, etc.

NOTE 9:	Future (e.g. predicted) information can refer to the next hour(s) or day(s), to the remaining serving time before shutdown/shortage etc.

### 6.15a.6	Network actions leveraging energy efficiency as a service criteria

#### 6.15a.6.1	Description

This clause addresses requirements to the 5G system that leverage energy-related information (e.g., energy consumption, energy efficiency), amongst others (e.g., network load), as criteria for network internal optimization actions targeting energy savings, within and across operators in a localized (i.e., geographically bound) and/or temporal (i.e., time bound) manner.

One of the strategies to save energy within mobile networks is to shut down some RAN nodes at times of low usage.

Eventually only one communication service could be used on a local basis among operators at times of low usage, as further energy saving gain to be exploited. Agreements could be put in place between operators so that in the low load periods (e.g., nighttime) only one of multiple mobile networks may be active in an area and will provide communication service to the subscribers of all networks, whereas the other networks can apply cell shutdown of their own infrastructure to obtain network energy savings.

Alternatively, based on risks of power outage nation-wide/region-wide, regulators could ask operators to “optimize” their coverage e.g., shutdown some nodes in overlapping coverage areas during energy peak hours and/or in specific geographical areas, whilst still guaranteeing minimum coverage/service (in particular to fullfill regulatory requirements for services such as emergency calls, PWS, MPS and MCS).

This can also apply between NPN operators and/or with PLMN operators.

#### 6.15a.6.2	Requirements

Subject to regulatory requirements and operators’ policies, the 5G system shall enable an operator to temporarily serve UEs of other operators within a geographical area for the purpose of saving energy of the other operators.

NOTE 1: The other operators are assumed to stop providing access to their own network infrastructure within the same geographical area to save energy during that time.

NOTE 2: Policies may include predefined times/locations, energy consumption/efficiency thresholds, etc.

NOTE 3: It is assumed that the 5G system can collect charging information associated with serving UEs of other operators.

THIRD CHANGE (NEW TEXT)

## 7.13	KPIs for energy-related characteristics exposure

The functional requirements related to exposing network energy-related characteristics can be found in clause 6.15a.5.

Exposing carbon equivalent emissions at subscriber level can rely on but does not imply any real-time collection, monitoring or exact subscriber-specific measurements or calculations. It can be provided by means of attribution based on a combination of statistics, computations and estimates as accurate as possible, e.g. based on subscriber’s data volume and operator’s carbon intensity to provide the communication service, which can be collected from external sources. Hence the accuracy of the exposed information depends on this input.

Deriving and providing such information too frequently to many users may consume network energy. Hence, the following KPIs have been defined as a compromise to achieve specific use cases whilst trying to minimize their energy impact.

Table 7.13-1 defines the performance requirements related to the exposure of network energy-related characteristics such as energy consumption and carbon emissions depending on the target use case and associated intended impact.

- To contribute to the comprehensive energy consumption and CO2e transparency in the end-to-end service chain (including both 3GPP and non-3GPP related providers), it is important that the frequency of such indications is meaningful to the user during a session of the communication service (e.g. not more frequently than once per minute when using a video streaming application).  Such information may impact the behavior of users during their current session.

- To empower mobile subscribers (individuals and companies) to contribute to overall energy and carbon footprint reduction by understanding the carbon equivalent emissions resulting from their use of the communication service, it is important to provide periodic information (e.g. per day over a month period), although such reports may only impact the behavior of users during future sessions.

Table 7.13-1: Performance requirements for energy-related characteristics exposure

|  |  | Characteristic parameter (KPI) |  |  |
| --- | --- | --- | --- | --- |
| Information | Use Cases | System Granularity | Time Granularity | Time Range | Obtainment (informative) |
| Carbon emissions (e.g. in kg of CO2e) | Subscriber’s carbon footprint information exposure to 3rd party. (NOTE 1) | UE | Day | Month | Calculation (e.g. average or statistical model) based on attribution of network energy consumption, carbon intensity and/or renewable energy ratio. |
|  | Energy Consumption and CO2e transparency in the end-to-end service chain (NOTE 2) | Service data flow | Minute | Session duration |  |
| Energy consumption (e.g. in kWh) | Energy Consumption and CO2e transparency in the end-to-end service chain (NOTE 2) | Service data flow | Minute | Session duration | Calculation of network energy consumption based on attribution e.g. measurements, service settings, external information and models. |
| NOTE 1: Such indication should be meaningful to the user to raise awareness on a monthly reporting basis or to incentivize the service consumer (out of scope of 3GPP). NOTE 2: Frequency of such indications should be meaningful to the user during the communication service (e.g. video streaming application). |  |  |  |  |  |

END OF CHANGES