3GPP TSG-SA WG1 Meeting #109	S1-250506

Athens, GR, 17-21 February 2025                                                   (revision of S1-250227)

Title:	Use case on energy aware smart resource allocation for sustainable 6G network

Agenda Item:	8.1.1 System and Operational Aspects

Source:	IIT Bombay, Nokia

Contact:	Rashmi Kamran (rashmi.kamran@iitb.ac.in)

Manjesh Kumar Hanawal (mhanawal@iitb.ac.in)

Abstract: This document proposes a use case on energy aware smart resource allocation for sustainable 6G network.

---------- Use Case template ----------

## 5.x	Use case on energy aware smart resource allocation for sustainable 6G network

### 5.x.1	Description

Mobile communication systems contribute significantly to energy consumption, with a substantial portion generated through processes emitting greenhouse gases. Considering the potential negative impact of greenhouse emissions on human society, ITU-R identifies sustainability as a foundational aspiration for the 6G system. Incorporating sustainability goals into mobile network design requires bringing energy awareness to communication and computing processes in these networks, which is currently overlooked. The 6G system is expected to accommodate a substantial volume of connected devices and high-data-rate applications compared to its predecessors. The extensive communication support, however, will result in heightened energy consumption in 6G systems. Hence it is necessary to integrate energy awareness into design and network decisions in 6G mobile networks.

Recent ongoing work in 3GPP also emphasizes energy awareness in mobile networks by collecting energy-related information from network functions, exposing it securely to users and third parties (application service providers), and adjusting the Quality of Service (QoS) for data flows based on energy-related information. Here, energy-related information includes per slice and user (subscriber) energy usage information for network functions and the ratio of the type of energy resources, conventional or renewable, used to serve a user. This information can also be shared with third parties (application providers). Such energy awareness in the network also can be utilized for making network decisions. This approach can result in energy aware decisions towards minimizing energy usage and increasing renewable energy usage ratio in the network to a great extent.

In addition to enhanced energy awareness in the network, AI-ML is touted as one of the key technologies for the 6G system; it is anticipated that ‘AI-for-networks’, i.e., learning-based network design and algorithms, will play a significant role in improving the resource allocation mechanisms, and the overall 6G system. Joint application of these two approaches ‘energy awareness into design and decisions’ with ‘AI-ML for network’ is a way forward towards sustainable networks.

### 5.x.2	Pre-conditions

Energy information (including energy usage information and the ratio of the type of energy resources being used) of the network resources is available.

Note: Network resource here includes all resources in the network that are used for user service delivery, e.g., compute, storage and communication resources.

### 5.x.3	Service Flows

Network A wants to save energy and maximize the renewable energy ratio for its functioning. It supports the AI-ML framework in the network for making network decisions like resource allocation. This AI/ML framework uses energy information about resources as input. As a result, the selection of resources is also based on their energy efficiency and renewable energy ratio. An example of resource selection is explained next. Figure 1 shows an energy-aware approach to resource selection. For example, using only a required number of resources ‘ON’ and keeping other resources in ‘power-saving mode/OFF’ may help save energy. As shown in the upper part of the figure, three out of six resources are sufficient and needed to serve existing information flows and are kept ON. The remaining three resources are kept in power-saving mode or OFF. Further applying AI/ML based energy aware resource allocation algorithm can select the resources based on their energy usage characteristics and renewable energy ratio, which will further result in energy saving and maximizing usage of renewable energy source-based resources. This case is visible in the lower part of the figure; only three resources are being used, similar to the conventional allocation, but the resources are chosen based on their energy usage and renewable energy ratio.

Figure 1: AI/ML driven energy aware resource selection approach

### 5.x.4	Post-conditions

Network saves energy and also maximize usage of renewable energy by taking energy information into account and using AI-ML for network.

### 5.x.5	Existing features partly or fully covering the use case functionality

The 5G system shall enable the network operator to define and statically configure a maximum resource assignment for a specific service that can be adjusted based on the network state (e.g., during congestion, disaster, emergency and DDoS events) subject to regional or national regulatory and operator policies. [TS 22.261 Clause 6.7.2]

The 5G system shall support network resource utilization efficiently and network optimization based on system information, including: - network conditions, such as network load and congestion information; [TS 22.261 Clause 6.11.2]

-	information on served UEs such as access information (e.g., 3GPP access, non-3GPP access), cell type (e.g., macro cell, small cell), user experienced data rate;

-	application's characteristics (e.g., expected traffic over time);

-	information on prioritized communication such as user subscription profile and priority level, priority services (e.g. MPS, Emergency, and Public Safety), application used for priority communications (e.g. voice, video, and data) and traffic associated with priority communications (signalling and media);

-	subject to user consent, enhanced traffic characteristic of UE (e.g., Mobility information (e.g. no mobility, nomadic, spatially restricted mobility, full mobility), location, sensor-level information (e.g. direction, speed, power status, display status, other sensor information installed in the UE), application-level information (e.g. foreground applications, running background application, and user settings).

The 5G access network shall support an energy saving mode with the following characteristics: [TS 22.261 Clause 6.15.2]

-	the energy saving mode can be activated/deactivated either manually or automatically;

-	service can be restricted to a group of users (e.g. public safety user, emergency callers).

NOTE:	When in energy saving mode the UE's and Access transmit power may be reduced or turned off (deep sleep mode), end-to-end latency and jitter may be increased with no impact on set of users or applications still allowed.

The 5G system shall support different energy states of network elements and network functions. [TS 22.261 Clause 6.15a.3]

5G system shall support dynamic changes of energy states of network elements and network functions. [TS 22.261 Clause 6.15a.3]

NOTE: This requirement also includes the condition when providing network elements or functions to an authorised 3rd party, the dynamic changes can be based on pre-configured policy (the time of changing energy states, which energy state map to which level of load, etc.)

The 5G system shall support different charging mechanisms based on the different energy states of network elements and network functions. [TS 22.261 Clause 6.15a.3]

Subject to operator's policy, the 5G network shall support energy consumption monitoring at per network slice and per subscriber granularity. [TS 22.261 Clause 6.15a.4.2]

NOTE 1:	Energy consumption monitoring as described in the preceding requirement is done by means of averaging or applying a statistical model. The requirement does not imply that some form of 'real time' monitoring is required. The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services.

Subject to operator’s policy and agreement with 3rd party, the 5G system shall support a mechanism for the 3rd party to provide current or predicted energy consumption information over a specific period of time. [TS 22.261 Clause 6.15a.5.2]

The 5G system shall support centralized automation and management of the network in order to reduce local management tasks. [TS 22.261 Clause 6.16.2]

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] Subject to operator policy, the 6G network shall support network AI/ML workload to utilize network energy consumption and energy supply mix information as one of the considerations to make decisions on network resource allocation.

Note: 	Examples for network resource allocation can be user plane path adjustments, decision to switch on/off cell targeting energy saving.