3GPP TSG SA WG 1 Meeting #109 	S1-250524

Athens, Greece, 17-21 February 2025	(revision of S1-250313)

Source:	Philips International B.V.

pCR Title:	Update use case 5.4 with multiple FWA devices

Draft Spec:	3GPP TR 22.870 v0.1.1

Agenda item:	8.1.1

Document for:	Approval

Contact:	Walter Dees (walter.dees@philips.com)

Abstract: This pCR proposes to update use case 5.4 in TR 22.870 with additional aspects related to having multiple FWA devices.

1. Introduction

Use case 5.4 describes the benefits of Fixed Wireless Access as a means to circumvent wired connectivity to people’s home with equivalent or even better performance, and that these benefits will also apply to 6G.

2. Reason for Change

In order to have good wireless network performance within people’s homes a single wireless access point built into or otherwise attached to a Fixed Wireless Access device is typically not sufficient, since good wireless coverage across all areas of people’s homes is difficult to achieve in particular in higher frequency bands for Wi-Fi such as 5GHz and 6GHz. Wi-Fi mesh routers may be used to achieve better coverage. However, with prices for FWA devices decreasing it would be beneficial to deploy multiple FWA devices to increase not only the coverage within people’s homes and e.g. backyard, but also to provide better performance of the cellular access to the 6G network since the load can be shared amongst the multiple FWA devices.

3. Proposal

Update use case 5.4  with additional aspects related to having multiple FWA devices, for inclusion in 3GPP  TR 22.870 v0.1.1.

* * * First Change * * * *

## 5.4	Use case on Fixed Wireless Access (FWA)

### 5.4.1	Description

Fixed Wireless Access (FWA) in 5G NR has transformed the Home Internet business and has become a significant revenue generator for mobile network operators (MNO). It is expected that FWA services will continue to grow in 6G. While FWA has been supported since LTE, 5G's speed and flexibility has enabled it to emerge as a transformative and successful 5G service. FWA has become a key strategy in expanding high-speed broadband services to previously unserved or underserved areas (i.e., a tool to bridge the Digital Divide), for in-building services, and enterprise (i.e., private network) deployments.

Some initial deployments of FWA used "fallow capacity" in markets where enhanced Mobile Broadband (eMBB)usage was not in high demand, but the underlying network protocols, procedures, and resource management schemes of the macro networks are designed for eMBB services and Smartphone devices that require mobility support. FWA devices (e.g. Residential Gateway with cellular backhaul) have a very different mobility profile, traffic usage pattern, and are able to take advantage of higher output power improving their in-building link performance. A Customer Premise Network (CPN) may benefit of access through multiple FWA devices as means to improve coverage, capacity, and reliability. For example:

•	Using two or more RG units will be able to ensure improved coverage in the whole CPN (independently of the non-3GPP network within the customer network).

•	Two or more RG units will be able to handle more UL/DL traffic as required in the CPN. By using load balancing between RG units, data traffic to/from multiple devices connected to the RG, in particular devices that are in coverage of both RGs, higher and more consistent throughput can be achieved.

FWA should provide a quantitative improvement in service by using 6G services and capabilities (e.g. AI/ML).

### 5.4.2	Potential New Requirements needed to support the use case

[PR 5.4-1] The 6G system shall provide optimized network capabilities for FWA (e.g. support stationary devices).

[PR 5.4-2] The 6G system shall support FWA in relevant bands taking into consideration the regulatory requirements for each specific band.

[PR 5.4-3] The 6G system shall enable the means to provide awareness of user service characteristics (e.g. data rate, latency) to support the RAN and CN in making real-time resource allocation for FWA.

[PR 5.4-4] The 6G system shall provide FWA a comparable level of security and privacy protection to other 6G services.

[PR 5.4-5] The 6G system shall support efficient FWA connectivity.

Editor's Note: this requirement is FFS.

[PR 5.4-6] The 6G system shall support load balancing between multiple RGs offering FWA connectivity to devices in a CPN .

* * * End of Change * * * *