3GPP TSG-SA WG1 Meeting #109 	S1-250530

Athens, Greece, 17-21 February 2025	(revision of S1-250138)

Title:	5G and 6G Service Continuity for Network Slicing

Agenda Item:	8.1.1

Source:	Nokia, Telefonica, Verizon

Contact:	Feifei Lou (feifei.lou@nokia.com)

Abstract: Network slicing has been introduced as a fundamental pillar of the 5G system to address the requirements of differentiated services and to enable new business operations and opportunities. In the 6G era, network slicing is expected to continue playing a crucial role. Thus, it is essential to enable 5G and 6G service continuity for network slicing.

---------- Use Case template ----------

## 5.x	5G and 6G Service Continuity for Network Slicing

### 5.x.1	Description

3GPP has introduced network slicing as a mandatory feature for 5GS and indeed this remains one of the few features that operators around the globe are trailing and attempting to monetize for differentiated services beyond traditional mobile broadband. While the 5GS and with its network slicing have had some learning curve to go through, adoption is accelerating. Therefore, we consider that network slicing will also needs to be supported in 6GS, not to create a compatibility issue with 5GS deployments and prevent the take up of the network slicing feature in 5GS if there is evidence that it will be discontinued with the 6G system. It is therefore assumed network slicing will remain a component of the 3GPP system as we move from 5GS to 6G system.

One of the well-known problems of 5G is that even though the 5G system supports network slicing, due to UE mobility to 4G or even 5G non standalone deployment area, some of the 5GS network slicing features are broken. This creates some adoption hurdle e.g. for features such as the NSSAA (Network Slice Specific Authentication and Authorization) and also several enhancements in 3GPP continued to be made in 3GPP over many releases to ensure mobility to the EPS does not break the connectivity for the end user.

Similar to 5G deployments, it is expected that 6G deployments will have uncovered areas which necessitate seamless mobility between the 5GS and the 6G system to ensure connectivity is preserved also when using network slicing features. This implies that facilitating seamless service continuity between 5G and 6G for network slicing is vitally important.

Scenarios like the following should be supported:

Scenario A:

- A UE connects to a 5G network and uses some 5GS network slicing features.

- The UE moves to 6G system

- It is not desirable that the 5GS network slicing features are broken, or service degrades because the network or the UE are unable (due to standards issues) to support the 5GS network slicing features.

- The UE moves again under the coverage of the 5G network. The UE will access again the initial service.

Scenario B:

- A UE is connected to a 6G network and uses some 6G network slicing features

- The UE moves to an area without 6G network coverage and camps on the 5G network cell.

- It is desirable that the UE does not experience service disruption (albeit a downgrade in 5GS can be understood if 6G has distinct improvement over 5GS) as it moves to the 5G network cell because the 5G network or the UE are unable (due to standards issues) to support compatibility with the 6G system network slicing features in the 5G network.

- The UE moves again under the coverage of the 6G network. The UE will access again the initial service.

To support scenarios like the ones described above, the system needs to support interworking between 6G and 5G ensuring that a UE can move from a 6G network to a 5G network while it is served by a certain network slice. Additionally, the UE should be able to move from the 5G network to the 6G network and be served by a certain network slice, similarly to the case from 6G to 5G. Furthermore, the UE should be supported in case the 5G network cannot provide the same service as the 6G network does, if the 6G network will offer enhanced capabilities compared to 5G. Moreover, the UE should be able to switch between the two networks with limited impact to the accessed services, i.e., handover within a network slice should be supported between 5G and 6G networks.

Finally, the 6G and 5G networks should facilitate the UE to switch between the two networks and minimize the impact on the service continuity. For example, the ability of the 6G network to configure, like in the 5G network, the UE with information needed may select connectivity supported over certain network slices and the ability in the UE to route specific traffic over connectivity supported by these network slices.

### 5.x.2	Potential New Requirements

[PR 5.x.2-001] 6G system shall support network slicing.

Editor's Note: alternatively, this can be captured in the support for legacy services sub-clause.

[PR 5.x.2-002] 6G system shall support service continuity of network slicing between 5GS and 6G system.